Safe Work Procedure

Starting A Air Compressor

General – Employees cannot use an air compressor without being trained in its proper use and operation. Serious injuries could result through exposure to impact and cutting, noise and compressed air.

Hazards – Severe bodily impact [including hearing and eyesight]

Equipment /property damage

PPE – Steel toed footwear

Eye protection or face shield

Ear protection

Warning :

Do not wear loose clothing or rolled up sleeves

Remove all jewelry

Job Steps:

1. Read and understand equipment manual before operating.

2. Check oil level on air compressor engine. Add oil if necessary.

3. Check air filters and clean as required.

4. Ensure all air valves are opened prior to starting.

5. Turn start/run lever to start position.

6. Prime carburetor by pumping choke control 12-15 times. Leave choke out fully.

7. Turn key to engage starter. If engine does not start, turn key to off, prime 2-3 more times,

then repeat step 4

8. When engine starts, slowly push choke in as engine warms up.

9. When engine has warmed up, turn start/run lever to run position.

10. Securely attach air hose to air outlet at one end, and blow-pipe at the other. In both cases insert safety locking pin.

11. After ensuring that valve at blow-pipe end is in “off” position, open valve at compressor.

12. Holding blow-pipe securely, slowly open valve at blow pipe.