| Developed By | Reviewed By | Updated |
| --- | --- | --- |
| Person 1 Person 2 Person 3 | WAGPI JOHS Committee | MM/DD/YYYY |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Round point shovel | Rock | Steel toed footwear |
| Gravel rake | Landscape fabric | Gloves |
| Wheelbarrow |  |  |
| Fan rake |  |  |
| Tarp |  |  |
| Utility Knife |  |  |
| Edger |  |  |

General – Rock is an alternative to mulch as a garden accent. Rock is more of an aesthetic look as it does not retain moisture and does not provide nutrients to the soil. Rock will hold heat and because of that choose your plants wisely.

Job Steps

- Lay out fabric and cut to the size of the area needing to be covered. If there are plants, you will need to either cut a hole [depending on size of plant] and carefully fit the fabric down over the plant. If the plant is too large you will need to fit pieces around the bottom of the plant. The fabric should be laid so that it is just touching the plant stem, shrub base etc. Fabric should be tucked in on itself along the edges of the garden.

- Lay tarp under tailgate of truck to minimize cleanup of debris.

- Fill wheelbarrow to personal comfort load and lift using good ergonomics. Transport to garden. Note-Plan your route to the work area. Sometimes the shortest route is not the best route to travel. Area may be soft, and damage may occur.

- Go to the farthest work area first and carefully dump the rock watching out for the plants. Use a garden rake or hands to move rock around base of plant and manipulate rock along edges of garden to have a clean edge.

- Continue to dump rock until the entire area is covered. Depth will depend on the type of rock. Do not pile rock against the stem or trunk of plant

- When job is completed take a broom and sweep [if concrete, brick, asphalt] area.  Use hose [if available] give a wash to the area and “water” rocks to give clean look. If lawn or mulch “fluff” the area.

- Clean the area around the truck. Load and secure tools if required.