Safe Work Practice

Gas Powered Edger [Bed Definer]

General-The gas-powered edger is a high speed fast cutting power tool. Special safety precautions must be observed to reduce the risk of personal injury.

Hazards- Severe bodily impact

Musculoskeletal injuries

Property damage

PPE-Steel toed footwear

Hearing protection

Eye Protection

Long Pants

Long Sleeved Shirt [snug but still allows movement]

Notice

-The edger works by being pulled backwards. Follow the arrow. DO NOT PUSH THE EDGER FORWARD

-Always hold and operate the edger on the left side of your body with the cutting attachment to the rear. Always maintain a safe distance from the cutting attachment.

-Do not operate edger if coworkers/public enter the immediate work area as debris can be thrown at a high rate of speed.

Job Steps:

-Perform hazard assessment of site

-Start the engine

-Start to cut at no less than half throttle and continue cutting at full throttle.

-Hold and guide the edger so the blade is vertical.

-Cut steadily so the engine is not lugged down-do not apply force.

-Operate at no more than a normal walking pace

-Do not push the blade into the ground.

-Remember - Always pull the edger not push it.

DO NOT USE EDGER ON HARD GROUND OR WET SOIL

The owner’s manual takes precedence over the SWP