Safe Work Practice

Angle Grinders

General – Angle grinders produce a lot of dust, sparks. They can also shatter and pieces can fly at a high rate of speed. The only way to reduce risk is through practical training in operating the machinery.

Hazards – Inhalation

Blindness

Burns

Electrocution

Bodily impact

PPE – Steel toed footwear

Eye Protection

Ear protection

Close fitting protective clothing to be worn

Headwear to contain long hair

### PRE-OPERATIONAL SAFETY CHECKS

    - Use only in designated grinding area – erect screens if necessary.

    - Examine the power cord, extension lead, plugs, sockets and power outlet for damage.

    - Ensure that the grinding disc, guard and attachments (including handle) are secure and correctly fitted.

    - Inspect the grinding disc for damage.  Do not use damaged grinding disc.

    - Always inspect the work piece to ensure that there aren’t any items which might damage the grinding wheel or cause injury to the operator.

    - Secure and support the work piece using clamps, bench vices, etc.

OPERATIONAL SAFETY CHECKS

- Ensure all other employees are clear of the immediate work area.

- Keep fingers and hands & power cords clear of the grinding disc.

- Never make adjustments while the angle grinder is running.

- Do not switch off the angle grinder when it is under load, except in an emergency.

- Allow angle grinder to reach operating speed before applying to work piece & increase load gradually.

- Do not lift or drag angle grinders by the cord.

- Keep flexible electrical cords clear of oil, grease, machines and sources of heat.

- Be aware of flying sparks.  Hold grinder so that sparks fly away from you, other people and flammable materials.

- Do not leave the angle grinder running & only use the grinder when hand held.

- Do not touch the work piece immediately after grinding operation as it may be extremely hot.

HOUSEKEEPING

- Avoid trip hazards & prevent damage to electrical cord/s.

- Do not walk on, wheel objects over, or drop materials / tools on flexible electrical cords.

- Clean bench and work area & place all waste material in bin.

- Return angle grinder & rolled up extension leads to storage area.