Safe Work Practice

Operation of Small Water Delivery Truck

General – Accidents/ injuries may occur if not familiar with the proper procedures of water delivery.

Hazards – Ergonomics [getting off and on truck, hauling hose, removing well crocks], Uneven terrain [walking and parking of vehicle], malfunction with pump [ fire, explosion] people, pets,

- Obey all regulations within the Highway Traffic Act.

- Do pre-trip of vehicle. This includes hoses and tank.

- Before turning on the water ensure truck is in park with parking brake engaged.

- Hook hose up to water valve.

- Carefully mount the trunk to put the other end of hose in the tank.

- Use caution when walking on deck of truck because it may be slippery.

- Carefully, dismount the truck.

- Watch closely as tank fills to ensure no leaks are occurring.

- When tank is full, turn off water.

- Carefully mount the truck to remove hose. Dismount carefully.

- Before leaving the lot, make sure all hoses are securely tied down

- Make sure you travel slowly when carrying full tank of water.

- When arriving at delivery site locate well or tank. Make sure there are no low power lines, wash lines, cable lines, tree branches that could snag on truck.

- Make sure there are no people, especially children or animals in the immediate area when moving the vehicle or when well cover is off

- Move truck into position

- Remove well cover

- Put hose into tank

- Turn on water

- Pay close attention while filling the tank as to not over fill.

- When done turn off water

- Remove hose, roll, and put securely onto truck laying them smooth as not to create kinks in hose

- Secure well cover before leaving