Safe Work Practice                                                                                              Nov.27/18

Tree Baler

General – A tree baler is a machine that wraps trees for easier shipment and storage. There are moving parts therefore attention is required when operating the machine.

Hazards-serious bodily impact

musculoskeletal injuries

noise

PPE -Steel toed footwear

Ear protection

Gloves

Snug clothing

*No jewellery

Before starting:

-Check fluids and string before starting.

-Check belts for wear and tear. Replace if needed.

-Check springs

-Grease all pullies

Operation:

-Only 1 person to operate the levers and to ensure no employee does anything to the baler while it is in operation.

-If a mechanical issue happens while running the baler. STOP baler first before trying to fix the problem.

-1 person to load trees one by one in the back shoot. This person attaches the hook to the stump of the tree and the twine.

-The lever is pulled to advance tree through the baler. Keep hands away from the tree as it is advancing through the baler.

-Pull lever to stop baler once tree has fully cleared the baler.

-Cut/tie twine on tip of trees. Release hook.

-Place tree off to the side of the baler.