| Developed By Blair Rawding | Reviewed By |  |
| --- | --- | --- |
|  | WAGPI JOHS Committee |  |

| Equipment Required | Material Required | Personal Protective Equipment |
| --- | --- | --- |
| Hammer drill | none | Safety Footwear |
| compressor |  | Ear Protection |
|  |  | Eye Protection |
|  |  | Respirator |
|  |  | Gloves |

Job Steps

- Unlike conventional drills, hammer drills can drill in concrete and brick as well as wood and metal. Be sure to position the mode selector switch for standard drilling (wood and metal) or hammer drilling (concrete and brick). 2. You must use percussion type drill bits when drilling concrete or brick with a hammer drill. These are special bits that are tipped with carbide steel which is much harder than drill bit steel. Carbide can fracture concrete and brick when the hammering action of a skill hammer drill is applied. 3. Always hold the drill by insulated gripping surfaces and use the auxiliary handle (if provided) for maximum control over torque reaction or kick-back. 4. Don’t force the tool, let the drill bit do the work. Keep the speed up, don’t stall. As you drill deeper, pull the bit out of the hole frequently to clean dust.