Safe Work Practice

Chop Saw

General - A chop saw is a powerful tool that uses an abrasive disc to quickly cut along a straight line. This type of saw is typically used for making fast, rough crosscuts, such as cutting large pieces of lumber into smaller sections or cutting pipes into the desired length.

PPE -Steel toed footwear

Gloves

Ear protection

Eye protection

No jewelry to be worn.

Contain long hair.

Procedures

1. Ensure the saw is properly secured to a worktable by bolts/clamps at

approximately hip height.

2. Ensure the saw is operated on ESD protected circuit.

3. Check workspaces and walkways to ensure no slip/trip hazards are present.

4. Check that all safety guards are in position and are operational.

5. Ensure you are familiar with the operation of the ON/OFF starter.

6. Keep table and work area clear of all tools, off-cut timber, and sawdust.

7. Start the dust extraction unit before using the machine.

8. Faulty equipment must not be used. Immediately report suspected equipment.

9.Do not use this saw to cut pieces that are too small to be securely clamped.

10. For operation of the chop saw, face shield and safety glasses must be worn.

11. Loose clothing, jewelry, or any dangling objects including long hair should not be

worn as they may catch in the rotating parts of the saw. Hair must be contained.

12. The maximum cut for the machine must not be exceeded.

13. Ensure all adjustments are secure before making a cut.

14. Use clamps to secure and support the work piece to a stable platform. Do not use

a length stops on the free scrap end of a clamped work piece.

15. Do not cut more than one work piece at a time.

16. Before turning on the saw, perform a dry run of the cutting operation to ensure

that no problems will occur when the cut is made.

17. All guards must be in place and operating. If a guard seems slow to return to its

normal position or hangs up, adjust it, or repair it immediately. Unplug before

adjusting or cleaning.

18. Do not start the saw with the blade touching the work piece. Allow the blade to

reach full speed before contacting the work piece.

19. Avoid reaching over the saw line. Do not cross arms when cutting.

20. Hands and fingers must be kept clear of the path in which the blade travels.

21. When using the right hand to pull the saw down, keep the left hand, especially the

thumb well clear of the line of cut.

22. Clean the lower guard frequently to help visibility and movement. Unplug before

adjusting and cleaning

23. Use only the recommended RPM and sizes of blades.

24. Regularly check and tighten the blade, be sure to lockout or unplug equipment

25. When installing or changing a blade, ensure that the blade and its related washers

and fasteners are correctly positioned and secured on the saw’s arbor.

26. Long material should be supported at the same height as the saw table.

27. Use the brake if one is installed. To avoid contact with a coasting blade, do not

reach into the cutting area until the blade has come to a complete stop.

28. After completing a cut, release the trigger switch and allow the blade to come to a

complete stop, and then raise the blade from the work piece. If the blade stays in

the cutting area after the cutting is complete, injury can result from accidental

contact.

*Be aware saw can make accidental “kick back”