Safe Work Practices                                                                                                  Updated Nov.13/18

Working Under Raised  Hydraulic Dump Boxes

General – Performing maintenance under the dump body of a dump truck presents extraordinary hazards to maintenance or servicing personnel. Employers or employees who service or maintain dump trucks must be educated about the hazard of falling dump truck beds

Hazards – severe bodily impact

Pinch points

Struck-bys

PPE -Steel toed footwear

other PPE for given task

-When the dump body must be raised for maintenance to occur, the machine or equipment shall be isolated from its energy source and rendered inoperative before under-body inspections or maintenance can take place.

-When there are two or more employees involved in the maintenance procedures their activities must be coordinated so that one employee’s actions do not increase the risk to the other employee. Each employee must be aware of the other’s location and activity at all times.

-Trucks with dump bodies shall be equipped with an engineered safeguard mechanism capable of being locked in position to prevent accidental lowering of the body while maintenance or inspection work is being done.

-Inspect the dump body hydraulics for damage or defect. Take equipment determined to be unsafe for use out of service immediately.

-Follow manufacturers’ instructions for detecting hydraulic leaks. No part of the body shall be used to locate or stop hydraulic leaks.

-Be aware of the hazards associated with raising/lowering the truck’s dump body.  Ensure that all personnel remain out of harm’s way.

-Dump bodies shall be equipped and maintained with manufacturers’ safety devices, instructions, warnings, and safeguards. Workers must follow manufacturer’s instructions.