Safe Work Practice

Operation of a Front End Loader

General -A front end loader is a large piece of equipment and if not operated by a trained experienced operator serious damage may occur to vehicles, properties and could result in grave injuries or death.

Hazards – equipment tip over, person being struck and/or being caught in or between

PPE-  Hearing protection, eye protection, safety footwear, hard hat, safety vest

General Use of a Front End Loader

• Pre-start checks SHALL be completed at the beginning of the shift and

defects reported to the Supervisor and/or Employer.

• Operators should aim to develop the sequence and rhythm of operation of

the machine. Speed and fluency of operation comes with experience.

• Don’t over activate the levers as jerky and erratic operation results causing

spillage.

• Avoid banging the bucket back against the stops as this causes cracks and

damages the hydraulics.

• ALWAYS visually check area around you before making directional

changes.

• Avoid overhangs

• Minimise cycle times by:

- Correct placement of trucks

- Minimising unnecessary travel distance

- Working the face close to the truck

- Minimising the time the bucket is engaged in the face

- Maintaining engine revolutions

- Keeping the pit floor clean and level

- Forward planning the excavation sequence

Truck Spotting

• Where possible position the truck at a 45o angle to the face. This will reduce

the amount of manoeuvring needed and reduce cycle time.

• As a guide you should not exceed one turn of the steering wheel from the

face to the truck.

• Set the bucket so that the lift arms just clear, (150mm to 200 mm), the tray

of the truck.

• Wait for the truck to back under the bucket with the transmission in reverse

and your right hand on the bucket lift lever.

• This will allow you to prevent the truck from hitting the loader should the

driver misjudge his/her approach.

• Maintain your engine revs while the truck is reversing.

• Place the machine so that the truck driver can line his/her tray up with the

bucket hinge pins.

• Ensure the truck turning area is large enough to facilitate easy

manoeuvring. Consider the difficulties you may be creating for them.

• Avoid setting the truck up to reverse over rocks or up the rill at the toe of the

face thus causing tyre damage.

• Use the horn to indicate to the truck driver to stop. Select neutral to blow

the horn then select the required gear to position the loader so the first

bucket lands in the centre of the tray.

• DO NOT discharge material into the tray to stop the truck.

Loading the trucks

• Maintain maximum revs at all times, (don’t feather the throttle). Doing this

you drop too many engine revs you will lose hydraulic and fuel pressure

and will consequently have less control of the machines functions.

• Reposition the loader, usually forward, to be placing the first bucket in the

centre of the tray. NOTE: ALWAYS ENSURE THAT THERE ARE NO

LARGE ROCKS IN THE FIRST BUCKET.

• Don’t leave the bucket in the air while waiting for trucks for an extended

period of time.

• This places pressure on the hydraulic seals and back through the hydraulic

system, which may cause a hydraulic hose to blow resulting in unnecessary

down time.

• Fill the bucket and place it against the face while waiting for trucks.

• Readjust the height of the bucket to minimise the distance the material in

the bucket will fall to the truck.

• Crowd the bucket forward slowly and evenly while raising the lift arms when

working at a high working face.

• If working at a low face ie. flat dumps or batter trimmings, drive the machine

more into the pile and use more torque.

• When the bucket is empty and the bucket teeth will clear the truck tray

engage reverse.

• As the bucket clears the truck, pull the bucket crowded lever back to the

detent and lower the lift arms for your next bucket of material.

• When loading large rocks ALWAYS ensure that there at least two (2)

buckets of loose material in the tray.

• This will minimise damage to the truck and reduce the likelihood of injury to

the truck driver. NOTE: ALWAYS WARN THE TRUCK DRIVER THAT

YOU ARE ABOUT TO LOAD LARGE ROCKS.

• Try not to load large rocks on to the top of a load where possible, as this will

reduce damage to the truck and spillage.

• Load from the front to the rear in most cases, as this will minimise spillage.

Always place the load centrally overall.

• As the bucket clears the tray on subsequent bucket loads crowd the bucket

forward while the lift lever is in the raise detent position.

• Only allow the bucket to lift as high as it necessary, crowding the bucket

forward can control this.

• DO NOT allow the lift arms to go up to the kick out level unnecessarily as

this destabilises the machine, reduces production, increases the chance of

damage to the truck and increases the likelihood of injury to the truck driver.

• DO NOT hit the truck tyres with the loader tyre.

• Use the transmission neutraliser brake, that’s what it’s for.

Loading the Bucket

• Always load in first (1st) gear only.

• Always ensure the full face of the bucket is working in the face as this sets

the machine up for easier manoeuvring and bucket corner loading twists the

lift arm linkages,

• DO NOT ARTICULATE WHILE LOADING.

• Set the working face so as full bucket face is working in the face and so

optimum-loading cycles can be maintained.

• Set the variable capacity torque control lever, VCTC, where applicable, to

2/3 to ¾.

• While the bucket is engaged in the face in tight digging flick the VCTC

switch to on to increase the torque to the wheels. This will assist with face

penetration.

• As digging becomes easier or as the bucket teeth leave the face flick the

VCTC switch to off this will help to reduce wheel spin and maximise

production.

• To leave the torque control lever set at ¾ to full application destroys the

torque converter and takes hydraulic pressure and therefore hydraulic

speed away from the bucket and lift cylinders which will reduce production.

• To keep a level floor you should be able to see faint marks left by the

bucket teeth.

• If you can’t the bucket is crowded too far back and you are unnecessarily

wearing the wear plates out on the bottom of the bucket.

• Use the crowd lever to fill the bucket, not the lift circuit, by working the

bucket back and forth.

• The lift lever may be in the raise detent position or used intermittently in

conjunction with the crowd control depending upon the material being

excavated

• If the teeth are not engaged in the face you have crowded the bucket back

too far and will be running the wear plates up the face.

• Never screw the articulation to fill the bucket as this causes unnecessary

wear on the machine.

• Never allow the rear wheels to lift off the ground by crowding the bucket

back in hard digging.

• In very heavy material or very blocky material it may be more productive to

only ¾ fill the bucket rather than fighting in the face for a full bucket.

• This will cause less wear on tires and components reduce systems

overheating and improve production.

Directional Changes

• When making a change of direction:

- Bring the machine to a complete stop using the transmission

neutralising brake pedal, the left braking pedal.

- Selecting the new direction.

• Keep engine revs at or near high RPM in order to maintain hydraulic oil

pressure and engine power.

• Don’t change direction and gear selection at the same time.

• Change directions first then select another gear range (i.e. don’t change

from 1st forward to 2nd reverse - change from 1st forward to 1st reverse to 2nd

reverse).

Working the Face

• Establish a load point and position your trucks accordingly, at a 45o angle to

the face. Where possible excavate a depression in the face and place your

truck in the depression.

• This will minimise your travel distance and improve your productivity.

• Where you are digging will be where you will position your trucks presently.

In this fashion, you will be able to work your way across the face and

maintain a square face.

• However, all faces are different and the important thing to remember is that

you must plan how you intend to excavate the face.

• Remember to work the face systematically.

• Avoid undercutting the face, by running the bucket up the face as

necessary.

• If the face collapses onto a truck or loader not only will damage result but

the risk of injury also exists.

• Pull down overhanging material and rock but avoid running the front tyres

up the face to do it as this damages the tires.

• Avoid working facing into the wind wherever possible

• Avoid loading downhill, where possible, as the machine may become

unstable and will be more difficult to stop.

• It is recognised that it is not always possible to avoid loading downhill and

extreme care should be exercised in these circumstances.

• When carrying a heavy load downhill, reverse down the hill.

• As the bucket teeth clear the face, put the lift arm raise lever into the raise

detent position and select reverse.

• Allow the bucket to continue to lift. You will be able to position the loader

ready to drive forward to load the truck, before the bucket will clear the side

of the truck tray.

• Many operators sit there waiting for the bucket to come up above tray

height and turn the front wheels to be able to square up to the truck while

stationary.

• This put excess pressure on the pivot pins and scrubs the tyres.

• Allow the loader to continue to reverse while the bucket is being raised such

that by the time you drive forward to the truck the bucket will clear the tray

and you are able to control the bucket raise function and material discharge

using the bucket crowd lever.

• Overall cycle time should not be affected.

Relocating the Loader

• When travelling the machine from one location to another the bucket should

be carried low to the ground (12” to 24”; 0.3m to 0.5m), so as to clear minor

spillage but still protect the transmission and drive train from larger rock

spillage.

• The bucket should be in normal loading position with the teeth slightly

angled towards the ground. This will assist in an emergency stop.

• Never travel down a ramp in a gear higher than second (2nd) gear. If the

ramp is steeper that 10% or the road surface is rough then select first (1st)

gear

• Never coast downhill in neutral.

• Keep the machine in gear and use the right hand brake pedal to moderate

speed.

• NEVER use the left brake pedal, the transmission neutraliser brake, whilst

travelling.

Parking

• ALWAYS leave the bucket on the ground and apply a slight down pressure.

• ALWAYS leave the transmission in neutral.

• ALWAYS apply the park brake.