Safe Work Practice                                                                                                                                             Updated Nov.14/18

Extension Cords

General- Used properly, extension cords are a good addition to the workplace and allow mobility and flexibility in where the work is produced. Improperly used cords and those in poor condition increase the risk of a shock or electrocution hazard to employees

Hazards- Severe bodily injury

Shock

Burns

Musculoskeletal injuries [trip]

Property Damage [could cause fire]

PPE – steel toed footwear

Practices to remember

Do’s

-Inspect the cord(s) to be used for any defects – cuts in the insulation revealing the inner wires or insulation pulled away from the plug, exposing the inner wires. Tag it out and remove from service for repair or replacement.

-Check the wattage rating of the tool to be used to make sure it matches that of the cord. Do not use a cord that has a lower rating than the tool.

-Make sure the plug is fully inserted into the outlet.

-If the outlet is too loose to hold the plug, tag the outlet for replacement. -Outlets do get worn over time and will result in a loose fit that can expose the prongs. Keep extension cords away from water.

-Use required GFCI protected power source on construction sites, even if it is connected to permanent wiring.

-Pull on the plug when removing from the outlet, not the cord.

-Unplug cords and roll them up when not in use to protect them from damage by equipment or foot traffic.

-Leave some slack on the cord to prevent putting tension on the plug at the outlet. If you are constantly finding the cord being pulled loose from the outlet, find a longer cord that will allow you to relieve the tension.

-Use stress relief if you have a large plug and cord, like those found on welders, to relieve the tension on the cords. Stress relief should be non-conductive, like leather or nylon cord.

-Look at the work area and the path of travel used for equipment and employees to determine the best location to plug a cord in. Use any cord arms or other devices that elevate the cords above heavy travel areas.

-Replace broken 3-prong plugs and make sure the third prong is properly grounded.

DON’T

-Use indoor cords for outdoor situations.

-Don’t daisy chain extension cords to reach the work area. Find the proper length cord to reach the work zone.

-Overload the cord with more than its stated load.

-Run extension cords across exit pathways, through doorways, windows, holes in walls, ceilings or floors.

-Bend or modify any of the prongs on the plug to make it fit better, or to fit into a two prong outlet.

-Force a prong into an outlet.

-Overheat an extension cord.

-Cover the extension cord with anything.

-Drive over extension cords. If the cord is in a travel area for equipment, like beside the shop, then use some form of cord protection.

-Attach the cord to walls with staples or nails.

-Run cords under carpets or high traffic areas.

-Use power strips intended for light duty office situations for high amperage tools. Use a properly rated industrial three way pig tail instead.

-Repair cords with electrical tape. Repairs must be made with heat shrink tubing or heat shrink tape that restores the cords original insulation value.

- Do not tie power cords in tight knots. Knots can cause short circuits and shocks. Loop the cords or use a twist lock plug.