Safe Work Practice

Portland Cement

General-Overexposure to portland cement can cause serious, potentially irreversible skin or eye damage in the form of chemical (caustic) burns, including third degree burns. The same serious injury can occur if wet or moist skin has prolonged contact exposure to dry portland cement.

PPE – Steel toed safety footwear [waterproof]

Eye Protection

Respirator

Gloves

Long sleeves

Hazards -hazardous materials

Dust

Contaminants

Practice

- • Wear waterproof Safety Boots high enough to prevent concrete from flowing in when workers must stand in fresh concrete - • Suitable eye protection where mixing, pouring, or other activities may endanger eyes - • Work in ways that minimize the amount of cement dust released. - • When and where possible wet-cut rather than dry-cut masonry products to prevent dust hazards. (Fill out code of practice concrete saw before work can begin.) - • When dry-cutting masonry products the cutter must wear a respirator. - • Mix dry cement in well-ventilated areas - • Make sure to work upwind from dust sources - • Clothing contaminated by wet cement should be quickly removed. - • Skin in contact with wet cement should be washed immediately with large amounts of cool clean water. - • Remove jewelry such as rings and watches because wet cement can collect under them. - • Refer to WHMIS guidelines for properties and ingredients in concrete recipe.