Traffic Control

General -

1. GETTING READY

A. Discuss traffic flow, breaks, and public directions with the supervisor and other flag person(s).

B. Do an equipment check:  CSA approved boots   Stop/slow paddle  White uniform    Airhorn  Florescent vest    Radios (if needed)  Safety glasses    Pylons  Florescent hard hat    Pencil and pad

2. LOCATE FLAG STATION A

. Locate spot on shoulder 50-100 M from worksite that is visible at least 150 M to oncoming traffic.

B. Ensure an escape route is possible.

C. Put out pylons if necessary.

3. PUT OUT SIGNS

A. Place signs approximately 100 M apart in this order:

 Flag person sign

 Speed Limit sign

 No Passing sign

 Speed Limit Ahead sign

 Construction Ahead sign

4. HANDLING TRAFFIC

A. Be courteous at all times.

B. Face traffic at all times.

C. Remain in station until front vehicle is stopped.

D. Hold Stop/Slow paddle at least 1.6 M high and proper side facing traffic.

E. Give verbal directions that are clear and concise.

F. Motion traffic with free hand.

G. Leave Post only when traffic is stopped or for escape.

H. Signal other flag person when releasing traffic or leaving.

I. Take accurate notes of all incidents.

5. LEAVING FLAGSTATION

A. Taking a break:

 Don’t leave until substitute is ready

 If no substitute, turn flag person sign around and notify supervisor before you leave

B. Leaving for the day:

 Signal your intentions to the supervisor and other flag person