CLEANING/DISINFECTING THE ENDOSCOPE CADDY TUBES

- Purpose

To ensure the endoscope caddy tubes are clean prior to insertion of the endoscope for disinfection; to prevent the spread of infection by removing microorganisms and reduce risk of transmission to patients, families and healthcare workers.

- Equipment/Supplies

- Hot water and liquid soap and/or detergent

- Super Sani-Cloth Germicidal Wipes

- Personal Protective Equipment (PPE) including: gloves, eye goggles and mask

- All endoscope caddy tubes

- Soft cleaning brush (i.e. toothbrush, bottle washer or other soft brush)

- Procedure

  - Don clean gloves

  - Inspect caddy tubes for cleanliness, proper labeling and overall condition.

  - At least monthly, or whenever caddy tubes become discolored, soiled or otherwise dirty in appearance, clean entire system with hot water and  liquid soap/detergent.

  - Use a soft brush to clean all joints and crevices with special attention to “eyeball mechanism”.

  - Rinse thoroughly with water.

  - Disinfect caddy tubes by wiping all surfaces with germicidal wipes.

  - Allow adequate time to air dry completely before returning to storage case.

  - OPA solution should be changed weekly and rinse water should be changed between facilities or where more than three procedures are completed.

- Considerations

  - Caddy tube for OPA disinfectant should be labeled and fresh solution replaced weekly.

  - Rinse water should be replaced after each facility use and if more than 3 procedures are performed.

  - You should always wear Personal Protective Equipment (PPE) when cleaning equipment.

Bibliography:

MSDS Medline OPA Disinfectant

MSDS PDI Super Sani-Cloth Germicidal Wipes