GOVERNMENT OF ANDHRA PRADESH

WATER RESOURCES DEPARTMENT

Name Of Work: FDR - Restoration of Yanamadurru Drain Right bank breaches at Km 0.800 in Nandamuru(V) and at Km 4.000 in Arulla(V), Tadepalligudem(M), West Godavari District. Estimate Cost: Rs. 22,30,000/-

| CIRCLE |  | - | IRRIGATION CIRCLE, ELURU |  |  |
| --- | --- | --- | --- | --- | --- |
| DIVISION |  | - | G.W. DIVISION, NIDADAVOLU |  |  |
| SUB-DIVISION - |  |  | DELTA SUB-DIVISION, TADEPALLIGUDEM |  |  |
| SECTION |  | - | CANAL SECTION, SETTIPETA. |  |  |

- Report for FDR - Restoration of Yanamadurru Drain Right bank breaches.

- Project Details

Name of Work: FDR - Restoration of Yanamadurru Drain Right bank breaches at Km 0.800 in Nandamuru(V) and at Km 4.000 in Arulla(V), Tadepalligudem(M), West Godavari District.
Estimated Cost: Rs. 22,30,000/-

- Background

The Yanamadurru Drain is a major drainage system in the Godavari Western Delta with a catchment area of 900 square miles. Originating from the Nandamuru aqueduct, it stretches for 60.330 km and discharges into Upputeru near Pathapadu village. This drain frequently encounters intensive floods, causing significant damage and crop loss due to the inundation of adjoining valuable wetlands in the Tadepalligudem, Tanuku, Attili, Pentapadu, Ganapavaram, Palakoderu, Bhimavaram, and Mogaltur mandals.

- Issue

The right bank of the Yanamadurru Drain at Km 0.800 in Nandamuru village and at Km 4.000 in Arulla village within the Tadepalligudem Mandal has been severely breached due to heavy rains from July 19 to July 22, 2024. These rains in the Yerrakalva catchment area resulted in substantial flooding downstream of the Nandamuru aqueduct. The increased water volume caused the banks at the specified locations to overtop, leading to significant breaches. Damages were reported in APFC form 20 on July 21 and July 22, 2024.

- Inspection and Instructions

Following the breach, the issue was reported to the Executive Engineer, who inspected the site and directed the preparation of an estimate for the necessary restoration work. Consequently, an estimate has been prepared for the restoration work.

- Proposed Restoration Work

The restoration work includes Homogeneous Embankment with gravel supported by country wood ballies and bamboo thatties. The following specifications are adopted in the estimate:

- Country Wood Ballies: 100 to 150 mm diameter

- Bamboo Thatties

- Labour Charges: Provision for sharpening ballies

- Gravel

- Providing Homogeneous Embankment

- 2T Roller Usage

- Excavation: In all kinds of soil

- Supply of empty cement bags

- Labour Charges: Provision for filling and stitching of cement bags

- Additional Provisions

- GST: 5.00%

- NAC: 0.01%

- Seigniorages

- Photographic Charges and Contingencies

- Estimate Preparation

The estimate is based on the current standard schedule rates for 2024-2025. For items without available rates, the rates as per the Superintending Engineer, Irrigation Circle, Eluru, (Memo No. SE/IC/Elr/DB/ATO: 4/F.SSR. Rates No. 2297SE Dt. 16.5.2014) are adopted.

- Submission

This estimate is submitted to obtain the necessary administrative approval from the competent authority.

- Summary of Estimated Costs

| Item | Cost (Rs.) |
| --- | --- |
| Country Wood Ballies | 57,600 |
| Bamboo Thatties | 28,800 |
| Labour Charges: Provision for sharpening ballies | 2,880 |
| Gravel | 13,95,869 |
| Providing Homogeneous Embankment | 2,46,032 |
| 2T Roller Usage | 18,291 |
| Excavation: In all kinds of soil | 21,315 |
| Supply of empty cement bags | 56,000 |
| Labour Charges: Provision for filling and stitching of cement bags | 64,000 |
| GST (5.00%) | 1,01,883 |
| NAC (0.01%) | 2,038 |
| Seigniorages | 87,750 |
| Photographic Charges & Contingencies | 663 |
| Total | 22,30,000 |

| Assistant Executive Engineer | Deputy Executive Engineer | Executive Engineer |
| --- | --- | --- |
| Canal Section | Delta Sub Division | G W Division |
| Settipeta | Tadepalligudem | Nidadavolu |

This report and estimate are submitted for review and approval to facilitate the necessary restoration works and to prevent future breaches and mitigate flood risks in the affected areas.