Construction &working of biogas plant

Biogas is a clean and efficient fuel. It is a mixture of methane (CH4), carbon dioxide (CO2), hydrogen (H2) and hydrogen sulphide (H2S). The chief constituent of biogas is methane (65%).

Principle :Biogas is produced as a result of anaerobic fermentation of biomass in the presence of water.

Construction:The biogas plant is a brick and cement structure having the following five sections:

- Mixing tank present above the ground level.

- Inlet chamber: The mixing tank opens underground into a sloping inlet chamber.

- Digester: The inlet chamber opens from below into the digester which is a huge tank with a dome like ceiling.The ceiling of the digester has an outlet with a valve for the supply of biogas.

- Outlet chamber: The digester opens from below into an outlet chamber.

- Overflow tank: The outlet chamber opens from the top into a small over flow tank.

Working :

- The various forms of biomass are mixed with an equal quantity of water in the mixing tank.

- This forms the slurry. The slurry is fed into the digester through the inlet chamber.

- When the digester is partially filled with the slurry, the introduction of slurry is stopped and the plant is left unused for about two months.

- During these two months, anaerobic bacteria present in the slurry decomposes or ferments the biomass in the presence of water.

- As a result of anaerobic fermentation, biogas is formed, which starts collecting in the dome of the digester.

- As more and more biogas starts collecting, the pressure exerted by the biogas forces the spent slurry into the outlet chamber.

- From the outlet chamber, the spent slurry overflows into the overflow tank.

- The spent slurry is manually removed from the overflow tank and used as manure for plants.

- The gas valve connected to a system of pipelines is opened when a supply of biogas is required.

- To obtain a continuous supply of biogas, a functioning plant can be fed continuously with the prepared slurry.