CLOVERLEAF

CLOVERLEAF CL4

Thank you for purchasing a Cloverleaf CL4 Biological Pond Filer. I hope you find the instructions below for fitting, and maintaining your filter helpful. The filter you have purchased is designed to filter a ponds of approximately 5,000 gallons (22,500 Litres) for Koi or 10,000 galloons (45,000 Litres) for Goldfish.

For best filtration we recommend this filter is gravity fed via a skimmer/bottom drain and via Cloverleaf Stainless Steel UV clarifier.

You will need to use a pump that turns your water around once every 2 hours

Please read and follow these instructions very carefully and if you require any guidance contact our help line

YOUR FILTER EXPLAINED

Your Cloverleaf CL4 can be either pump or gravity fed. If the filter is being pump fed you will need a Cloverleaf pump adaptor (Available from your local stockist, part number CLPA4). This will convert the pump pressure to volume to maximise the efficiency of the vortex of the filter.

Your filter comprises of 4 separate biological/mechanical chambers.

The media does not come supplied with your CL4 as there are variations to media set up. If using the Cloverleaf CL4 media kit a recommended set up is described below

The vortex acting as the first chamber has an inlet arm set off centre and is media free.

The second chamber is fitted with mechanical brushes which rest on top of a grid.

The third chamber contains Japanese Matting which again rests on top of a grid

The fourth chamber contains your outlet and media of Aqua rock and foams, the rock again rests on a grid and foams placed on top of the Aquarock, foams should follow the order of fine on bottom, medium and then coarse.

Each chamber has 2 discharge options (8 in total) to allow for easy waste control

INSTALLATION

The filter needs to be stood on hard level ground (i.e. cement base or paving slabs)

The filter must be back filled using a weak mix of sand and cement. The cement should come to the bottom of the taper and spread to about 1 foot out from the width of the filter. Failure to install the filter in this way will invalidate manufactures warranty. Each chamber carries approx 1 tonne of water and if not supported in the recommended manor the filter is liable to break.

Note always install waste pipe work before back filling

Pump Fed

The inlet arm on the vortex needs 10 to 12mm removed from the front of the arm, and the pump adaptor (part number CLPA4) is pushed on without lubricant or sealant, the universal hose tail then needs to be cut down to required size for pump inlet and glued using solvent glue to front of pump adaptor.

The outlet needs 5 to 6mm removed and a 6” single socket coupling (part number SSC6) fitted. You now should use 6” soil pipe for return to your pond. Inside the filter the outlet pipe needs to have the top removed to act as an overflow, this should be cut 1-2” down.

Gravity Fed

The top lip of the filter should sit level with the water level of your pond. You need to remove 10 to 12mm from the front of the inlet arm and then fit a 4” single socket coupling (part number SSC4) which will push on without restricting your flow from your pond, ideally the filter should be fed by a bottom drain or skimmer via a Cloverleaf slide valve (part number CSV4).

The outlet is 1 ½” bsp thread should be linked to your pump via flexible hose or rigid pipe, and then back to your pond.

WASTE SET UP

The waste set up can be set in a variety of options either using a manifold linking all discharges into one waste pipe, or each can be discharged individually. Either installation is just as effective therefore choose the best set up for the location of your filter.

NOTE: Every chamber must have at least 1 discharge

Once a waste installation has been designed you need to drill out the chosen discharges taking care not to damage threads. You need to fix 2” tank connectors into your chosen discharges and then connect using pipe work to your chosen discharge route

CHECK YOUR DISHARGES BY FILLING EACH CHAMBER TO THE TAPER ONLY BEFORE BACK FILLING

PLEASE NOTE FLUSH NEW AQUAROCK BEFORE INSTALLATION!

MAINTENANCE

Your Cloverleaf filter has been designed to be simple and easy to maintain and clean.

There are no hard and fast rules to the filters maintenance as every pond is unique in its own way, regular observation and cleaning is the only true way of optimising the filters efficiency.

The discharges allow you to release sediment collected in each chamber when required.

If using CL4 media kit:

The brushes in the second chamber should be cleaned annually by removing and flushing with the use of pond water.

The Japanese matting in the third chamber should be cleaned annually again by removing and flushing with pond water.

The aqua rock in the forth chamber virtually needs no maintenance, but if you notice a reduced flow a 50% replacement of rock will be required. The foams should be cleaned using pond water. NOTE: foams should be replaced each spring

PLEASE NOTE FLUSH NEW AQUAROCK BEFORE INSTALLATION!

NEVER USE TAP WATER TO FLUSH FILTER OUT ALWAYS USE POND WATER

WARNING:

We recommend your filter is not installed during winter months as sudden impact at cold temperature may damage filter.

If for any reason your pump is turned off during winter months each chamber should be empty, if freezing occurs the expanding ice may damage your filter

All Cloverleaf filters are designed to run with standard Soil or Waste pipe which is available through your Cloverleaf stockist or through  plumbing merchants

Replacement brushes, foams, Japanese matting  and rock, or any pipe fittings will are available from your Cloverleaf stockist

If you have any problems locating pipe or fittings, or any queries on the installation please do not hesitate to contact our help line on 01277 366002