CLOVERLEAF OZONE INSTALLATION

MODEL CBPF0/2

1. Remove any packing form the unit

2. Connect 9mm flexible pipe to bottom connection of the ozone body and connect other end of the flexible pipe to the venturi situated on top of the fractionator.

3. Insert ozone bulb into green ozone holder of the ozone body.

4. Connect mains lead onto the ozone bulb end via the 2 pin connector.  Place rubber cap over bulb end and secure using supplied steel screws.

5.   Only when the ozoniser/fractioniser is completely assembled and in place plug the unit into a suitable power supply that is waterproof and safety trip protected and switch on.  The water flowing though the fractionator will draw ozone into the mixing chamber via the venturi.

6. We recommend that the ozone bulb be replaced every six months for optimum results. The use of a redox meter will enhance the control and monitoring of the system.