HEADSET/HANDSET CONNECTOR PIN-OUTS:

4-PIN FEMALE:

SMH210:					    DMH220:

PIN 1 - BLUE/SHIELD (- GROUND)		    PIN 1 – SHIELD (- GROUND)

PIN 2 – RED (+ MIC)				    PIN 2 – RED (+ MIC)

PIN 3 – GREY (- SPKR)				    PIN 3 – BLUE (- SPKR)

PIN 4 – YELLOW (+ SPKR)			    PIN 4 - YELLOW/GREY (+ SPKR)

SMH910:					    DMH920:

PIN 1 - BLACK/GREEN			                PIN 1 - BLACK/GREEN

PIN 2 - YELLOW					    PIN 2 - YELLOW

PIN 3 - RED/ORANGE				    PIN 3 - RED/ORANGE

PIN 4 – BLUE/BROWN				    PIN 4 - BLUE/BROWN

SMH310:                                  		                SMH710/DMH320:

PIN 1 - SHIELD (MIC)                 		       	    PIN 1 – SHIELD (MIC)

PIN 2 - WHITE (MIC) Next to Shield     	   	    PIN 2 - RED (MIC) Next to Shield

PIN 3 - SHIELD (SPKR)     			    PIN 3 – SHIELD (SPKR)

PIN 4 – RED (SPKR) Next to Shield + Red  	    PIN 4 – RED/WHITE (SPKR) Next to Shield + Red

5-PIN FEMALE (BINAURAL - EACH CIRCUIT IN ONE EAR):

SMH210:                                       DMH220:	          DMH920:	          DMH320:

PIN 1 - BLUE/SHEILD (- GROUND)    PIN 1 – SHIELD        PIN 1 - GREEN/SHELD     PIN 1 – SHEILD (Same cable as Red)

PIN 2 – RED (+ MIC)                         PIN 2 - RED            PIN 2 – YELLOW              PIN 2 - RED

PIN 3 - GRAY (- SPKR)                     PIN 3 – BLUE           PIN 3 - RED/ORANGE      PIN 3 – 2 SHIELDS (That come together)

PIN 4 – YELLOW (+ SPKR)                PIN 4 – GRAY           PIN 4 – BLUE                  PIN 4 - WHITE

PIN 5 – JUMPER FROM PIN 4            PIN 5 – YELLOW       PIN 5 – BROWN	           PIN 5 - RED

5-PIN MALE (BINAURAL – RTS BELTPACKS):

SMH710					DMH320

PIN 1 – SHIELD (Same cable as Red)                   	PIN 1 – SHEILD (Same cable as Red)

PIN 2 – RED					PIN 2 - RED

PIN 3 – SHIELD (Same cable as Red & White)	PIN 3 – 2 SHIELDS (That come together)

PIN 4 – RED					PIN 4 - WHITE

PIN 5 – WHITE					PIN 5 - RED

TELEPHONE HANDSETS 4-PIN FEMALE:

HH10AB:                                             HH10BB:

PIN 1 - SHIELD (GRND MIC-)		PIN 1 - SHIELD/BLUE  (MIC-)

PIN 2 - WHITE (MIC+)			PIN 2 - RED (MIC+)

PIN 3 – BLACK (EAR-)			PIN 3 - GREY (EAR-)

PIN 4 - RED (EAR+)			PIN 4  -YELLOW (EAR+)