Village of Hamler Water Department

Annual Drinking Water Quality Report for 2025

# The Village of Hamler has a current, conditional license to operate our water system PWS ID: OH3500312

The Village of Hamler has prepared this report to provide information to you, the consumer, on the quality of our drinking water. Included within this report is the general health information, water quality test results, how to participate in the decisions concerning your drinking water and water system contacts.

Source Water Information. The source of the drinking water for the Village of Hamler is wells. These wells are over 600 feet deep and are of sulfur content of over 36 ppm and are located just north and east of the water plant. The source is called ground water; this is collected from the wells in an unnamed aquifer of clay, loam, and slate rock. This water is treated in the water treatment plant located at 500 East Hubbard St. to ensure its safety and is delivered to you in an extensive underground piping system.

The aquifer that supplies drinking water to the Village of Hamler has a low susceptibility to contamination due to the low sensitivity of the aquifer in which the drinking water well is located. This does not mean that this well field cannot become contaminated; only that likelihood of contamination is relatively low. Future contamination can be avoided by implementing protective measures. For more information, please call Ken Griffith, Water Superintendent, at 419-274-7651.

The source of drinking water, both tap water and bottled water, include rivers, lakes, streams, ponds, reservoirs, springs, and wells. As water travels over the surface of the land or through the ground, it dissolves naturally occurring minerals and, in some cases, radioactive materials, and can pick up substances from the presence of animal or from human activity.

Health Information. Contaminants that may be present in source water include: (A) Microbial contaminants, such as viruses and bacteria, which may come from sewage treatment plants, septic systems, agriculture livestock operations and wildlife; (B) Inorganic contaminates, such as salts and metals, which can be naturally occurring or result from urban storm water runoff, industrial or domestic wastewater discharge, oil and gas production, mining or farming; (C) Pesticides and herbicides which may come from a variety of sources such as agriculture, urban water runoff, and residential uses; (D) Organic chemical contaminants, including synthetic and volatile organic chemicals, which are by-products of industrial processes and petroleum production, and can also come from gas stations, urban storm water runoff, and septic systems; (E) In order to ensure that tap water is safe to drink, the EPA prescribes regulations which limit the amounts of certain contaminants in the water provided by public water systems. FDA regulations establish limits for contaminants in the bottled water which must provide the same protection for public health.

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants. The presence of contaminants does not necessarily indicate that water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the Environmental Protection Agency's Safe Drinking Water Hotline at 1-800-426-4791.

Who needs to take special precautions? Some people may be more vulnerable to contaminants in drinking water than the general population. Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplant, people with HIV/AIDS or other immune system disorders, some elderly and infants can be particularly at risk from infection. These people should seek advice about drinking water from their health care providers. EPA/CDC guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline at 1-800-4264791.

How do I participate in decisions concerning my drinking water? We want our customers to be informed about their water utility. If you want to learn more, please feel free to attend the regular monthly meeting of the Board of Public Affairs on the First Monday of the month at 5:30 p.m. in the Hamler Municipal Building located at 500 East Hubbard Street. You may also call Ken Griffith, Water Superintendent, at 419-274-7651.

About your drinking water. The EPA requires regular sampling to ensure drinking water safety. The Village of Hamler conducted samples for lead, copper, THHMs, HAAS, bacteria nitrate in 2025.  The Ohio EPA requires us to monitor for some contaminants less than once per year because the concentrations of these contaminants do not change frequently. Some of our data, though accurate, is more than one year old.

Maximum Contaminant Level (MCL) Exceedance: Hamler Village was in violation of exceeding the MCL of 80 ppb during TTHM during 1 st, 2nd, 3rd, and 4th quarters of 2025; the levels found during 2025 ranged from 62.3 – 131.7 ppb. You do not need to use an alternative (e.g., bottled) water supply. However, if you have specific health concerns, consult your doctor. The levels detected do not pose an immediate risk to your health. Some people who drink the water containing trihalomethanes in excess of the MCL over many years may experience problems with their liver, kidneys or central nervous system, and may have an increased risk of getting cancer. Hamler Village has been working with consultants and the Ohio EPA to address and resolve the MCL violations

Hamler Village also had a few CCR content deficiencies in 2024, which are summarized below. These were reported incorrectly on last year’s report.  If you have detailed questions regarding this information, please contact us.

- The report did not provide information regarding the lead service line inventory.

- Certain ongoing conditions or violations continue to be met.  While the report mentioned conditioned LTO, the statement was not explained.  Statements similar to the following were required to have been included in the CCR:

- “We have a conditioned license to operate our public water system.  The conditions require us to address ongoing violations.  For more information on these conditions, please contact (name and phone number)”

- The required Table of Detected Contaminants was incomplete and/or inaccurate in the report.  Please see Section 8 of the Consumer Confidence Report Template and Instructions for more information about what is required to be included in the Table of Detected Contaminants.

- The Haloacetic Acids (HAAS) data reported in the 2024 CCR was incorrect.

- The reported Level Found of 8.53 ppb should have been reported as 10.50 ppb.

- The reported Range of Detection of 6.0 – 8.5 ppb should have been reported as 0-9.6 ppb.

- The Total Trihalomethanes (TTHM) data reported in the 2024 CCR was incorrect.

- The reported Range of Detection of 65 – 111.9 ppb should have been reported as 69.10 – 112.3 ppb.

- The 90th percentile values for lead and copper were incorrect.  Ohio EPA has a worksheet online to assist PWSs in the calculation.

- The 90th percentile for Lead during the January – June 2024 monitoring period was reported as 15 ppb but should have been 0 ppb.

- The 90th percentile for Lead during the July – December 2024 monitoring period was reported as 15 ppb but should have been 0 ppb.

- The 90th percentile for Copper during the January – June 2024 monitoring period was reported as 1.30 ppm but should have been 0.120 ppm.

- The 90th percentile for Copper during the July – December 2024 monitoring period was reported as 1.30 ppm but should have been 0.210 ppm.

- The Lead statement regarding number of samples over the Action Level for the July – December 2024 monitoring period was missing from the report.

- The MCLG for Copper was not reported, however, it should be 1.3 ppm.

- The MCLG for Lead was not reported, however, it should be 0 ppb.

Table of Water Quality Test Result For the 2025 Consumer Confidence Report

| Contaminants (Units) | MCLG | MCL | Level Found | Range of Detections | Violation | Sample Year | Typical Source of Contaminants |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Disinfectant and Disinfectant By-Products |  |  |  |  |  |  |  |
| Total Chlorine (ppm) | MRDLG 4 | MRDL 4 | 0.86 | 0.33 to 1.2 | No |  | Water additive used to control microbes |
| Halo acetic Acids (HAA5) (ppb) | N/A | 60 | 9.05 | 6.0 – 10.6 | No | 2025 | By-product of drinking water disinfection |
| Total Trihalomethanes (TTHM) (ppb) |  | 80 | 95.58 | 62.3 – 131.7 | Yes | 2025 | By-product of drinking water disinfection |
| Inorganic Contaminants |  |  |  |  |  |  |  |
|  | 4 | 2 | 2.43 | N/A | Yes | 2024 | Erosion of natural deposits; Water additive which promotes strong teeth; Discharge from fertilizer and aluminum factories |
| Xylene (ppm) | 10 | 10 | 0.00085 ppm | 0.0 -0.00085 ppm | No | 2022 | Discharge from Petroleum factories; discharge from chemical factories |
| Lead and Copper |  |  |  |  |  |  |  |
| Contaminants (units) | Action Level (AL) | Individual Results over the AL |  | 90% of test levels were less than | Violation | Year Sampled | Typical source of Contaminants |
| Lead (ppb) | 15 ppb | 0 |  | 0.0 | No | July-Dec 2025 | Corrosion of household plumbing systems; erosion of natural deposits |
|  | _O_ samples were found to have lead levels in excess of the lead action level of 15 ppb. |  |  |  |  |  |  |
| Copper (ppm) | 1.3 ppm | 0 |  | O. 120 mg/l | No | July-Dec 2025 | Erosions of natural deposits; leaching from wood preservatives; Corrosions of household plumbing systems |
|  | _ O_ samples were found to have copper levels in excess of the copper action level of 1.3 ppm. |  |  |  |  |  |  |

Definitions of some of the terms contained within the report.

Maximum Contaminant Level Goal (MCLG): The level of a contaminant in the drinking water below which there is no known or expected risk to health. MCLGs allow for a margin of safety.

(OAC): Ohio Administrative Code

CL2: Chlorine that is a chemical used in drinking water for disinfection purpose and is mg/l.

Maximum Contaminant Level (MCL): The highest level of a contaminant that is allowed in the drinking water. MCLs are set as close to the MCLGs as feasible using the best available treatment technology.

Maximum Residual Disinfectant Level (MRDL): The highest residual disinfectant level allowed.

Maximum Residual Disinfectant Level Goal (MRDLG): The level of residual disinfectant below which there is no known or expected risk to health.

Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment of other requirements which a water system must follows. N/A: Not Applicable NA: Not Available

(ppb): Parts per Billion or microgram per liter (ug/l) is a unit of measure for concentration of a contaminant. A part per billion corresponds to one second in 31.7 years; pCi/l if Picocuries per Liter.

(ppm): Parts per Million or Milligrams per Liter (mg/l) are units of measure for concentration of a contaminant. A part per million corresponds to one second in a more than 11.5 days.

Less than “<” symbol:  A symbol which means ‘less than’.  A result of “<5” means that the lowest level detected was 5 and the contaminant in that sample was not detected.

The Village of Hamler exceeds the secondary limit for fluoride. The secondary MCL for fluoride is 2.0 mg/l, and our result was 2.43 mg/l. The Ohio EPA requires us to send out a public notification letter to all customers.

The Village of Hamler is required to monitor for routine coliform bacteria samples once per month. This was done for all 12 months in 2025. All samples came back negative for bacteria and the water is safe for drinking. If you have, any questions please call 419-274-7651 at the water plant.

The Village of Hamler is required to monitor for Total trihalomethane’s.  We were in violation for all four quarters of 2025.  We will begin construction of the GAC filtration building in the fall of 2026 with operation to begin in the spring of 2027.  This process will adsorb the disinfection byproduct causing this violation.

Lead Education. "If present", elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. The Village of Hamler Water Department is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline at 800-426-4791 or at http://www.epa.gov/safewater/lead.

Our distribution system has no lead, galvanized requiring replacement, or lead status unknown service lines.  To determine this, we used the following sources: Historical records and visual inspections.

In 2020, our PWS was sampled as part of the State of Ohio's Drinking Water Per- and Polyfluoroalkyl Substances (PFAS) Sampling Initiative. Six PFAS compounds were sampled, and none were detected in our finished drinking water. For more information about PFAS, please visit pfas.ohio.gov.  Beginning in 2025, routine sampling is required and results will be posted in 2026 CCR.

The EPA recently completed a study of the Village of Hamler’s source of drinking water to identify potential contaminant sources and provide guidance on protecting the drinking water source. According to this study, the aquifer (water-rich zone) that supplies water to the Village of Hamler has a low susceptibility to contamination. This determination is based on the following:

- Presence of a thick protective layer of clay overlying the aquifer,

- Significant depth (over 46 feet below ground surface) of the aquifer,

- No evidence to suggest that the ground water has been impacted by any significant levels of chemical contaminants from human activities, and no apparent significant potential contaminant sources in the protection area.

The susceptibility means that under currently existing conditions, the likelihood of the aquifer becoming contaminated is low. This likelihood can be minimized by implementing appropriate protective measures. More information about the source water assessment or what consumers can do to help protect the aquifer is available by calling Ken Griffith, Water Superintendent at 419-274-7651 or email at hamlerwater@gmail.com.

In 2025, the Village of Hamler operated under a conditioned license to operate our public water system. The conditioned license was issued as a result of the violations for exceeding the maximum contaminant level (MCL) standard of 0.080 mg/L as established in the Ohio Administrative Code (OAC) section 3745-81 for TTHM.  The construction of our GAC water filtration system will begin in September of 2026 to resolve these issues.

Hamler Board of Public Affairs

500 East Hubbard St.

P.O. Box 435

Hamler, Ohio 43524

419-274-7651