TENT CHECKLIST

(IFC)

www.CROWDMANAGERS.COM

- Permit required?

- Certificate from approved testing agency verifying flame resistance (3104.2)

- Permanently affixed label indicating size, type of fabric (3104.3)

- Adequate number of exits provided (3103.12)

- Exits spaced evenly around tent (3103.12.1)

- Exit signs provided for exits serving 50 or more (3103.12.6)

- No guy wires, ropes, or other support components projecting below 8 feet above the exit surface.

- Tent is adequately braced, anchored.  Documentation submitted (if required by code official) (3103.3.9)

- No Smoking Signs Posted (3104.6)

- No cooking or open flame in or within 20 feet if open to the public (3104.7)

- No fireworks within 100 ft.  (3104.8)

- 3 ft. clearance between fabric and contents (3104.11)

- Portable fire extinguishers provided   (3104.12)

- No heating or cooking equipment within 10 ft. of exits (3104.15.3)

- Outdoor cooking that produces grease laden vapors or sparks located at least 20 ft. from tents (3104.15.6)

- LP gas containers located outside (3104.16.2)

- Small (under 500 gal.) LP containers at least 10 ft. from tent (3104.16.2.1)

- Large (over 500 gal.) LP containers at least 25 ft. from tent (3104.16.2.2)

- LP gas containers secured and protected from damage (3104.16.3)

- No flammable liquids or liquid fueled equipment inside tents; storage of flammable and combustible liquids at least 50 ft. away (3104.17)

- No refueling within 20 ft. of the tent (3104.17.3)

- One trained crowd manager for each 250 occupants provided (3104.20.2)

- Combustible vegetation removed within 30 ft. of tent (3104.21)

- Combustible waste removed from tent and area within 30 ft. of tent (3104.22