SkillsUSA 2025 Additive Manufacturing State Challenge

Make It Run

Welcome to the “Make It Run” challenge!

The task at hand is to design and fully print a 4 wheeled vehicle powered only by a single rubber band. The vehicles will then be tested on a “track” for functionality, and additional scoring.

Design Considerations:

- Interlocking parts

- Printed Assemblies

- Snap fits

- Printable Tolerances

- Motion

- Kinetic to Potential Energy

Example of Basic Design

Competition Requirements

- The design must be completely 3D printed.

- The design must not contain any outside hardware (axles, screws, washers

- The design can be 3d printed using any technology.

- The design must contain a legibly printed team number/name

- The design can contain 3D printed bodies that are assembled after printing for the final part.

- The final design can use super glue for assembly, for a loss of points (see grading rubric)

- Parts must have printed wheels

- The design must contain at least 3 moving parts

- Wheels can not be larger than 3 inches in diameter

- The design must be powered only by a single rubber band

- The printed design must have moving bodies.

- The design must not exceed 6” x 4” x 4”

- 3D Printed Design - Students must create a design that:

  - Is original and designed by contestant

  - Print all parts in less than 12 hours total

  - Uses less than 5 cubic inches of model and/or support combined for all parts.

- Students must submit files to be printed via State designated file share site no later than March 5, 2025 at 4:00pm. Final prints will be delivered on the day of the contest so that students can test, assemble/modify and be evaluated.

Tips for Competitors

Here are some tips to maximize the points awarded to you:

- Build debossed text on a horizontal surface for best results. This may require building the part on its edge or standing up.

- Utilize soluble support structures for print in place assemblies

- Understand the achievable design tolerance of your printer for print in place, or hand assembled designs to allow motion between parts.

- Leverage post-processing techniques to smooth printed bodies.

- Additional moving parts may add to your score but can produce more points of failure on the final assembly.

- Use online resources (YouTube, GrabCAD Tutorials)

- Whenever intellectual property (IP) deters you from a project, try using approximate geometries to communicate the design intent.

- Optional design for additive manufacturing learning resources:

- Stratasys Think Additively™ Masterclass:

  - https://youtube.com/playlist?list=PLUYaY5EIPtNBdU-s-7l9rl05lBHHlTarI

State Competition Procedure

Before or on contest day:

- Students submit Engineering Notebook (Engineering notebook guidelines below)

- Students submit print files in both CAD (.step, .iges, .sldprt, etc.) and mesh (STL, 3MF, OBJ, etc) format to [https://wvk12-my.sharepoint.com/:f:/g/personal/mwilkinson_k12_wv_us/EiUkVpKL1nVMkvqjlCz7NxoBcRhLIHQLZQazPTK4fqYLBQ]

- Students submit physical parts

- Students submit final assembly if applicable

- Students submit their Presentation

State Competition Judging Criteria

- The Engineering Notebook should contain robust content, including at a minimum the following:

  - Be clearly labeled with contestant name(s), date and page # on each page

  - Begin with a problem statement

  - Include discovery and documentation of approach to solve problem

  - Include sketched design concepts with critical features labeled

  - Critical dimensions clearly labeled in design sketch

  - Considerations for designing for additive manufacturing distinctly addressed (i.e. part strength, part orientation) especially including any expected risks during printing

  - Screenshots of the print time and material usage for all printed parts

  - Design decisions and alternatives are documented and evaluated thoughtfully

- The design must adhere to the Competition Requirements stated in the prior page.

- Quality of final assembly

  - Does it perform the function in the manner it was designed to do?

  - Does it meet all requirements in contest guidelines?

  - Do inserted components or multiple printed parts mate together properly?

  - Did the students design the part with additive manufacturing in mind?

  - Is there sufficient tolerance between parts for movement?

- The design must illustrate best practices for “design for additive manufacturing (DFAM)”. Below are some potential DFAM metrics to optimize for.

  - Build Time

  - Post-Processing/Support Removal Time

  - Functionality Optimization (gear ratio, pliability, strength, etc.)

  - Monetary Savings

  - Material Consumption

  - Energy Usage

  - Component Consolidation (lack of store-bought hardware)

  - Lightweighting for Ergonomics

- Presentation Criteria

  - The team clearly describes their understanding of the problem to be solved.

  - Design Process: good design logic is used for key design choices. Intentional and well-communicated

  - The presentation is professional and well-rehearsed

  - The presentation emphasizes quantitative improvements (measured and estimated) of the time, quality, or cost of the improvement as well as any DFAM tactics employed.

  - Practical evaluation: team demonstrates visually (videos, photos, drawings, animation, etc) the task they improved, both before and after.

- Racetrack Setup

  - Track will have a stating line and distance markers at 1”, 6”,12”, and with marks every foot after up to 6 feet. Ruler or measuring tape will be used for final measurement above 12”.

  - Front tire/tires must begin behind the starting line.

  - A small nudge can be used to help get the car moving (see grading rubric)

  - Each design will have 2 chances to run on the track. The better of the two scores will be used for final judging.

  - Final distance of vehicle is measured where the front wheels touch the ground