Planned Event:

The SCTA Board, with a vote from the member clubs, has approved a test run of 2 vehicles of the El Mirage Speed Trials 125 Club during the October 2026 SCTA El Mirage Event.

Purpose:

Complete a successful test run of the 125 Club as a proof of concept and work out issue prior to submitting changes to the SCTA Rules and Records Book, Bylaws and El Mirage Procedures for the 2027 season.

Participants:

We currently have 2 volunteers who will be running their vehicles.

-	Valerie Fenn				-       Mark Lintner

Deviation from current SCTA El Mirage Procedures:

All racers will follow existing SCTA El Mirage Procedures with the following exceptions:

- Race vehicles will be allowed to operate under their own power but must follow an escort vehicle and follow all posted speed limits.

  - Only driven in the following situations:

    - Inspection to pit area

    - Pit area to pre-stage

    - End of run down the return road to the pit area

- Racers will be called to pre-stage with the 76 to 100 and have a yellow pre-stage number received from registration (100-1 and 100-2)

- Pre-stage will provide a yellow escort vehicle number sheet for the escort vehicle.

- No roll cage is required for this class.

- No arm restraints are required for this class.

- Head/Neck Restraints are required for this class.

- Production sprung seats are allowed.

- Production 3-point are allowed.

- No fire suppression system is required for this class.

- Door and steering wheel locks will not be disabled.

- Additional throttle return springs or over center positive stop modifications are not required for this class.

- No flywheel shield, Automatic / Planetary Transmission Shield, or driveshaft loop is required for this class.

- OEM lug nuts are allowed in this class.

- OEM valve stems are allowed in this class.

- Push Bar or Tow Rope attachment point are not required.

- Frameless windows are allowed in this class.

- Main battery disconnect switch are not required for this class.

Additional Supervision:

Representatives from the SCTA El Mirage Speed Trials planning team will be coordinating with all the racers to get them through inspection, registration, pre-stage, starting line, start, run, and return to the pit area.