Crazy4Bouncing Method Statement

Project overview

- Purpose: To set up, operate, and dismantle inflatable bouncy castles safely for hired events.

- Scope: Covers all operations related to the hire of inflatable equipment, including transport, installation, supervision, and removal.

- Key personnel: Name(s) of trained and competent staff responsible for carrying out the work. All staff must be familiar with and sign this document.

- Documentation: This method statement is to be used in conjunction with the company's risk assessment, equipment manuals, and daily safety checklist.

1. Site arrival and access

- Vehicle safety: All drivers will adhere to standard road safety regulations. When driving on-site (e.g., across a field), drivers will use hazard lights and maintain a maximum speed of 10 mph.

- Site inspection: Before unloading, a final suitability check of the setup area will be conducted. This includes:

  - Verifying that the site is a flat, level area free of any abrasive objects like large stones, sticks, or glass.

  - Checking for sufficient overhead clearance from trees, power lines, or lights.

  - Ensuring adequate clear space around the inflatable (at least 2 metres from closed sides and 4 metres from open sides).

2. Equipment installation and setup

- Unloading: Equipment will be unloaded by hand or with a sack truck, taking care to protect the inflatable from damage.

- Positioning: A ground sheet will be laid down, and the inflatable unrolled and positioned correctly.

- Inflation:

  - Check that the correct blower is being used as per the manufacturer's instructions.

  - Connect the blower to the inflatable's inflation tube, ensuring a secure fit.

  - Connect power via an RCD circuit breaker.

  - Inflate the unit, ensuring no one is on or near it during the process.

- Anchoring:

  - For outdoor use, all manufacturer-specified anchor points must be used.

  - Securely fasten the anchor ropes to ground stakes that are at least 380mm long and 16mm wide.

  - If on a hard surface, use ballast (e.g., sandbags) weighing a minimum of 163kg per anchor point.

  - For indoor use, follow manufacturer guidance on using ballast weights.

- Final checks:

  - Visually inspect the inflatable for tears, rips, or misshapen features after inflation.

  - Confirm all anchor points are secure.

  - Position impact-absorbing safety mats at the entrance and exit.

3. Safe operation and supervision

- Continuous supervision: The inflatable must be constantly supervised by a competent, trained attendant aged 16 or over.

- User rules: The attendant will enforce strict rules for all users, including:

  - Separating users by size and age.

  - Restricting the number of users to the manufacturer's limit.

  - No shoes, glasses, sharp objects, or jewelry on the inflatable.

  - No climbing on walls, no rough play, and no flips.

  - No food, drinks, or chewing gum.

- Weather monitoring:

  - The wind speed will be measured hourly using a handheld anemometer.

  - If wind speeds exceed 24 mph (Force 5), or if there are strong gusts, the inflatable must be immediately deflated.

- Emergency procedures: In the event of a power failure, the attendant will immediately and calmly assist users to exit the deflating unit.

4. Dismantling and removal

- Evacuation: Ensure all users are off the inflatable before beginning the deflation process.

- Deflation: Disconnect the blower and allow the unit to deflate.

- Cleaning: The inflatable and all accessories will be wiped down and cleaned before rolling up.

- Packing: The unit will be folded and rolled up in a controlled manner to prevent strain or injury.

- Loading: The equipment will be loaded back onto the vehicle carefully.

- Site check: The area will be checked for any remaining debris or equipment before departure.

5. Emergency contacts and procedures

- Accidents: In case of an accident, a trained first-aider will administer care and contact emergency services if necessary. The incident will be recorded in the company's accident book.

- Equipment damage: Any defects or damage to equipment will be reported and the item taken out of service until repaired by a competent person.