MAGKRA Rookie Caged CLASS (5-8)

This is a learning class only/controlled speed class - If you want to race in a

competitive class, this class is not for you!

#250 min weight Predator

▪ Harbor Freight Predator engines only, NO DUCAR OR Box Stock Project Predator 212cc engines (SKU 69730) allowed.

▪6.5HP Harbor freight  Predator as supplied by manufacturer.

▪ Factory unaltered muffler only

▪ Stock fuel tank may be removed and replaced with a top plate and approved pulse type single outlet

fuel pump,  pump must be pulsed only from rocker cover, sided cover, or block. Replacement tank

may not be mounted to roll cage, must be mounted to chassis frame / floor pan etc. in a secure /

safe manner.

▪ Stock fuel tank may be used as well but must be unaltered other than fuel fill cap

(see below) ▪ All fuel line connections must be secured by clamps or similar.

▪ Governor MUST be complete and functional, including the governor set screw / No MAXIMUM RPM

CHECK in  2025. Governor must fully surge at some point if checked. Stock air filter, stock muffler, and

factory supplied  spark plug must be used and unaltered. Stock main and low speed jets only. NO GO

0.031 main and 0.018 low  speed jet. Low oil sensor must remain in the engine but may be

disconnected. Red 0.375 NO GO unaltered ARC or  Speed karts restrictor plate MUST be used. One

intake gasket on each side of restrictor plate MUST be used.  ▪ Fuel cap may be drilled for ventilation /

Max of three holes 0.125 max hole sizes.

2. Hemi Predator SP carburetor is the ONLY legal carburetor that may be used in this class /

Unaltered

▪ Vent hose from fuel tank to air box may be disconnected but must be plugged and visible. ▪ NO

other alterations to predator engine are allowed /will be compared to known stock.

▪ MAGKRA may at any time exchange engines with a new engine. If the engine is found to be legal it will  be

returned to competitor the following event and the new engine returned to MAGKRA. If found illegal

competitor will be suspended from track for one month or two race events.

20

▪ 12 clutch driver only! / Open clutch / Any disc or drum clutch may be used / NO wet clutches allowed,

61-70T rear sprocket only, 35 pitch chain only.

▪ Hoosier treaded tire only, max outside diameter 36” RR tire after race.

Predator engine MUST both run the spec gearing at all times.

▪ See chassis rules, cage may be clamped to frame, or spring loaded and properly secured. Cage may not

be  welded to frame at any point. Cage must be mounted in four spots two ahead of the driver seat

and two behind  the driver seat. Subject to track official inspection

▪ No enclosures of any type allowed!

This class is intended to be a non-competitive class, this class is intended for

the new young racer to learn to: Enter the track, Exit the track, learn how to

race around others, learn what the lights and flags mean and most importantly

how to respect other racers and track officials.