Decoustics "Ceilencio” Ceiling System

## QUADRILLO QPP-19 CEILING PANELS

# Guide Specification

#### PART 1 GENERAL

  - Scope

- Include all labour, materials, and plant as required for the fabrication and delivery of acoustical wall system complete in all respects as described and specified herein.

  - Related Work by Other

- Gypsum board partition, and wall and ceiling structure.

- Paint finish of ceiling surface behind and between panelling.

- Lighting fixtures and electrical services and connections.

- Air handling, heating and air conditioning, plumbing and other mechanical services as required.

- Public address system, speakers, wiring, support and installation.

- Life safety equipment.

- Ceilings by others not included in this scope.

  - Quality Assurance

- Manufacturer & Installer: Firm manufacturing the specified product shall have adequate capacity required for projects listed and have successfully completed similar projects for a period of not less than five years. The Installer should be approved by the Manufacturer as qualified to perform work required.

- Reference Standards:  Conform to all governing laws, building codes, and the following performance criteria:

  - Fire Performance Characteristics: Provide acoustic ceiling panels with surface-burning characteristics as determined by testing panel components in accordance with ASTM E84 test procedures (building code requirements may necessitate composite panel testing using identical materials and construction representative of a typical installation, using the specified finish(es) - Decoustics has a considerable number of composite ASTM E84 panel tests on file. ASTM E-84 testing must be performed by a testing organization acceptable to authorities having jurisdiction.

ASTM E-84		Classification		      	Class "A" or "1"

Flame Spread:		25 or less

Smoke Developed:		450 or less

  - Acoustical Performance Characteristics: Provide ceiling panels with acoustical absorption characteristics as indicated in Part 2, which have been determined by testing fully assembled production material in accordance with ASTM C-423 (Type E 400 mounting method as defined by ASTME-795) by a testing organization acceptable to authorities having jurisdiction.  Approved testing organization must be independent of the manufacturer.

  - Seismic Performance: The Decoustics Ceilencio Ceiling systems with Quadrillo - Wood panels shall be seismically tested as per procedures prescribed in Chapter 13 of the ASCE 2010-07 and the relevant ICC-ES standards. Shake table testing of ceiling systems using ICC-ES AC156 (ICC-ES 2015) protocol must be conducted, witnessed and documented by third party practicing structural engineers expert at an accredited laboratory and pass the most severe level of shaking prescribed in the building codes without any damage.

  - Submittals

- Submit to the Architect of record, three (3) complete sets of CAD generated shop drawings prepared by the manufacturer showing all necessary details and dimension requirements which will subsequently be field verified and revised as required by the Architect.

- Samples: Submit (3 sets) manufacturers standard 8" X 11" (203mm x 280mm) sample panels of each type of product as specified in Part 2 to the Owner for approval.  Product shall be original production material in veneer finish specified for final use.

- Certification: Submit to the owner a certificate of compliance to specified acoustical and fire performance criteria as stated section 1.03 and Part 2 of this specification, signed by an officer of the panel manufacturer, and attach independent laboratory test results for each product used, showing that the products supplied as components meet or exceed the specified requirements. Submit additional test results to owner as requested detailing compliance to updated code requirements.

- Prior Approval: Manufacturers not listed in section 2.01 of this specification and wishing to be submitted as an equal on the project must submit information in accordance with section 1.04 of this specification fifteen days prior to the bid date as outlined in the contract documents. The information submitted must give evidence to show that the alternative product equals or exceeds the attributes and performance of the specified product.

- Manufacturers Approval: The manufacturer shall have the right to approve the selection of the installing contractor and to verify that said contractor has sufficient experience and expertise to complete the project in a satisfactory manner.

- Single Source: It is the clear intent of this specification to provide a complete, fully integrated system, supplied by a single company. “Stick built” parts and pieces from various and different manufacturers will not be accepted. All custom acoustical wall and ceiling panels shall be purchased from a single supplier

  - Product Delivery, Storage and Handling

- Deliver fabricated units and related components to the site for installation in accordance with a reasonable schedule furnished by the contractor. On-site storage shall be such as to assure that all panels and associated materials are protected from damage.

- Prior to panel installation the site must be free of all wet and dusty trades and the climatic conditions stabilized to normal operational levels. Panels shall be allowed to stabilize on site 72 hours prior to installation.

- PANELS MUST BE STORED, INSTALLED AND MAINTAINED ONLY IN SECURE AMBIENT ENVIROMENT (HUMIDITY MIN. 35% - MAX. 55%, TEMPERATURE NOT TO EXCEED 80º F (27°C).

  - Guarantee

- Furnish to the Architect in the Owner's name, the manufacturers written guarantee covering the products supplied against defects in materials and workmanship under normal operating conditions for a period of one year from the date of shipment. Submit certificates of compliance showing warranty period by dates for each project completed to the Owner.

- Manufacturer must demonstrate the ability to fully bond the project for its full material value.

PART 2 PRODUCTS

  - Quadrillo Acoustic Panels

Furnish and deliver prefabricated Quadrillo QPP-19 ceiling panels as described in this section for installation in areas as shown on drawings meeting or exceeding the following requirements:

- Description:  Quadrillo panels, as manufactured by:

Decoustics

61, Royal Group Crescent,		TEL: (800) 387-3809

Woodbridge, Ontario		FAX: (905) 652-2505

Canada, L4H 1X9			Email: sales@decoustics.com

- The panels shall be Type QPP19 comprised of a sandwich construction of the following: A face sheet of selected real wood veneer laminated to 1/4" (6mm) medium density FR treated fiber board perforated by means of transverse “V” routing on 5mm centers. The membrane is fully bonded to a 6 to 7 lb./cu.ft (96 to 112 kg/cu.m.) density acoustically absorptive core material of 3/4" (19 mm) thickness. The panel shall be backed with 1/8" (3mm) perforated hardboard fully bonded to the core material. The overall panel thickness is 1 1/8” (28.6 mm). The perimeter shall be constructed of particle core of adequate thickness to match the core material. The boards shall be fabricated to sizes required by field dimensions supplied by the installing contractor and will not vary from determined sizes by more than +/- 1/16" (2mm) vertically and horizontally. Intermediate framing support shall be incorporated on 24" (610mm) centres or as required. The panel shall be constructed using fire retardant core components (refer to paragraph 1.03 B 1 for ASTM E 84 (fire) information). The panels shall be installed into the extruded aluminum Decoustics Ceilencio Custom grid system. The grid system shall consist of main tees and cross tees, which shall incorporate a continuous “panel location” fin to ensure correct panel alignment during installation and future access. The grid system shall be completely engineered and fabricated in the factory, to avoid any field cutting of the grid components. The extruded aluminum Ceilencio Custom grid system shall be manufactured as shown on the architectural and approved shop drawings. The lower edge of the extruded aluminum grid (G grid) shall be supplied painted black as required.

- Veneer to be:

- Select veneer from Manufacturers’ standard range of lacquer finished real wood veneers OR

- Architect / Designer specified finish

Source and laminate Architects specified veneer species, stained and lacquered to match Architects sample

Unless otherwise specified, veneer will be quarter cut, slip matched. Veneer will not be sequenced.

Clear lacquer finish to 30% sheen.

- Installation shall be by use of torsion springs, field engaged into factory supplied spring retainers, and field installed in the correct location on the back of the panel. This panel assembly is then lifted into place, and the torsion springs are engaged into the factory supplied “butterflies” which have been field installed on the already suspended factory supplied grid. The panel is then gently lifted into place as the torsion springs take the load.

All grid suspension hardware, hanger wires, rods, anchors, mouldings, etc., are to be supplied by the installing contractor.).

- Installation shall be in accordance with the manufacturers’ instructions and as shown on Decoustics shop drawings. Installer shall provide shimming and adjustments as required to maintain consistent alignment of joints and of finished panel faces.

- Panels shall have noise reduction coefficient values of the following when tested in accordance with section 1.03 of this specification.

Frequency (Hz)

Panel Type		125	250	500	1000	2000	4000	NRC	SAA

QPP-19 Type 5	0.78	0.77	0.61	0.86	1.04	0.70	0.80	0.82

- Suspended ceiling system with fully assembled panels as indicated shall be tested to the maximum level of the building code (SDS of 2.00g). There is to be no damage to the ceiling grid members or to the panels, with no panels dislodging after the maximum test level has been reached; per the requirements of the International (or California) Building Code and ASTM standards to be certified for all regions with high seismicity activity.

PART 3 INSPECTIONS AND APPROVAL

  - Adjustment and Replacement

- The Owner shall inspect the installation and product on completion. The manufacturer shall provide repair or replacement of components not conforming to requirements as stated herein and said work will then become bound by the terms of this specification.

- Installation labour for removal and replacement of product improperly installed and not conforming to specified installation instructions as detailed in section 1.03 and Part 2 and shown on plans, shall be the responsibility of the installing Contractor.