SOLO 16 PLANKS

Guide Specification

PART 1. GENERAL

  - Scope

- Include all labor, materials, and plant as required for the fabrication and delivery of acoustical wall/ceiling system complete in all respects as described and specified herein.

  - Related Work by Other

- Gypsum board partition and wall structure.

- Paint finish of wall surface behind and between panelling.

- Lighting fixtures and electrical services and connections.

- Other acoustical products.

  - Quality Assurance

- Manufacturer & Installer: Firm manufacturing the specified product shall have adequate capacity required for projects listed and have successfully completed similar projects for a period of not less than five years. The Installer should be approved by the manufacturer as qualified to perform work required.

- Reference Standards: Conform to all governing laws, building codes, and the following performance criteria:

  - Fire Performance Characteristics: Provide Solo planks with surface-burning characteristics as determined by testing panel components in accordance with ASTM E84 test procedures. ASTM E-84 testing must be performed by an independent testing organization acceptable to authorities having jurisdiction. Panels will be finished with Fire retardant clear lacquer.

ATM E-84		Classification		      	Class "A" or "1"

Flame Spread:		25 or less

Smoke Developed:		450 or less

  - Acoustical Performance Characteristics: Provide Solo planks with acoustical absorption characteristics as indicated in Part 2, which have been determined by testing fully assembled production material (using 96-112kg/cu.m. (6 - 7lb/cu.ft.) density fiber glass insulation) in accordance with ASTM C-423 (Type F5 mounting method as defined by ASTME-795) by a testing organization acceptable to authorities having jurisdiction. Approved testing organization must be independent of the manufacturer.

  - Submittals

- Submit to the Architect of record, three (3) complete sets of Solo plank data sheets showing all necessary details and dimensions.

- Samples: Submit (3 sets) manufacturers standard 8" X 5" sample planks of each type of product as specified in Part 2 to the Owner for approval. Product shall be original production material in veneer finish specified for final use.

- Certification: Submit to the owner a certificate of compliance to specified acoustical and fire performance criteria as stated section 1.03 and Part 2 of this specification, signed by an officer of the panel manufacturer and attach independent laboratory test results for each product used, showing that the products supplied as components meet or exceed the specified requirements. Submit additional test results to owner as requested detailing compliance to updated code requirements.

- Prior Approval: Manufacturers not listed in section 2.01 of this specification and wishing to be submitted as an equal on the project must submit information in accordance with section 1.04 of this specification fifteen days prior to the bid date as outlined in the contract documents. The information submitted must give evidence to show that the alternative product equals or exceeds the attributes and performance of the specified product.

- Manufacturers Approval: The manufacturer shall have the right to approve the selection of the installing contractor and to verify that said contractor has sufficient experience and expertise to complete the project in a satisfactory manner.

- Single Source: All custom acoustical wall and ceiling panels shall be purchased from a single supplier.

  - Product Delivery, Storage and Handling

- Deliver Solo planks and related components to the site for installation in accordance with a reasonable schedule furnished by the contractor. On-site storage shall be such as to assure that all panels and associated materials are protected from damage.

- Prior to panel installation the site must be free of all wet and dusty trades and the climatic conditions stabilized to normal operational levels. Planks shall be allowed to stabilize on site 72 hours prior to installation.

- PLANKS MUST BE STORED, INSTALLED AND MAINTAINED ONLY IN A SECURE AMBIENT ENVIRONMENT (HUMIDITY MIN. 35% - MAX. 55%, TEMPERATURE NOT TO EXCEED 80º F (27º C)).

  - Guarantee

- Furnish to the Architect in the Owner's name, the manufacturers written guarantee covering the products supplied against defects in materials and workmanship under normal operating conditions for a period of one year from the date of shipment.  Submit certificates of compliance showing warranty period by dates for each project completed to the Owner.

- All system components shall be manufactured and supplied by the named manufacturer. Individual components, provided by individual suppliers will not be considered or accepted.

PART 2 PRODUCTS

  - Acoustical Absorptive Panel

Furnish and deliver Decoustics SOLO planks as described in this section for installation in areas as shown on drawings meeting or exceeding the following requirements:

- Description: Decoustics SOLO PLANKS, as manufactured by:

Decoustics

61 Royal Group Crescent				TEL: (800) 387-3809

Woodbridge, Ontario				FAX: (905) 652-2505

Canada, L4H 1X9					Email: sales@decoustics.com

    - The planks shall be comprised of a single piece no added formaldehyde Class A MDF core of 16mm (5/8”) thickness faced with a real wood veneer. The structure is perforated by means of “V” routing on 16mm (5/8”) centers and intersecting perforations of 8mm (5/16”) diameter on 16mm (5/8”) centers. An acoustically transparent black mat/tissue is laminated to the back.

    - Veneer to be:

      - Select veneer from Manufacturers standard range of lacquer finished real wood veneers.

OR

      - Architect / Designer specified finish

Source and laminate Architects specified veneer species, stained and laquered to match Architects sample

Unless otherwise specified, veneer will be quarter cut, slip matched. Veneer will not be sequenced. Clear lacquer finish to 30% sheen

    - Solo planks of 192mm (7 9/16”) shall be installed by means of tongue and groove connection on hardwood blocking or metal furring horizontally mounted at 600mm (23 5/8”) centres. Semi ridged glass fiber acoustical insulation of adequate thickness required to meet the acoustic absorption requirements shall be installed between furring or horizontal wood blocking prior to panel installation. The boards shall be supplied with a black facing on the back side to prevent fiber glass insulation color from “reading” through, and shall be site fabricated to sizes required and neatly fitted to adjacent materials. Trim perimeter as detailed. Installer shall provide for shimming and adjustment as required to maintain consistent alignment of joints and of finished panel faces

    - Solo Planks shall have noise reduction coefficient values of the following when tested in accordance with section 1.03 of this specification.

Frequency (Hz)

Description	Thickness	    125	   250	  500	 1000	2000	4000	NRC	SAA

Solo 16-25	1 5/8"  (41mm)   0.14  0.47	  1.04	 1.04	0.70	0.58	0.80	0.81

Solo 16-50	2 5/8"  (67mm)    0.32  0.88   1.16	 1.00	0.73	0.61	0.95	0.93

*Solo 16 with 1" (25mm) Space   0.06    0.13   0.32   0.76   0.60     0.40    0.45     0.45

*No acoustic absorption material used.

PART 3 INSPECTIONS AND APPROVAL

  - Adjustment and Replacement

- The Owner shall inspect the installation and product on completion. The manufacturer shall provide repair or replacement of components not conforming to requirements as stated herein and said work will then become bound by the terms of this specification.

- Installation labor for removal and replacement of product improperly installed and not conforming to specified installation instructions as detailed in section 1.03 and Part 2 and shown on plans, shall be the responsibility of the installing Contractor.

- Solo is a real wood veneer product. Every effort is made to maintain the overall appearance; however, natural variations in grain, texture, shade and aging may occur in varying site conditions. For these reasons, the manufacturer cannot guarantee the exact matching of grain, pattern and color.