SAN LEON MUNICIPAL UTILITY DISTRICT

443 24TH STREET

SAN LEON, TEXAS 77539

Emergency Procedure for the Water System

Before the Storm

Purpose:

This plan seeks to pressure a potable Water Supply both during a Hurricane and after a Storm passes.  In the event of a catastrophic Hurricane that may strike the Galveston San Leon area, the M.U.D. will secure the Water System by following the procedure below.

Step 1

1.	Make sure that all Water Tanks have been topped off 				(completely full).

2.	Make sure the Water Well is operational (in case it is needed 			after the storm).

3.	Check all Generators and assure that Diesel tanks are full.

Step 2

If and when the order for mandatory evacuation is given then; 	Complete 4, 5, and 6

4.	Isolate Tank Two (2) from the system (far back tank) (close all 			valves)

5.	Isolate pumps 1 and 2 from the system (close valves and turn 			off power)

6.	Make sure the Water Well is operational (in case it is needed 			after the storm)

By following items 1-6, the system will be in a minimal operational mode. During a storm, the use of Ground Storage Tank One and the Generator will keep the system in service. Pumps 3 and 4 will remain on to keep the system pressurized for as long as water is available in the ground storage.

Step 3

1.	Make sure that all of the vehicles and equipment is stored 			in a safe location.

2.	Make sure that essential records are backed up at a safe location 		and secure the District’s office. Take the computers to a secured 		location.

EMERGENCY MANAGEMENT PLAN

I.	Purpose

The purpose of this plan is to provide service plans for emergencies 	when extra 	measures must be taken to protect life and property.

II.	Situations and Assumptions

A.	Situations

San Leon Municipal Utility District is subject to a number of 			disaster circumstances that could occur locally and would create 		a need for emergency services.

B.	Assumptions

1.	All District equipment and personnel will be available to 				cope with any anticipated disaster until evacuation is 				necessary.

2.	Without assistance, the District department does not have 			sufficient resources to cope with a disaster.

3.	Local contractors and industry have resources to assist 				District recovery efforts.

4.	Assistance may be available from outside the jurisdiction 			through mutual aid and other existing agreements.

5. 	Repair and restoration of essential services and vital 				facilities is possible and such that the District can 					reasonably be able to accomplish this task.

6.	The District will be able to accomplish debris clearance 				with assistance in an emergency.

III.	Concept of Operations

A.	General

Overall responsibility for providing services during 				emergencies rests with the District Managers.  Authority to 			implement this Plan has been delegated by him/her.  The 				District Managers will manage his/her resources and equipment 		and coordinate District activities with all other area agents.

B.	Phases of Management

1.	Litigation

a.	Train personnel in emergency procedures.

b.	Work with legislative body to ensure that 						ordinances are created to protect District systems.

c.	Identify vulnerabilities in the District.

d.	Initiate local private contractors who can provide 					support during emergencies.  Acquaint them with 					emergency plans and procedures.

e.	Participate in design and execution of emergency 					preparedness exercises.

f.	Provide input into after-action reviews to improve 					preparedness, response, and recovery capabilities.

g.	Work with Galveston County to ensure that new 					constructions do not increase hazard of 						vulnerability threat.

h.	Work with appropriate legislative body to improve 				building codes.

2.	Preparedness

a.	Ensure that infrastructure (storm sewers, water 					distribution system, sewer treatment, etc.) is in 					good repair.

b.	Ensure that debris removal equipment is in good 					repair.

c.	Check with Galveston County Sheriff’s office and 					Department of Public Safety to see that adequate 					barrier and road block materials and equipment are 				available.

d.	Review and update all public work maps of 						jurisdiction.  Notify the Galveston County 						Emergency Management Coordinator of 						availability of updated maps for use.

e.	Review emergency staffing plans.

f.	Protect all equipment against damage.

g.	Assist in formation of damage survey teams.

h.	Place standby equipment in operational readiness.

i.	Coordinate communications procedures with 					Galveston County Emergency Management.

j.	Review contingency plans and coordinate task 					assignments with other agencies and volunteer 					groups.

k.	Develop procedures to support accomplishment of 					tasks outlined in this Plan.

3.	Response

a.	Ensure safety of all District personnel.

b.	Survey disaster areas and evaluate damage.

c.	Develop and make recommendations to alleviate 					problems.

d.	Maintain contact with Galveston County 						Emergency Management.

e.	Repair emergency facilities and equipment, as 					necessary.

f.	Assess damage.

g.	Clear roads; make emergency repairs to water and 					sewer systems.

h.	Call out private contractors and other assistant as 					necessary.

i.	Assist in search and rescue, is possible.

j.	Construct levees and dike, as necessary.

4.	Recovery

a.	Repair public works and buildings.

b.	Support decontamination work, as necessary.

c.	Coordinate private and volunteer repair Utilities.

d.	Participate in preparing after-action report and 					critiques.  Make necessary changes and 						improvements in emergency management plan.

e.	Make recommendations to legislative body about 					changes in planning, zoning, and building code 					ordinances to mitigate impact of future disasters.

IV.	Organization and Assignment of Responsibilities

A.	During emergencies public works activities will be managed by 		the District Managers through the existing personnel.

B.	Task Assignments

1.	District Managers

a.	Coordinate emergency activities.

b.	Train personnel in emergency procedures.

c.	Develop and initiate mutual aid agreements.

d.	Identify local contractors and industry contacts that 				provide backup support.

e.	Develop resource lists.

f.	Repair and restoration of essential services and 					vital facilities.

g.	Participate in development and execution of 						emergency preparedness exercises.

2.	Field Supervisor (Manager)

a.	Maintain reasonable stock piles of emergency 					system repair materials.

b.	Secure emergency repairs of essential water and 					sewer systems when safe to do so.

c.	Debris removal from public rights-of-way, storm 					drains, and in or around those structures where 					public safety and/or health are endangered.

d.	Assist in decontamination.

e.	Place barricades when needed for public safety.

f.	Provide equipment as needed.

g.	Maintain contact with Galveston County 						Emergency Management.

h.	Support other groups with available resources.

V.	Direction and Control

A.	The usual supervisors will exercise operational control of 			personnel, with the District Managers maintaining overall 			management of personnel and equipment.  The District				Managers will set priorities for resources and coordinate 				activities with the Galveston County Emergency Management.

B.	The District Managers will coordinate the call up and 				development of mutual aid and/or volunteer forces needed to 			accomplish the District’s functions.  Mutual aid forces will 			operate under their own supervision while volunteer forces will 			work under the supervision of the senior District official on site.

VI.	Increased Readiness Actions

A.	Condition 4 – Beginning of Disaster Vulnerability Season

1.	Review personnel assignments.

2.	Conduct test and check readiness of equipment.

3.	Check readiness of facilities.

4.	Inspect and/or procure as needed equipment and supplies.

5.	Conduct tests and training as necessary.

B.	Condition 3 – Situation Exists That Could Develop Into A 					  Hazardous Condition

1.	Alert key personnel.

2.	Correct all deficiencies in equipment, supplies and/or 				facilities; heavy equipment fuel, sand bags, generators, 				chainsaws, etc.

3.	Check securing of all facilities.

C.	Condition 2 – Definite Characteristics of Developing Into a 					  Hazardous Condition

1.	Alert personnel of possible emergency duty.

2.	Place off-duty personnel on standby.

3.	Ensure safety of District personnel.

D.	Condition 1 – Hazardous Conditions Imminent

1.	Mobilize personnel on emergency basis.

2.	Deploy equipment if necessary.

3. 	Be prepared to handle requests.

4.	Maintain 24 hour operation.

5.	Evacuate if necessary.

6. 	Issue identification documents for District personnel as 				needed.

VII.	Continuity of Government

A.	District

The District Managers will be succeeded by the Field 				Superintendent.  In cases when a third person is necessary, the 			Sewer Plant Operator will fill the position.

B.	Essential Records

Will be backed up at a safe location.

VIII.	Administration and Support

A.	Administration

1.	The timely and efficient response of District forces will 				require extraordinary coordination between field forces 				and the District Managers.  Priorities assigned by the 				District Managers will facilitate an orderly use of District 			forces.

2.	The District Managers will develop procedures for the 				emergency hiring of private contractors and individuals 				to assist in response and recovery, with the approval of 				the Board of Directors of the District.

B.	Supply and Support

1.	During periods of increased readiness, supplies are to be 				stock piled to the extent independent operations could be 				sustained for at least four days.  A pre-planned list will 				be prepared with stock-level requirements.

2.	Emergency requests are to be coordinated with the 					District Managers.  Subject to approval of the Board of 				Directors, the District Managers is authorized to purchase 			equipment, supplies, and personnel services to support 				response and recovery efforts.  Adequate records of all 				purchases will be maintained.

C. 	Transportation and Equipment

1.	During periods of increased readiness, full fuel loads, 				extra fuel tanks and appropriate check-outs of essential 				transportation and heavy equipment will be 						accomplished.

2.	Any request for additional heavy equipment will be 				relayed to the District Managers where outside support 				will be pursued.

D.	Records

All records generated in am emergency will be collected and 			filed in an orderly manner so a chronology of events can be 			reviewed for future planning, settlement of claims, and lessons 			learned.