Key Board Functions – Grounds Coordinator

- Conduct weekly walk-through of AGHA grounds

- Report monthly grounds activities

- Conduct bid processes for contractors as needed

- Oversee Grounds and Irrigation Contractors

- Maintain schedule of contactor tasks needed (example below)

- Get Board approval of minor work needed outside of contracts

- Identify major tasks, get bids, seek Board approval (tree removal, fence repair, etc)

- Do minor grounds and irrigation tasks (bulb replacement, shrub pruning, etc)

- Determine irrigation zone times and days, locations, set clocks

| Plant | Location | Instructions |
| --- | --- | --- |
| March |  |  |
| Ornamental Grasses | Park, tennis courts, Parker Rd | Cut down to 8”, avoid/expose drip hoses |
| Muhly and other grasses | Parker Road sidewalk cobble | Cut down to 8” |
| Bluestem Joint Fir | park entrance | Trim top and sides |
| Broom Genista | park entrance | Trim top and sides |
| Blue Mist Spirea | Park, tennis courts, Parker Rd | Cut down to 1’ |
| Russian Sage | Park, tennis courts, Parker Rd | Cut down to 1’ |
| Rabbitbrush | Park, Parker Rd | Cut down to 3’ |
| Apache plume | Park, tennis courts, Parker Rd | Cut down to 4’ |
| Butterfly bushes (not Silver Fountain) | Park, Parker Rd | Cut down to 1’ or old cuts |
| Genista low shrubs | Parker Road | Trim to old cuts (6”) |
| 3-leaf sumac (grouping of 7) | Parker Road | Cut down to 4’ |
| Barberry shrubs | tennis courts, Parker Road | Prune/shape |
| Roses | Park, tennis courts, Parker Rd | Cut down to 1’ |
| Nearly Wild roses | Next to Norm’s lot | Do not trim |
| April |  |  |
| Turf, especially along creek | Park, Park Road north of creek | Apply pre-emergent and fertilizer |
| Native grass | North of creek, especially along creek | Apply pre-emergent |
| Turf | Park, Park Road north of creek | Aerate |
| Entrance median and inside brick areas | Entrance median and both sides | LIGHT rake-off of needles (leave mulch) |
| May |  |  |
| Turf | Park, Parker Road north of creek | Mow every week or every other week to 4” |
| August |  |  |
| Turf areas, especially along creek | Park, Parker Road north of creek | Apply weed control with crabgrass killer |
| Native grass | North of creek, especially along creek | Apply weed control with crabgrass killer |
| Native grass | North of creek | Mow to 1’ with trimmer |
| Native grass | North edge of creek to water line | Mow to 1’ with trimmer, as close to creek as possible, do not cut reeds |
| Native grass | South edge of creek to water line | Mow to 1’ with trimmer, as close to creek as possible, do not cut reeds |
| September |  |  |
| Turf | Park, Parker Road north of creek | Aerate |
| Turf | Park, Parker Road north of creek | Apply fertilizer |
| Silver Fountain butterfly bushes | Park, Parker Rd | Trim top to 5’, round sides |
| Junipers | Park, tennis courts, Parker Rd | Prune (rounded, not sharp) |
| Junipers | start of creek, Parker Road | Even-up tops |
| Junipers | overhanging spillway, sidewalk to tennis courts | Severely prune sides to edge of sidewalk and railing |
| Junipers and other shrubs | Around tennis courts | Prune tops and sides, inner at least 28” from fence |
| All shrubs next gazebo | tennis court gazebo | Trim to railing height, away from balusters |
| Russian olive sprouts | North of creek | Cut off all sprouts below 7’ on trees north of creek |
| October |  |  |
| Honeysuckle (3 groupings) | Along fence on Parker Road | Prune to 5' at SLOPE to ground |