Brewer Cote of Arizona – Acrylic Cote

Safety Data Sheet according to the Hazard Communication Standard (CFR29 1910.1200

Date of issue: 07/29/2016

Version: 1.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product form

Product name & code

Mixture

Acrylic Cote

Product Code: 420101A – Bulk

1.2. Relevant identified uses of the substance or mixture and uses advised against

Sealcoat for asphalt pavements. Asphalt Based

Use of the substance/mixture

:

1.3. Details of the supplier of safety data sheet

Brewer Cote of Arizona

5226 W Missouri Ave

Glendale AZ 85301

(888)931-3728

T 623-931-3727 Tech Service: Monday - Friday; 7:00am - 5:00pm AZ

1.4. Emergency telephone number

Emergency number: CHEMTREC (800)424-9300

SECTION 2: Hazards identification

2.1. 	Classification of the substance or mixture

GHS-US classification

Serious Eye Irritation 2A

Carcinogenicity 1A

Specific Target Organ Toxicity After Repeated Exposure 1

2.2. 	Label elements

GHS08

GHS-US labeling

Hazard pictograms (GHS-US)

Signal word (GHS-US) 
Hazard statements (GHS-US)

Precautionary statements (GHS-US)

GHS07 
Danger

Cause serious eye irritation.  May cause cancer. Causes damage to lungs through prolonged or repeated exposure

Obtain special instructions before use. Do not handle until all safety precautions have been 
read and understood. Do not breathe dust fume/gas/mist vapors/spray. Wash hands 
thoroughly after handling. Do not eat, drink or smoke when using this product. Contaminated 
work clothing should not be allowed out of the workplace. Wear protective gloves/protective 
clothing/eye protection face protection. If swallowed: Immediately call a POISON CENTER or 
doctor/physician. Rinse mouth. If on skin: Wash with plenty of soap and water. Take off 
contaminated clothing and wash it before reuse. If skin irritation or rash occurs: Get medical 
advice/attention. If in eyes: Rinse cautiously with water for several minutes. Remove contact 
lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical 
advice/attention. If exposed or concerned: Get medical advice/attention. Store locked up. 
Dispose of contents and container in accordance with all local, regional, national and  international regulations.

2.3. 	Other hazards

No additional information available

2.4. 	Unknown acute toxicity (GHS-US)

Acrylic Cote of the mixture consists of ingredient(s) of unknown acute toxicity.

| Component | CAS-No. | Weight |
| --- | --- | --- |
| Asphalt | 08052-42-4 | <40.00 |
| Polymer | NA | <20.00 |
| Additives Emulsifiers | NA | <2.00 |

If breathing is difficult, remove to fresh air and keep at rest in a position comfortable for 
breathing. Get medical advice/attention if you feel unwell.

In case of contact, immediately flush skin with plenty of water. Remove contaminated clothing 
and shoes. Wash clothing before reuse. Call a physician if irritation develops and persists.

In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. If easy to 
do, remove contact lenses, if worn. If irritation persists, get medical attention.

If swallowed, do NOT induce vomiting unless directed to do so by medical personnel. Never give 
anything by mouth to an unconscious person. Get medical advice/attention if you feel unwell.

4.2. 	Most important symptoms and effects, both acute and delayed

First-aid measures after skin contact

| Water | NA | <40.00 |
| --- | --- | --- |

SECTION 4: First aid measures 	.

4.1. 	Description. of first aid measures

First-aid measures after inhalation

First-aid measures after eye contact

First-aid measures after ingestion

Symptoms/injuries after inhalation 
Symptoms/injuries after skin contact

May cause respiratory irritation.

May cause skin irritation. Symptoms may include redness, drying, defatting and cracking of the 
skin. May cause sensitization by skin contact.

Causes serious eye irritation. Symptoms may include discomfort or pain, excess blinking and 
tear production, with marked redness and swelling of the conjunctiva.

Harmful if swallowed. May cause stomach distress, nausea or vomiting.

Symptoms/injuries after eye contact

Symptoms/injuries after ingestion

4.3. 	Indication of any immediate medical attention and special treatment needed

Symptoms may not appear immediately. In case of accident or if you feel unwell, seek medical advice immediately (show the label or SDS where 
possible).

SECTION 5: Firefighting measures

Brewer Cote of Arizona – Acrylic Cote 
Safety Data Sheet according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012.

SECTION 3: Composition/information on ingredients

3.1. 	Substances

Not applicable

3.2. 	Mixture

5.1. 	Extinguishing media.

Suitable extinguishing media 
Unsuitable extinguishing media

5.2. 	Special hazards arisinq from the substance or mixture

Fire hazard                                                                             : Water Stream may case violent eruption and spreading of asphalt.

5.3. 	Advice for firefighters

Protection during firefighting

Keep upwind of fire. Wear full fire fighting turn-out gear (full Bunker gear) and respiratory 
protection (SCBA).

Dry Chemical Carbon Dioxide or Foam 
None known.

Brewer Cote of Arizona – Acrylic Cote
Safety Data Sheet according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012.

SECTION 6: Accidental release measures

Use personal protection recommended in Section 8. Isolate the hazard area and deny entry to 
unnecessary and unprotected personnel.

6.1. 	Personal. precautions, protective equipment and emergency procedures

6.1.1. 	For non-emergency personnel

Protective equipment

6.1.2. 	For emergency responders

No additional information available

6.2. 	Methods and material for containment and cleaning up

For containment

Contain and/or absorb spill with inert material (e.g. sand, vermiculite), then place in a suitable 
container. Do not flush to sewer or allow to enter waterways. Use appropriate Personal 
Protective Equipment (PPE).

Methods for cleaning up 		Clean up small spills using appropriate techniques such as sorbent materials where feasible, remove contaminated soil and follow prescribed procedures

6.3. 	Reference to other sections

No additional information available

SECTION 7: Handling and storage

7.1. 	Precautions for safe handling

Precautions for safe handling

Avoid contact with skin and eyes. Do not swallow. Handle and open container with care. When 
using do not eat, drink or smoke. Obtain special instructions before use. Do not handle until all 
safety precautions have been read and understood. Use personal protective equipment as 
required. Do not breathe dust fume/gas/mist vapors/spray. Do not eat, drink or smoke when 
using this product. Wash hands thoroughly after handling.

7.2. 	Conditions for safe storage, including any incompatibilities

Storage conditions 	Keep out of the reach of children. Keep container tightly closed .

. 7.3. 	Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. 	Control parameters

May Occur					Conditions to Avoid  		None

Will not occur 		Yes

Hazardous

Polymerization

INCOMPATABILIRY (Materials to avoid)

MAY REACT WITH STRONG OXIDIZING MATERIALS

| Stability | Unstable |  |
| --- | --- | --- |
|  | Stable | Yes |

Brewer Cote of Arizona – Acrylic Cote
Safety Data Sheet

according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012.

9.2. 	Other information

Hydrogen sulfide and other hazardous vapors can collect in the head space of storage tanks or other enclosed vessels

8.2. 	Exposure controls

Appropriate engineering controls

Use ventilation adequate to keep exposures (airborne levels of dust, fume, vapor etc.) below 
recommended exposure limits.

Avoid all unnecessary exposure.

Wear suitable gloves.

Personal protective equipment

Eye protection 			       Wear safety glasses

: ~-

SECTION 9: Physical and chemical properties

9.1: 	Information on basic physical and chemical properties

Physical State						Liquid

Color			Brown

Odor							Asphalt Odor

Odor threshold						No data available
pH 							No data available

Relative evaporation rate (butyl acetate=1) 			7- 8.8
Melting point 						No data available

Freezing point 						32 degrees F

Boiling point 						212 degrees F

Flash point 						No data available

Self ignition temperature 					No data available
Decomposition temperature					No data available
Flammability (solid, gas)					No data available 
Vapor pressure 						60 (mm Hg)

Relative vapor density at 20 ·C 				No data available
Relative density 						No data available

Solubility 						In Water

Log Pow 						In Water

Log Kow 						No data available

Viscosity, kinematic 					No data available

Skin and body protection                                                	Wear suitable protective clothing.

Respiratory protection	In case of insufficient ventilation, wear a suitable respiratory equipment. 
Maintain levels below Community environmental protection thresholds.

Other Information

Do not eat, smoke or drink where material is handled, processed or stored. Wash hands carefully 
before eating or smoking. Handle according to established industrial hygiene and safety practices.

Water stream may cause violent eruption and spreading of asphalt

SECTION 10: Stability and reactivity

Brewer Cote of Arizona – Acrylic Cote
Safety Data Sheet  according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012.

10.1. 	Reactivity

No dangerous reaction known under conditions of normal use.

10.2. 	Chemical stable

Stable under normal conditions.

10.3. 	Possibility of hazardous reactions

No dangerous reaction known under conditions of normal use.

10.4: 	Conditions to avoid

Incompatible materials.  May react with strong oxidizing material

10.5. 	Incompatible materials

None known.

10.6. 	Hazardous decomposition products

None, normal combustion forms carbon dioxide and water vabor

Acute toxicity

Skin corrosion/irritation

Serious eye damage/irritation 
Respiratory or skin sensitization 
Germ cell mutagenicity 
Carcinogenicity

Based on available data, the classification criteria are not met 
Causes serious eye irritation.

May cause an allergic skin reaction. 
May cause genetic defects.

May cause cancer.

SECTION 12  Toxicity

12.1. 	Toxicity

Ecology - general

12.2. 	Persistence and degradability

Persistence and degradability

12.3. 	Bioaccumulative potential

Bioaccumulative potential

12.4. 	Mobility in solid

No additional information available

12.5. 	Other adverse effects

No additional information available

Brewer Cote of Arizona – Acrylic Cote

Safety Data Sheet according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012

SECTION 13: Disposal considerations

13.1.· Waste treatment methods 
Place contaminated materials in disposable containers and dispose of in a manor consistent with applicable regulations.

Contact local environmental or health regulators for approved disposal

SECTION 14: Transport information

In accordance with DOT

14.1. 	UN number Not Applicable

14.3. Additional information  Other information Special transport precautions Not applicable

14.2. 	. UN proper shippi.ng name Not applicable

SECTION 15: Regulatory information

May cause long-term adverse effects in the aquatic environment

No supplementary information available.

Do not handle until all safety precautions have been read and understood.

15.1. US Federal regulations

Section 16 – Other Information

Indication of changes			None

Other Information			None

Prepared by Brewer Cote of Arizona 
Printed Date: 07/29/2016

Disclaimer- Although careful consideration was exercised in the preparation of this document, some information was 
obtained from external sources which we believe is reliable. However; the information provided is without warranty, 
expressed or implied, regarding its accuracy or completeness.