Brewer Cote of Arizona – Coal Tar Emulsions

Safety Data Sheet according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012

Date of issue: 09/08/2015

Version: 1.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product form

Product name & code

Mixture

Coal Tar Emulsion

Product Code: 410001A – Bulk

:410005A – 5 gal pail

:410055A – 55 gal drum

1.2. Relevant identified uses of the substance 'or mixture and uses advised against

Sealcoat for asphalt pavements. Asphalt Based

Use of the substance/mixture

1.3. Details of the supplier of safety data sheet

(888)931-3728

Brewer Cote of Arizona

5226 W Missouri Ave

Glendale AZ 85301

T 623-931-3727 Tech Service: Monday - Friday; 6:00am - 5:00pm AZ

1.4. Emergency telephone number

Emergency number: Chemtrec  - 800-424-9300

SECTION 2: Hazards identification

2.1. 	Classification of the substance or mixture

GHS-US classification

Acute toxicity 4 (Oral)

Serious Eye Irritaiton 2A

Skin Sensitisation. 1

Germ Cell Mutagenicity 1 B 
Carcinogenicity 1A 
Reproductive Toxicity 1 B

Specific Target Organ Toxicity After Repeated Exposure 1

2.2. 	Label elements

GHS08

GHS-US labeling

Hazard pictograms (GHS-US)

Signal word (GHS-US) 
Hazard statements (GHS-US)

Precautionary statements (GHS-US)

GHS07 
Danger

Harmful if swallowed. May cause an allergic skin reaction. Causes serious eye irritation. May 
cause genetic defects. May cause cancer. May damage fertility or the unborn child. Causes 
damage to lungs through prolonged or repeated exposure.

Obtain special instructions before use. Do not handle until all safety precautions have been 
read and understood. Do not breathe dustlfume/gas/mistlvapours/spray. Wash hands 
thoroughly after handling. Do not eat, drink or smoke when using this product. Contaminated 
work clothing should not be allowed out of the workplace. Wear protective gloves/protective 
clothing/eyeprotectionlface protection. If swallowed: Immediately call a POISON CENTER or 
doctor/physician. Rinse mouth. If on skin: Wash with plenty of soap and water. Take off 
contaminated clothing and wash it before reuse. If skin irritation or rash occurs: Get medical 
advice/attention. If in eyes: Rinse cautiously with water for several minutes. Remove contact 
lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical 
advice/attention. If exposed or concerned: Get medical advice/attention. Store locked up. 
Dispose of contents and container in accordance with all local, regional, national and  international regulations.

2.3. 	Other hazards

No additional information available

2.4. 	Unknown acute toxlcity (GHS-US)

Coal Tar Emulsion of the mixture consists of ingredient(s) of unknown acute toxicity.

Brewer Cote of Arizona – Coal Tar Emulsions 
Safety Data Sheet according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012.

SECTION 3: Composition/information on ingredients

3.1. 	Substances

Not applicable

3.2. 	Mixture

| Name | Product identifier |  |  |
| --- | --- | --- | --- |
| Pitch, coal tar, high-temperature | (CAS No) 65996-93-2 | 10 - 30 |  |
| Kaolin | (CAS No) 1332-58-7 | 10 - 30 |  |
| Bentonite | (CAS No) 1302-78-9 | <10 |  |
| The exact percentage (concentration) of composition has been withheld as a trade secret in accordance with paragraph (i) of §1910.1200. |  |  |  |

SECTION 4: First aid measures 	.

4.1. 	Description. of first aid measures

First-aid measures after inhalation

If breathing is difficult, remove to fresh air and keep at rest in a position comfortable for 
breathing. Get medical advice/attention if you feel unwell.

In case of contact, immediately flush skin with plenty of water. Remove contaminated clothing 
and shoes. Wash clothing before reuse. Call a physician if irritation develops and persists.

In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. If easy to 
do, remove contact lenses, if worn. If irritation persists, get medical attention.

If swallowed, do NOT induce vomiting unless directed to do so by medical personnel. Never give 
anything by mouth to an unconscious person. Get medical advice/attention if you feel unwell.

4.2. 	Most important symptoms and effects, both acute and delayed

First-aid measures after skin contact

First-aid measures after eye contact

First-aid measures after ingestion

Symptoms/injuries after inhalation 
Symptoms/injuries after skin contact

May cause respiratory irritation.

May cause skin irritation. Symptoms may include redness, drying, defatting and cracking of the 
skin. May cause sensitisation by skin contact.

Causes serious eye irritation. Symptoms may include discomfort or pain, excess blinking and 
tear production, with marked redness and swelling of the conjunctiva.

Harmful if swallowed. May cause stomach distress, nausea or vomiting.

Symptoms/injuries after eye contact

Symptoms/injuries after ingestion

4.3. 	Indication of any immediate medical attention and special treatment needed

Symptoms may not appear immediately. In case of accident or if you feel unwell, seek medical advice immediately (show the label or SDS where 
possible).

SECTION 5: Firefighting measures

5.1. 	Extinguishing media.

Suitable extinguishing media 
Unsuitable extinguishing media

Treat for surrounding material. 
None known.

5.2. 	Special hazards arisinq from the substance or mixture

Fire hazard 	: Products of combustion may include, and are not limited to: oxides of carbon.

5.3. 	Advice for firefighters

Protection during firefighting

Keep upwind of fire. Wear full fire fighting turn-out gear (full Bunker gear) and respiratory 
protection (SCBA).

Use personal protection recommended in Section 8. Isolate the hazard area and deny entry to 
unnecessary and unprotected personnel.

SECTION 6: Accidental release measures

6.1. 	Personal. precautions, protective equipment and emergency procedures

6.1.1. 	For non-emergency personnel

Protective equipment

6.1.2. 	For emergency responders

No additional information available

6.2. 	Methods and material for containment and cleaning up

For containment

Contain and/or absorb spill with inert material (e.g. sand, vermiculite), then place in a suitable 
container. Do not flush to sewer or allow to enter waterways. Use appropriate Personal 
Protective Equipment (PPE).

Methods for cleaning up 		Scoop up material and place in a disposal container.

6.3. 	Reference to other sections

No additional information available

SECTION 7: Handling and storage

7.1. 	Precautions for safe handling

Precautions for safe handling

Avoid contact with skin and eyes. Do not swallow. Handle and open container with care. When 
using do not eat, drink or smoke. Obtain special instructions before use. Do not handle until all 
safety precautions have been read and understood. Use personal protective equipment as 
required. Do not breathe dustlfume/gas/mistlvapours/spray. Do not eat, drink or smoke when 
using this product. Wash hands thoroughly after handling.

7.2. 	Conditions for safe storage, including any incompatibilities

Storage conditions 	Keep out of the reach of children. Keep container tightly closed .

. 7.3. 	Specific end use(s)

No additional information available

| Pitch, coal tar, high-temperature (65996-93-2) |  |  |
| --- | --- | --- |
| USA ACGIH | ACGIH TWA (mg/m') | 0.2 mg/m' |
| USA OSHA | OSHA PEL (TWA) (mg/m') | 0.2 mg/m3 (benzene soluble fraction) |

SECTION 8: Exposure controls/personal protection

8.1. 	Control parameters

Quartz (14808-60-7)

USA ACGIH

ACGIH TWA (rnq/rn")

0.025 mg/m3

| Titanium dioxide (13463-67-7) |  |  |
| --- | --- | --- |
| USA ACGIH | I ACGIH TWA (mg/m') | 10 mg/m' |
| USA OSHA | I OSHA PEL (TWA) (mg/m') | 15 mg/m' |

| Naphthalene (91-20-3) |  |  |
| --- | --- | --- |
| USA ACGIH | ACGIH TWA (ppm) | 10 ppm |
| USA ACGIH | ACGIH STEL (ppm) | 15 ppm |
| USA OSHA | OSHA PEL (TWA) (rng/m") | 50 mg/m3 |
| USA OSHA | OSHA PEL (TWA) (ppm) | 10 ppm |

Chrysene (218-01-9)

USA OSHA

OSHA PEL (TWA) (mg/m')

0.2 mg/m3

Benzoapyrene (50-32.8)

USA OSHA

OSHA PEL (TWA) (mg/m3)

0.2 mq/m"

Brewer Cote of Arizona – Coal Tar Emulsions
Safety Data Sheet

according to the Hazard Communication Standard (CFR29 1910.1200) HazCom 2012.

9.2. 	Other information

VOC content 	< 50 g/L

Viscosity, dynamic Explosive properties 
Oxidizing properties 
Explosive limits

8.2. 	Exposure controls

Appropriate engineering controls

Use ventilation adequate to keep exposures (airborne levels of dlJlst, fume, vapOf"etc.) below 
recommended exposure limits.

Avoid all unnecessary exposure.

Wear suitable gloves.

Personal protective equipment

Eye protection 			       Wear safety glasses

: ~-

SECTION 9: Physical and chemical properties

9.1: 	Information on basic physical and chemical properties

Physical State						Liquid

Color			No data available.

Odor							No data available

Odor threshold						No data available
pH 							No data available

Relative evaporation rate (butylacetate=1) 			No data available
Melting point 						No data available

Freezing point 						No data available

Boiling point 						100 degrees C

Flash point 						No data available

Self ignition temperature 					No data available
Decomposition temperature					No data available
Flammability (solid, gas)					No data available 
Vapor pressure 						No data available

Relative vapor density at 20 ·C 				No data available
Relative density 						No data available

Solubility 						No data available

Log Pow 						No data available

Log Kow 						No data available

Viscosity, kinematic 					No data available

Skin and body protection                                                	Wear suitable protective clothing.

Respiratory protection	In case of insufficient ventilation, wear  suitable respiratory equipment. 
Maintain levels below Community environmental protection thresholds.

Other Information

Do not eat, smoke or drink where material is handled, processed or stored. Wash hands carefully 
before eating or smoking. Handle according to established industrial hygiene and safety practices.

SECTION 10: Stability and reactivity

10.1. 	Reactivity

No dangerous reaction known under conditions of normal use.

10.2. 	Chemical stable

Stable under normal conditions.

10.3. 	Possibility of hazardous reactions

No dangerous reaction known under conditions of normal use.

10.4: 	Conditions to avoid

Incompatible materials.

10.5. 	incompatible materials

None known.

10.6. 	Hazardous decomposition products

May include, and are not limited to: oxides of carbon.

Acute toxicity

Quartz (14808-60-7)

LD50 oral rat

9H-Carbazole (86-74-8)

> 5000 mg/kg

LD50 oral rat

Skin corrosion/irritation

Serious eye damage/irritation 
Respiratory or skin sensitization 
Germ cell mutagenicity 
Carcinogenicity

Based on available data, the classification criteria are not met 
Causes serious eye irritation.

May cause an allergic skin reaction. 
May cause genetic defects.

May cause cancer.

Pitch, coal tar, high-temperature (65996-93-2)

IARC group

Quartz (14808-60-7)

IARC group

Titanium dioxide (13463-67-7)

IARC group

2B

Naphthalene (91-20-3)

IARC group

Benzanthracene (56-55-3)

IARC group

2B

Indeno( 1,2,3-cd )py rene (193-39-5)

IARCgroup

2B

Reproductive toxicity

Specific target organ toxicity (single exposure)

Specific target organ toxicity (repeated 
exposure)

Aspiration hazard

Symptoms/injuries after inhalation 
Symptoms/injuries after skin contact

May damage fertility or the unborn child.

Based on available data, the classification criteria are not met.

Causes damage to lungs through prolonged or repeated exposure.

Based on available data the classification criteria are not met

May cause respiratory irritation.

May cause skin irritation. Symptoms may include redness, drying, defeating and cracking of the 
skin. May cause sensitization by skin contact.

| Titanium dioxide (13463-67-7) |  |
| --- | --- |
| LD50 oral rat | I > 10000 mg/kg |
| Naphthalene (91-20-3) |  |
| LD50 dermal rabbit | I > 20 g/kg |
| LC50 inhalation rat (mg/I) | I > 340 mg/m3 (1 h) |

Causes serious eye irriation.           Symptioms may include discomfort or pain, excess

Blinking and tear productions, with marked redness

And swelling of the conjunctiva

. Brewer Cote of Arizona – Coal Tar Emulsions
Safety Data Sheet

Brewer Cote of Arizona – Coal Tar Emulsions

according to the Hazard Communication

Standard (CFR29 1910.1200) HazCom 2012

Symptoms/ injuries after ingestion   Harmful if swallowed.  May cause stomach distress,

Nausea or vomiting

IChrysene (218-01-9)

IARC group

Benzo(a)pyrene (50-32-8)

Benzo(b)fluoranthene (205-99-2)

2B

IARC group

IARC group

2B

19H-carbazoe (86-74-8)

12B

Dibenzo(a,h)pyrene (189-64-0)

IARC group

IARC group

Benzo(j)fluoranthene (205-82-3)

2B

Benzo(k)fluoranthene (207-08.9)

IARC group

2B

IARC group

SECTION 12: Ecolo ical information

12.1. 	Toxicity

Ecology - general

12.2. 	Persistence and degradability

Persistence and degradability

12.3. 	Bioaccumulative potential

Bioaccumulative potential

12.4. 	Mobility in solid

No additional information available

12.5. 	Other adverse effects

No additional information available

SECTION 13: Disposal considerations

13.1.· Waste treatment methods 
No additional information available

SECTION 14: Transport information

In accordance with DOT

14.1. 	UN number Not Applicable

14.3. Additional information  Other information Special transport precautions Not applicable

14.2. 	. UN proper shippi.ng name Not applicable

SECTION 15: Regulatory information

May cause long-term adverse effects in the aquatic environment

No supplementary information available.

Do not handle until all safety precautions have been read and understood.

15.1. US Federal regulations

Pitch, coal tar, high-temperature (65996-93-2)

Listed on the United States TSCA (Toxic Substances Control Act) inventory

. Quartz (14808-60-7)

Listed on the United States TSCA (Toxic Substances Control Act) inventory

Aluminum silicate (1327-36-2)

Listed on the United States TSCA (Toxic Substances Control Act) inventory

Feldspar (68476-25-5)

Listed on the United States TSCA (Toxic Substances Control Act) inventory

| Titanium dioxide (13463-67-7) | I |  |
| --- | --- | --- |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |  |
| Naphthalene (91-20-3) |  | ! |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |  |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| EPA TSCA Regulatory Flag | I T - T - indicates a substance that is the subject of a Section 4 test rule under TSCA. |  |
| SARA Section 313 - Emission Reporting | I 0.1 |  |

| Indeno(1,2,3-cd)pyrene (193-39-5) |  |  |
| --- | --- | --- |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |  |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| SARA Section 313 - Emission Reporting | 1 0.1 |  |
| Chrysene (218-01-9) |  |  |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |  |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| SARA Section 313 - Emission Reporting | 11.0 |  |
| Benzo(b)fluoranthene (205-99-2) |  |  |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| SARA Section 313 - Emission Reporting | 10.1 |  |
| Benzo(a)pyrene (50-32-8) | ' .. |  |
| Listed on the United States TSCA (Toxic SUbstances Control Act) inventory |  |  |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| SARA Section 313 - Emission Reporting | I 0.1 |  |
| 9H-Carbazole (86-74-8) |  |  |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |  |
| Dibenzo(a,h)pyrene (189-64-0) |  | I |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| SARA Section 313 - Emission Reporting | 10.1 |  |
| BenzoO)fluoranthene (205-82-3) |  | J |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| SARA Section 313 - Emission Reporting | 10.1 |  |

| Benz[a]anthracene (56-55-3) |  | I |
| --- | --- | --- |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |  |
| Listed on SARA Section 313 (Specific toxic chemical listings) |  |  |
| SARA Section 313 - Emission Reporting | I 0.1 |  |

SECTION 16: Other information

Indication of changes 
Other information

| Brewer Cote of Arizona – Coal Tar Emulisions |  |
| --- | --- |
| State or local regulations | This product contains Crystalline Silica, Quartz and may also contain trace amounts of |
|  | other chemicals known to the State of California to cause cancer, birth defects or other |
|  | reproductive harm. |

None. 
None