BREWER COTE® of Arizona

RESURFACER II +

Brewer Cote of Arizona

5226 W Missouri Ave

Glendale AZ 85301

Phone: 888-931-3728

Fax: 623-842-0714

www.brewercoteaz.com

DESCRIPTION:

Resurfacer II + is a milled blend of asphalt, selected mineral fillers and polymers.  It is designed specially to increase the life expectancy of your asphalt. Resurfacer II + beautifies and protects both old and new parking lots, recreational areas, and roadways against moisture and water penetration, ultraviolet rays from the sun, and oxidation. The tough, durable formula fills pavement voids and low spots by replacing fines that are lost during the aging process.

Advantages:

● Saves your pavement		● Better resistance to fuel & chemicals than competitive products

● Increase life expectancy		● Meets all state & public works Specifications

● Long lasting			● Can be squeegee applied to fill minor cracks & depressions

● Stops raveling			● Smooth flexible surface that is easy to clean

RESURFACER II +

Coverage:

Resurfacer II + should be applies mixed (Resurfacer II, water, sand, and additives)  at a rate of 50 - 70 square feet per gallon, per coat.  If necessary an additional 10% water may be added for smooth

Application.

**Two coats are recommended on unsealed pavements, consult project manager.

Method of Application:

Make sure area is clean and free from all loose material and dirt.  Squeegee or mechanical spray equipment product onto surface.  Product may be diluted with a maximum of 20% water.  An additional two pounds (50 – 70 mesh) maybe added for added filling properties along with additives for increased durability.  Do not over dilute Resurfacer II +.  Two coats are recommended.  Consult your project manager if needed.

Color:	Jet black finish

Weather Limitations:

For best results, do not apply on rainy days or when the temperature is expected to drop below 50 degrees F during the next 24 hours. On hot days, the pavement shall be pre-dampened with fog system.  Ambient and surface temperature must be at least 55 F and rising during application.  Do not apply if rain is in the forecast for the next 24 hours.  Always allow new asphalt to cure through a summer season before sealing.

Drying time: 	5 hours maximum at 70 degree's F, 50% relative humidity.  Allow sealer to dry thoroughly between coats.  Finial coat should cure for 24 hours.

Clean up: 		Water before dry, use biodegradable solvent when dry..

Packaging: 	5 gallon pail (ship weight 54 lbs), 55 gallon drums (ship weight 610 lbs), or bulk transport (10 lbs per gallon).

Specifications:

METHOD		MIN		MAX		RESULTS

Weight per gallon		ASTM D244		10.5 lbs		10.8lbs 		10.5 lbs

Polymer Modified% (Based on Asphalt Content)		3%				Pass

% Non-Volatile		ASMA A-1			50		60		55

% Non-Volatile		ASTM D2042			20		35		30

(Soluble in Trichloroethylene by wt)

Penetration-Base Asphalt	D5			10		45		35

Softening Point (Ring + Ball)  D36			195 deg			210 deg

Asphalt Content, Weight %	D2172			15		25		20%

Aggregate Content By Weight				33% 		40% 		35%

*Wet track abrasion, 		ASTM D3910				15.3		4.6

(Grams per sq/ft)

Percentage Loss						5%		1.5%

Water Resistance		D4585							No Loss

Resistance to Impact		D2939							Pass

Resistance to Heat		D2939							Pass

QUV UV Aging, 1000 HAS.	D4799							Pass

PAH %											< .10%

Volatile Organic Compound (VOC)							< 8 G/L

*Modified for Sand Seal, 8 pounds + 10 minutes

REV. 8/2019