Brewer Cote of Arizona  Hot Pour Loop Sealant  SDS

Section 1: Identification

In this section you will find the product name which will be listed on the label as well as any contact information on the manufacturer.

Product Identification and Product Code

Product Use Description

Manufacturer/Supplier

Emergency Phone Number

Hot Pour Loop Sealant

430501A 34# Box

Asphalt Joint Filler

Brewer Cote of Arizona

5226 W Missouri Ave

Glendale AZ 85301

888-931-3728

CHEMTREC – 800-424-9300

Section 2: Hazard(s) Identification

When looking at the different classifications of hazards this chemical meets you will see category numbers to tell you how severe the hazard is. 
Please remember, these numbers ARE NOT the same as the old NFPA/HMIS system. In this case, the numbering generally starts at 1 (most 
hazardous) and ends at 5 (least hazardous.) There are letters used in certain categories as well. Always remember, the lower the number or letter, 
the more severe the hazard.

O.S.H.A. Hazard Classifications:

Eyes Irritation- Category 2A 
Skin Irritation- Category 4

US OSHA Hazard Symbol:

Signal Word: Warning!

US DOT SYMBOLS

Non Regulated

Hazard Statements: May be harmful if swallowed. May cause burns if contact with skin. Will cause eye irritation.

Page 1 of 5

Section 3: Composition/Information on Ingredients

This section lists the ingredients of this product. For Exposure limits please refer to section 8 of this document.

| Component | CAS-No. | Weight |
| --- | --- | --- |
| Asphalt Cement | 08052-42-4 | <50.00 |
| Rubber | 2630-26-4 | <50.00 |
| Extenders | NA Mixture | <30.00 |
| Fillers | NA Mixture | <35.00 |

Section 4: First-Aid Measures

This area will tell you what to do if there is an exposure to this chemical. If there is an exposure situation that requires emergency care, remember to 
bring a copy of this document to help in aid.

Inhalation:

Skin Contact:

Eye Contact:

Ingestion:

Remove from contaminated area. If breathing is difficult, give oxygen. If breathing stops, give 
artificial respiration - Call Physician

Remove contaminated clothing. Cool with water, Remove with mineral, soap & water, get medical help for several burns

Immediately flush with water for at least 15 minutes holding eyelids open. Call a physician if irritation 
develops

If conscious give 1 to 2 glasses of milk or water, induce vomiting and call a physician. NEVER GIVE 
ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON.

Section 5: Fire Fighting Measures

This section lists how to extinguish a fire but more importantly any specific hazards that can be created should this product burn.

Suitable Extinguishing 
Equipment:

Specific Hazards:

Special Protective Equipment:

Other Information: N/A

Dry chemical,C02, or standard foam. 
Use self controlled breathing apparatus in enclosed areas & Full body protection gear including face shield, trench coat

Although this material is considered non-flammable, if it is in the fire area, an 
approved, self-contained breathing apparatus should be worn to prevent exposure 
to thermal decomposition products and residual monomers.

For fires in enclosed areas, wear self-contained breathing apparatus.

Page 2 of 5

Section 6: Accidental Release Measures

This section provides recommendations on the appropriate response to spills, leaks, or releases including containment and cleanup practices to 
prevent or minimize exposure to people, properties, or the environment.

Personal Precautions:

Environmental 
Precautions:

Methods for Clean Up:

Avoid prolonged inhalation. Open all windows and doors. Advise proper authorities. 
Wear protective clothing, safety glasses, and impervious gloves.

Prevent product from entering storm drains, sewers.

Allow to cool and solidify cover with earth to reduce adhesiveness, place in appropriate containers for transporting to a waste disposal site

Section 7: Handling and Storage

This section provides guidance on the safe handling practices and conditions for safe storage of chemicals.

Precautions for Safe 
Handling:

Conditions for Safe Storage:

Use only as directed in well ventilated area. Avoid prolonged contact with skin. Avoid 
contact with eyes. Avoid breathing mist or vapors. Do not eat, smoke, or drink with 
contaminated hands. Do not discard into sewers or drains.

Store away from ignition sources in tightly sealed containers at room temperature. Do 
NOT store in Direct Sunlight.

Section 8: Exposure Controls/Personal Protection

This section indicates the exposure limits, engineering controls, and personal protective measures that can be used to minimize worker exposure.

OSHA Permissible Exposure Limits: N/A

Control Parameters: N/A

Recommendations for Personal Protection: Gloves, Eye Protection

Other Special Instructions: N/A

Page 3 of 5

Section 9: Physical and Chemical Properties

This section identifies physical and chemical properties associated with the substance or mixture.

Appearance- Black 
Odor- Petroleum Order

pH- 3-4

Melting Point- N/A 
Freezing Point - 32*F 
Initial Boiling Point- 600*F 
Flash Point- N/A

Evaporation Rate- Equivalent to Water 
Flammability- Non Flammable 
Explosive Limits- N/A

Vapor Pressure- N/A

Vapor Density- N/A
Solubility(ies)- NIL 
Viscosity- 135+ KU's

Section 10: Stability and Reactivity

This section describes the reactivity hazards of the chemical and the chemical stability information. This section is broken into three parts: reactivity, 
chemical stability, and other.

Reactivity: 	Avoid use around oxidizing agents.

Chemical Stability: Stable

Other:

Section 11: Toxicological Information

This section identifies toxicological and health effects information or indicates that such data are not available.

Non-Toxic

Section 12: Ecological Information (non-mandatory)

This section provides information to evaluate the environmental impact of the chemical(s) if it were released to the environment.

Bury in an approved waste dump area or burn in approved incinerator follow

Page 4 of 5

Section 13: Disposal Considerations (non-mandatory)

This section provides guidance on proper disposal practices, recycling or reclamation of the chemical(s) or its container, and safe handling practices.

Although this product is believed to have no adverse effects on the environment, ALWAYS dispose of unused material in 
accordance with Federal, State and local laws.

Section 14: Transport Information (non-mandatory)

This section provides guidance on classification information for shipping and transporting of hazardous chemical(s) by road, air, rail or sea.

Class 55

Section 15: Regulatory Information (non-mandatory)

This section identifies the safety, health, and environmental regulations specific for the product that is not indicated anywhere else on the SDS.

Keep pallets intact until material is used to prevent container deterioration.

Section 16: Other Information

This section indicates when the SDS was prepared, when the last known revision was made and other useful information.

Prepared by Brewer Cote of Arizona 
Printed Date: 7/28/2016

Disclaimer- Although careful consideration was exercised in the preparation of this document, some information was 
obtained from external sources which we believe is reliable. However; the information provided is without warranty, 
expressed or implied, regarding its accuracy or completeness.

Page 5 of 5