2020+ Ford Explorer ST intercooler directions

This product is an aftermarket (off road) performance enhancer. Ultimate Performance has no control over the installation and/or use of the products we manufacturer or sell. The purchaser assumes ALL responsibility for the safety, proper installation, performance and applicability of these products. Ultimate Performance will not be held liable for personal, property, legal or financial damages or claims stemming from the use or misuse of any of our products. If you have any questions or concerns about the installation, function, performance or proper use of these products please call us before proceeding with the installation.

Typical install time is 2-4 hours

- 1. Before starting your install be sure to jack the vehicle up to a sufficient height that you can safely work under. Chock your rear wheels and use good quality jack stands to secure the vehicle at a workable height. NEVER work under your car unless it is properly secured in this manner.

- 2. Disconnect battery

- 3. Remove the lower splash shield

- 4. remove bumper bolts/plastic fasteners in both passenger and driver side wheel well

- 5. Peel back fender liner to gain access to bumper cover bolts remove 2 bolts on each side.

- 6.  Remove 4 bolts from lower core support They are large diameter and long

- 7. remove 4 remaining upper bumper cover bolts and gently remove bumper cover, set aside.

- 8. Remove 4 bumper bolts from both sides

- 9. Remove both headlights

- 10. Disconnect hood release cable inside plastic cover

- 11. Remove core support upper bolts

- 12. Pop out radiator clips by squeezing the sides of the release clip, push radiator back so it is not hooked in core support anymore.

- 13. Disconnect electrical harness connectors

14. Remove core support and set aside.

- 15. Modify grill shutters by removing 2 lower shutters and cut off part of plastic bracket as seen in picture:

- 16. Pull out harness plastic clips running parallel to grill shutters and move harness to passenger side a couple inches to help clearance for new intercooler:

- 17. Grind or file off sharp edge on AC condensor so it does not cut into new intercooler:

- 18. Install some weather strip on ac condensor to help new intercooler and condensor not chaffe

- 19. Remove plastic hose connector, remove map sensor out of side of core and install it into the new intercooler using stock hardware, bolt will thread itself into new intercooler. Remove hose clamps on stock IC from both sides and pull stock intercooler out and set aside.

- 20.  Remove vampire tangs off stock hose clamps and install supplied couplers onto stock piping.

21. Position mounting bracket on new intercooler straight up and tighten down bolt.

- 22. Install new intercooler by tilting and rocking it into place then hook up couplers and plug in plastic tube. Tighten hose clamps.

- 22. Reinstall core support, start from bottom and ensure plastic nipples on radiator stick into rubber grommets on core support, then rotate core support up and backward toward engine, plug in map sensor, once up into position gently line up radiator and push tabs in until they click and lock in. Reinstall upper core support bolts and all remaining core support bolts in the reverse order of removal.

- 23. Plug in all harnesses that were unplugged during removal. Reinstall everything in reverse order.

- 24. Enjoy the power gains from your new Ultimate Performance Garrett core intercooler!!!!