City of Marble Hill
Administrative Procedures

Bill No. 2025-04              Ordinance No. 2025-04

Floodplain Management Procedures:

All procedures and regulations are hereby established by 60.3(b) Floodplain Management Ordinance No. 09-25-25, passed and adopted on October 20, 2025, by the City of Marble Hill.

Definitions:

"Flood" or "Flooding" means a general and temporary condition of partial or complete inundation of normally dry land areas from (1) the overflow of inland and/or (2) the unusual and rapid accumulation or runoff of surface waters from any source.

"Floodplain" or "Flood-prone Area" means any land area susceptible to being inundated by water from any source.

"Special Hazard Area" means an area having special flood hazards and shown on an FHBM or FIRM as zones (unnumbered or numbered) A, AO, AE, or AH.

The floodplain is determined by Flood Insurance Rate Map number 29017CV000A (Effective Date: August 16, 2023) The maps are available to view at the Marble Hill City Hall.

The City of Marble Hill Administrator has been designated as the Floodplain Administrator whose duties are as follows:

Duties of Floodplain Administrator:

- Understand regulations of the National Flood Insurance Program (NFIP)

- Review Floodplain Development Permits/Applications

- Conduct inspection in the floodplain

- Initiate enforcement actions, when applicable

- Interacting in variance and appeal processes

- Record keeping

- Investigate complains of violations

- Maintain and update administrative forms

- Maintain floodplain maps and flood data

- Disseminate floodplain management information

Requirements for Development Permits in Floodplain Area

- Require Floodplain Development Permits for activities in the floodplain area including:

  - Water course alterations

  - New development (residential and non-residential)

  - Manufactured homes

  - Substantially damaged structures

  - Substantially improved structures

  - Subdivision

  - Filling, grading, channelizing and excavating in the floodplain

- Require the following information on Floodplain Development Permits

  - Technical documentation

  - Permits from other parties or agencies when applicable

  - Elevation Certificates

  - Floodproofing certificates if applicable

  - Plans or blueprints

  - Cost analysis for substantial damage or substantial improvement

  - Cost of new facilities

Information Required for Floodplain Permits

- A Floodplain Development Permit is necessary for any development in the floodplain

- An NFIP Elevation Certificate signed by a professional engineer, surveyor, or architect showing the elevation of the site and of the lowest floor, which must be one foot above the Base Flood Elevation (BFE) for new or substantially damaged or improved structures

- Wet floodproofing may be allowed for accessory structures used solely for parking, building access, or limited storage with proper entry and exit for flood water, and constructed with water resistant materials

- If dry floodproofing is used for non-residential structures, a certificate must be signed by a professional engineer

- Site information needed includes elevation data, property lines, location of streets and placement of structures

- Building plans should include type of structure, use placement of lowest floor, details of construction, new or add-on construction, type of materials, anchoring requirements, any other data pertinent to the construction

Permit Review Process

- Floodplain Development Permit applications are to be submitted to the Floodplain Administrator for review

- Floodplain Administrator will review the permit to ensure the required information is listed

- Floodplain Development Permits will be approved or denied and signed by the Floodplain Administrator

Inspection

- First inspection will be to confirm if the site information and data conform to requirements

- Second inspection will determine if the lowest floor elevation conforms with Base Flood Elevation requirements and construction is proceeding according to the submitted plans

- Third inspection will determine if construction compiled with the plans including, but not limited to anchoring, openings, materials, etc.

Enforcement Actions

Should a permit applicant desire to appeal the decision of the City’s Floodplain Administrator, they may plead to the Appeal Board of the City of Marble Hill. The Board of Appeals shall hear and decide appeals when it is alleged that there is an error in any requirements, decision or determination made by the Floodplain Administrator in the enforcement or administration of the ordinance. Any person aggrieved by the decision of the Board of Appeals my appeal such decision to the Circuit Court of Bollinger County, Missouri, as provided in Section 89.110, RSMo. When violations occur, the person will be informed verbally. If no action is taken to remedy the violation, the person will be informed in writing by certified mail of the specific violation and given a reasonable time frame to correct deficiencies. Possible penalties may include stop work orders, fines, revoking permits, or a combination of these activities.

Variances

Floodplain Management Variance Criteria: In passing upon such applications, the Board of Appeals shall consider all technical data and evaluations, all relevant factors, standards specified in the Floodplain Management Ordinance, and the following criteria:

- The danger that material may be swept onto other lands to the injury of others

- The danger to life and property due to flooding or erosion damage

- The susceptibility of a proposed facility and its contents to flood damage and the effect of such damage on the individual owner

- The importance of the services provided by the proposed facility to the community

- The necessity to the facility of a waterfront location, where applicable

- The availability of alternative locations, not subject to flooding or erosion damage, for the proposed use

- The compatibility of the proposed use with existing and anticipated development

- The relationship of the proposed use to floodplain management program for that area

- The safety of access to the property in times of flood or ordinary and emergency vehicles

- The expected heights, velocity, duration, rate of rise and sediment transport of the floodwaters and the effects of wave action, if applicable, expected at the site

- The cost of providing governmental services during and after flood conditions including maintenance and repair of public utilities and facilities such as sewer, gas, electrical, and water systems, and streets and bridges.

Conditions for Variances

1. 	Generally, variances may be issued for new construction and substantial-improvements to be erected on a lot of one-half (1/2) acre or less in size contiguous to and surrounded by lots with existing structures constructed below the base flood level, providing items 2 through 6 below have been fully considered.  As the lot size increases beyond the one-half acre, the technical justification required for issuing the variance increases.

2.	Variances may be issued for the reconstruction, rehabilitation, or restoration of structures listed on the National Register of Historic Places, the State Inventory of Historic Places, or local inventory of historic places upon determination provided the proposed activity will not preclude the structure’s continued historic designation.

3.	Variances shall not be issued within any designated floodway if any increase in flood levels during the base flood discharge would result.

4.	Variances shall only be issued upon a determination that the variance is the minimum necessary, considering the flood hazard, to afford relief.

5.	Variances shall only be issued upon (a) a showing of good and sufficient cause, (b) a determination that failure to grant the variance would result in exceptional hardship to the applicant, and (c) a determination that the granting of a variance will not result in increased flood heights, additional threats to public safety, extraordinary public expense, create nuisances, cause fraud on or victimization of the public, or conflict with existing local laws or ordinances.

6.	A community shall notify the applicant in writing over the signature of a community official that (1) the issuance of a variance to construct a structure below base flood level will result in increased premium rates for flood insurance up to amounts as high as $25.00 for $100.00 of insurance coverage and (2) such construction below the base flood level increases risks to life and property.  Such notification shall be maintained with the record of all variance actions as required by this ordinance.

Determinations for Variances

- Showing of good and sufficient cause;

- A determination that failure to grant the variance would result in exceptional hardship to the applicant

- A determination that the granting of a variance will not result in increased flood heights, additional threats to public safety, extraordinary public expense, create nuisances, cause fraud on or victimization of the public, or conflict with existing local laws or ordinances.

Conditions for Approving Accessory Structure Variances

Any variance granted for an accessory structure shall be decided individually based on a case by case analysis of the building's unique circumstances.  Variances granted shall meet the following conditions as well as those criteria and conditions set forth in the Floodplain Management Ordinance.

In order to minimize flood damages during the 100-year flood and the threat to public health and safety, the following conditions shall be included for any variance issued for accessory structures that are constructed at-grade and wet-floodproofed.

1.	Use of the accessory structures must be solely for parking and limited storage purposes in zone A only as identified on the community's Flood Insurance Rate Map (FIRM).

2.	For any new or substantially damaged accessory structures, the exterior and interior building components and elements (i.e., foundation, wall framing, exterior and interior finishes, flooring, etc.) below the base flood elevation, must be built with flood-resistant materials.

3.	The accessory structures must be adequately anchored to prevent flotation, collapse, or lateral movement of the structure.  All of the building's structural components must be capable of resisting specific flood-related forces including hydrostatic, buoyancy, and hydrodynamic and debris impact forces.

4.	Any mechanical, electrical, or other utility equipment must be located above the base flood elevation or floodproofed so that they are contained within a watertight, floodproofed enclosure that is capable of resisting damage during flood conditions.

5.	The accessory structures must meet all National Flood Insurance Program (NFIP) opening requirements.  The NFIP requires that enclosure or foundation walls, subject to the 100-year flood, contain openings that will permit the automatic entry and exit of floodwaters.

6.	Equipment, machinery, or other contents must be protected from any flood damage.

7.	No disaster relief assistance under any program administered by any Federal agency shall be paid for any repair or restoration costs of the accessory structures.

8.	A community shall notify the applicant in writing over the signature of a community official that (1) the issuance of a variance to construct a structure below base flood level will result in increased premium rates for flood insurance up to amounts as high as $25.00 for $100.00 of insurance coverage and (2) such construction below the base flood level increases risks to life and property.

Appeals

Once the request for a variance is heard, it must be either granted or denied. If the variance is denied, then the property owner has the right to appeal that administrative decision to the Bollinger County Circuit Court.

Record Keeping

The City of Marble Hill Floodplain Administrator will establish and maintain a file of records pertaining to floodplain management in the City Hall, both blank and completed forms, including but not limited to development permits, elevation certificates, floodproofing certificates, and variance forms.

Maps

The City will keep on file the applicable NFIP map panel for the City of Marble Hill.

Investigate Complaints

The Floodplain Administrator should investigate all complaints within one week of receiving the compliant.

Damage Assessment

In the event of a major flood or rainfall event which affects numerous structures, the Floodplain Administrator will conduct the following damage assessment procedures:

- Compile a listing of all flood damage structures

- Inspect each structure for extent of damages

- Make a detailed list of damages such as foundations, basements, floors, walls, cabinets, built-in appliances, furnaces, air conditioners, roofs, and any other damages that can be determined

- Attempt to ascertain a total damage cost for each facility

- Request the market value of each structure from the St. Francois County Assessor

- Make a determination of substantial damage by comparing cost of repair to the market value

- Use the permit procedures for all applications for repair or construction

Appendices

Attached to this document are the forms to be used in implementing this procedure:

- Floodplain Development Permit/Application

- Elevation Certificate

- Floodproofing Certificate

- Floodplain Variance Request form

- : ENFORCEMENT AND EFFECTIVE DATE

- This Ordinance shall take effect immediately upon passage and approval.
The City Administrator and Utility Department shall oversee implementation and enforcement of these fees and procedures.
The ordinance and its fee schedule shall be posted at City Hall and made available via the City’s website.

- WHEREAS, this Ordinance having been read for the first time, this Oct. 20th 2025, the Board of Aldermen voted as follows:

- Alderwoman Caldwell ____________________
Alderman McCain ____________________
Alderman Lynch ____________________
Alderman Barker ____________________

- WHEREAS, this Ordinance having been read for the second time, this Oct. 20th 2025, the Board of Aldermen voted as follows:

- Alderwoman Caldwell ____________________
Alderman McCain ____________________
Alderman Lynch ____________________
Alderman Barker ____________________

Accepted by the City’s Board of Aldermen:

Date:____________________________

Signature:_________________________		Signature:_________________________

Mayor								City Clerk

___Seal______________________________