# Appendix L: Equipment Failure Triage — Fix Fast, Fail Smart

## Purpose

When equipment fails, people freeze. A triage plan keeps product flowing while maintenance fixes the issue. This isn’t about heroics—it’s about discipline under pressure. Every minute matters, and every move must be deliberate.

## 1. The 3-Minute Rule — Establish Control Fast

Within three minutes of a failure, every leader on the floor should be able to answer three questions:

1. What failed — equipment ID or area

2. Where it happened — location, zone, or workstation

3. What’s impacted — tasks, people, or orders

If those answers aren’t clear within three minutes, it’s not triage—it’s chaos. Train supervisors to respond automatically and communicate clearly.

## 2. The 5-Step Triage Flow

### Step 1: Secure and Report (0–3 Minutes)

• Stop all movement in the immediate area.
• Tag out unsafe equipment (lockout for electrical or hydraulic).
• Notify the Maintenance Lead with equipment ID, issue, and location.
• Log downtime start in WMS or downtime tracker.
• Place safety cones and signage before resuming work nearby.

### Step 2: Assess and Prioritize (3–10 Minutes)

Maintenance classifies severity and estimates downtime. Supervisors adjust labor or reroute work immediately.

| Severity Level | Description | Response Time |
| --- | --- | --- |
| Critical | Stops flow to shipping or receiving | Immediate |
| Major | Impacts one zone or work cell | <30 minutes |
| Moderate | Slows performance or localized issue | <2 hours |
| Minor | Nuisance or cosmetic issue | <24 hours |

### Step 3: Contain and Continue (10–30 Minutes)

• Reassign operators to backup equipment or manual work.
• Divert orders to alternate routes (e.g., backup printer, secondary dock).
• Communicate ETA for recovery to affected leads.
• Keep floor calm—control tempo and posture.
Goal: Keep product moving, even at reduced speed.

### Step 4: Repair and Verify (During Fix)

• Maintenance works under proper lockout/tagout.
• Supervisor monitors progress and communicates any delays.
• Operator verifies functionality after repair, with supervisor present.
• Remove lockout only after verbal confirmation from both maintenance and supervisor.
• Confirm equipment status in WMS before resuming normal operations.

### Step 5: Debrief and Log (End of Incident)

• Record total downtime, cause, fix, and operator impact.
• Identify if failure was random or pattern-based (battery neglect, missed PM, misuse).
• Post incident summary on huddle board for 24 hours.
• If failure repeats twice, schedule preventive maintenance review.

## 3. Equipment Severity Matrix

| Level | Description | Example | Response Time | Action |
| --- | --- | --- | --- | --- |
| Critical | Halts core operation | Conveyor mainline down | Immediate | Shift labor, escalate to Ops Manager |
| Major | Affects one zone or key tool | Forklift hydraulic leak | <30 min | Swap equipment, call tech |
| Moderate | Slows performance | Label printer jam | <2 hr | Switch to backup, fix locally |
| Minor | Nuisance or repetitive issue | Loose guard, warning light | <24 hr | Log for PM schedule |

## 4. Backup Playbook — Stay Ready, Not Lucky

Every DC should have an Equipment Contingency Kit, tested quarterly:
• 1 spare label printer per shift
• 2 pre-configured portable scanners
• 1 manual pallet jack per 5 powered units
• Extra charged batteries rotated daily
• Backup handheld with maintenance app access
• Paper pick sheets and blank BOLs

Label it, seal it, and test it quarterly—unverified backups don’t count.

## 5. Preventive Maintenance Tie-In

| Metric | Target | Purpose |
| --- | --- | --- |
| PM Compliance | 100% | Every scheduled check completed on time |
| Downtime | <2% of total operating hours | Keeps failures rare |
| Top-Cause Review | Weekly 15-min standup | Closes the loop between Ops and Maintenance |
| Visual Tracker | Days since last unplanned outage | Builds awareness and pride |

## 6. Communication Protocol

| Audience | Timing | Message Example |
| --- | --- | --- |
| Supervisors | Within 5 min | “Forklift #12 down in Zone B—reassigning pickers.” |
| Maintenance Lead | Immediate | “Conveyor B motor failure—mainline stopped.” |
| Ops Manager | Within 10 min | “Critical outage, ETA 45 min recovery, 3 orders delayed.” |
| Customer Service | Within 15 min (if delay) | “Minor delay due to equipment repair; ETA adjusted.” |

## 7. Metrics That Matter

| Metric | Target | Purpose |
| --- | --- | --- |
| Average Downtime per Incident | <30 minutes | Tracks repair agility |
| Repeat Equipment Failures | <5% per month | Identifies PM gaps |
| % of Downtime Logged with Triage | 100% | Ensures accountability |
| % of Downtime Mitigated via Backup | 80%+ | Measures operational resilience |

## 8. Field Wisdom

Don’t blame the equipment. Machines fail—how you respond defines leadership.
When chaos hits:
• Keep people safe
• Keep product moving
• Document everything

Calm under pressure isn’t discovered—it’s trained. Run triage drills quarterly until reaction becomes reflex.