# Warehouse Safety Checklist Form

## Purpose

This form helps warehouse leaders perform short, consistent safety walkthroughs. It ensures aisles, equipment, and people follow safety standards daily. The goal is to prevent incidents, spot hazards early, and reinforce accountability.

## Instructions

1. Frequency: Complete this checklist once per shift or at least weekly.
2. Scope: Walk all active zones: receiving, storage, picking, and shipping.
3. Scoring:
   ✅ Yes = Safe/Compliant
   ⚠️ Partial = Needs attention
   ❌ No = Unsafe or noncompliant
4. Follow-up: Log issues in the 'Corrective Action' column and assign an owner.
5. Time: The Walk should take no more than 20–30 minutes.

## Section 1 – Aisles & Housekeeping

| Item | Standard | Yes | Partial | No | Corrective Action / Notes |
| --- | --- | --- | --- | --- | --- |
| Aisles clear | Minimum 6 ft. for two-way traffic, no obstructions or debris | ☐ | ☐ | ☐ |  |
| Staging organized | No pallets blocking travel paths | ☐ | ☐ | ☐ |  |
| Corners and intersections | Mirrors or cutouts for visibility | ☐ | ☐ | ☐ |  |
| Floors dry | Slip hazards cleaned or marked immediately | ☐ | ☐ | ☐ |  |

## Section 2 – Equipment & Tools

| Item | Standard | Yes | Partial | No | Corrective Action / Notes |
| --- | --- | --- | --- | --- | --- |
| Forklift horns operational | Horn sounds before movement | ☐ | ☐ | ☐ |  |
| Inspection tags current | Pre-use check tags signed and visible | ☐ | ☐ | ☐ |  |
| Chargers and cables secure | No frayed cords or tripping hazards | ☐ | ☐ | ☐ |  |
| Battery room organized | PPE available and ventilation functional | ☐ | ☐ | ☐ |  |

## Section 3 – Personal Protective Equipment (PPE)

| Item | Standard | Yes | Partial | No | Corrective Action / Notes |
| --- | --- | --- | --- | --- | --- |
| High-visibility vests | All personnel wearing proper color | ☐ | ☐ | ☐ |  |
| Foot protection | Steel/composite toe shoes required | ☐ | ☐ | ☐ |  |
| Gloves and eyewear | Used for material handling and cutting | ☐ | ☐ | ☐ |  |
| Hearing protection | Available and worn in loud zones | ☐ | ☐ | ☐ |  |

## Section 4 – Racking & Storage

| Item | Standard | Yes | Partial | No | Corrective Action / Notes |
| --- | --- | --- | --- | --- | --- |
| Loads secure | No damaged or overhanging pallets | ☐ | ☐ | ☐ |  |
| Weight limits posted | Visible and legible signage | ☐ | ☐ | ☐ |  |
| Damaged beams tagged | Unsafe areas marked and reported | ☐ | ☐ | ☐ |  |
| Overhead clearance | Meets OSHA 1910.176(b) requirements | ☐ | ☐ | ☐ |  |

## Section 5 – Fire & Emergency

| Item | Standard | Yes | Partial | No | Corrective Action / Notes |
| --- | --- | --- | --- | --- | --- |
| Fire extinguishers inspected | Tags current within 30 days | ☐ | ☐ | ☐ |  |
| Exits clear | No blocked doors or equipment | ☐ | ☐ | ☐ |  |
| Exit lights functional | Visible from 100 ft. | ☐ | ☐ | ☐ |  |
| Eyewash/shower stations | Accessible and operational | ☐ | ☐ | ☐ |  |

## Section 6 – Observations & Employee Input

What’s the biggest safety concern noticed today? ________________________________

Any near misses or incidents this week? _______________________________________

Who suggested today’s safety improvement? ____________________________________

## Inspector Details

| Inspector Name | ________________________________ |
| --- | --- |
| Date / Time | ________________________________ |
| Department / Zone | ________________________________ |
| Follow-Up Owner | ________________________________ |

Next Review Date: ___________________________

## Field Tip

Safety improves when it’s visible, consistent, and honest. The best checklists are conversations, not audits.