PERMANENT SEEDING NOTES

Scope: Planting permanent, long lived vegetative cover on graded and/or cleared areas and areas that have been in temporary vegetation for more than 6 months.

Standards: The following notes shall conform to Section B-4 of the “2011 MARYLAND STANDARDS AND SPECIFICATIONS FOR SOIL EROSION AND SEDIMENT CONTROL” published jointly by the Maryland Department of Environment - Water Management Administration, the National Resource Conservation Service and the Maryland Association of Soil Conservation Districts.

The seed bed shall be prepared by loosening the soil to a depth of 3 to 5 inches and incorporating the lime and fertilizer into this loosened layer of soil. See section B-4-2.

For sites over 5 ac. soil tests will be performed. Soil tests will be conducted by the University of Maryland or a recognized commercial laboratory. Minimum soil conditions shall meet the requirements of section B-4-2-A-2-a, otherwise soil amendments or topsoil will need to be applied. Topsoiling may occur when soil conditions meet the minimum requirements as stated in section B-4-2-B. Soil amendments must meet the requirements as set forth in section B-4-2-C and must be applied as indicated by the soils tests.

For sites of 5 ac. or less of disturbance, the following fertilizer and lime rates shall apply.

Fertilizer shall consist of a mixture of 10-20-20 and be applied at the following rates:

N = 45 lb. per acre (1 lb. per 1000 sq.ft.)    P205 = 90 lb. per acre (2 lb. per 1000 sq.ft.)

K20 = 90 lb. per acre (2 lb. per 1000 sq.ft.)

Lime shall be applied at a rate of 2 tons per acre (90 lb. per 1000 sq.ft.)

Seed type, turfgrass or sod application shall meet the requirements in section B-4-5. Seed tags shall be made available to the inspector to verify the type and application rate of seed used. Mulch type and its application will meet the requirements in section B-4-3 a, b and c, and will be applied along with seed or immediately after seeding

Seeding mixtures shall be selected from or will be equal to those on Table B-3.  The seeding chart below will need to be placed on and filled in on the sediment control plan

| Hardiness Zone (from Figure B.3): _______ |  |  |  |  | Fertilizer Rate |  |  | Lime Rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seed Mixture (from Table B.3): _________ |  |  |  |  | (10-20-20) |  |  |  |
| No. | Species | Application Rate (lb/ac) | Seeding Dates | Seeding Depths | N | P2O5 | K20 |  |
|  |  |  |  | 1/4-1/2 in | 45 pounds per acre (1.0 lb/1000 sf) | 90 lb/ac (2 lb/1000 sf) | 90 lb/ac (90 lb/1000 sf) | 2 tons/ac (90 lb/1000 sf) |
|  |  |  |  | 1/4-1/2 in |  |  |  |  |
|  |  |  |  | 1/4-1/2 in |  |  |  |  |