TEMPORARY SEEDING NOTES

Scope: Planting short term (no more than 6 Months) vegetation to temporarily stabilize any areas where soil disturbance has occurred, until the area can be permanently stabilized with vegetative or non-vegetative practices.

Standards: The following notes shall conform to Section B-4 of the“2011 MARYLAND STANDARDS AND SPECIFICATIONS FOR SOIL EROSION AND SEDIMENT CONTROL” published jointly by the Maryland Department of Environment - Water Management Administration, the National Resource Conservation Service and the Maryland Association of Soil Conservation Districts

The seed bed shall be prepared by loosening the soil to a depth of 3 to 5 inches and incorporating the lime and fertilizer into this loosened layer of soil. See section B-4-2

For temporary stabilization, fertilizer shall consist of a mixture of 10-20-20 and be applied at a rate of 436 lb. per acre (10 lb. per 1000 sq. ft.) and will meet the requirements in section B-4-2. Lime shall be applied at a rate of 2 tons per acre (90 lb. per sq. ft.) and shall meet the requirements in section B-4-2 and B-4-4

Seed type and application shall meet the requirements in section B-4-3 Seed tags shall be made available to the inspector to verify the type and rate of seed used.

Mulch type and its application will meet the requirements in section B-4-3 a, b and c and will be applied along with the seed or immediately after seeding

Seeding mixtures shall be selected from or will be equal to those on Table B.1 (page B.20).

Temporary Seeding Summary

The seeding chart below will need to be placed on and filled in on the sediment control plan

| Hardiness Zone (from Figure B.3): _______ |  |  |  |  | Fertilizer Rate (10-20-20) | Lime Rate |
| --- | --- | --- | --- | --- | --- | --- |
| Seed Mixture (from Table B.1): _________ |  |  |  |  |  |  |
| No. | Species | Application Rate (lb/ac) | Seeding Dates | Seeding Depths |  |  |
|  |  |  |  |  | 436 lb/ac (10 lb/1000 sf) | 2 tons/ac (90 lb/1000 sf) |

barb\bryan\temporaryseeding_notes