ENGINE START

Exterior Inspection/ Cockpit set up	COMPLETE

Parking Brake	ON

Doors	CLOSED/ LIGHT OFF

PAX Brief	COMPLETE

Annunciator Panel	Appropriate

Anti-Collision Light	RED

Boost Pump	ON

Engine	2, 3, 1 Start

Engine Parameters	CHECKED

# AFTER START/ BEFORE TAXI

DC Power Selector	NORMAL

External Power	DISCONNECTED

Circuit Breakers	IN

Bus Tie	FLIGHT NORMAL /LIGHTS OUT

Electrical Volts/Amps	CHECKED

Engine Controls	CHECKED

Fuel Panel	6 ON/ 4 CLOSED

Bleed Air	TESTED/ CHECKED

Compass	COMPARE

Windshield Heats	NORMAL/ ON

*Anti-Ice 	TESTED / SET FOR TO

Cockpit Lighting	AS REQUIRED

EMER Horizon	ON

Altimeters	SET

GPWS	TESTED

Anti Skid	TESTED

Hydraulics	2 PRESSURES 2 QUANTITES

Hydraulic Standby Pump	ON

*No 1 Stall	Tested

Auto Pilot	CHECKED/DISINGAGED

Yaw damper	AS REQUIRED

Transponder/TCAS	TA/ Tested

Slats/Flaps	CYCLED/SET FOR T.O.

Flight Director & Instruments	BRIEFED

T.O. Briefing	BRIEFED

Masks & Oxy	CHECKED

DV Window	CLOSED

# TAXI

Parking Brake	OFF

Taxi Light	ON

Brakes 1&2	CHECKED/ #1& ON

Thrust Reverser	CHECKED/ STOWED

*Engine Computers	CHECKED/ ON

Flight Controls	CHECKED

Xpndr, TCAS	TA

Radar	AS REQUIRED

# BEFORE TAKEOFF

Parking Brake	IN/ LIGHT OUT

APU& Bleed air	CONFIRM OFF

Start Selectors	AIR START

Exterior lights	SET

FATSP	CHECK

# LINE UP CLEARED FOR TAKE OFF

Pitot Heats	ON

Landing Light	ON

Warning Panel / Flags	CLEAR

Engine Instruments	RNWY Heading ALT

# CLIMB

LDG Gear/ Taxi Light/ YD	UP OFF ON

Slats & Flaps	CLEAN

Start Selectors	GROUND START

Anti-ice	AS REQUIRED

Cabin Pressure	CHECK

Cabin Temp	CHECKED

Hydraulics	CHECK

Climb Power	CHECK

# * First Flight of The Day

# N980DM  FALCON 50

# 10K CHECKS

Lights	SET

No Smoking Seat Belts	AS REQUIRED

Stby Hyd Pmp	AUTO

# 18K CLIMB/ CRUISE/ STATION

Masks/ Oxy	CHECK

Altimeters	29.92

Circuit Breakers	CHECK

Gen/ Volts/Amps	CHECK

Fuel Panel	CHECK

Anti-ice systems	AS REQUIRED

Ext Lts. / S. Belts/ No Smoking 	AS REQUIRED

Engine Parameters	CHECK

Cabin Press / Temp	CHECK

Hydraulics	CHECK

# DESCENT

Cabin Door Curtain	AS Required

Cabin Controller	SET

Anti-icing.	AS REQUIRED

Land Data/ Bugs	COMPLETE / SET

ATIS	NOTED

Risk Assessment	BRIEF

Fuel Panel XFR Intercom/X-feed    	CLOSED

| 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 106 | 108 | 110 | 112 | 114 | 116 | 118 | 120 | 121 | 123 | 125 |

# 18K DESCENT/ APPROACH

Altimeters (at transition)	SET/CROSS CHECK

Circuit Breakers	CHECK

Gen/ Volts/Amps	CHECK

Anti-ice	AS REQUIRED

Ext lights/ Belts/ Smoke Lights AS REQUIRED

LDG Light	AS DESIRED

Engine Parameters	CHECK

Cabin Press / Temp	CHECK

Hydraulics/ STBY Pump	CHECK/ON

FMS/Avionics	CHECK/ SET

Radio Altimeter	SET

Approach Brief	COMPLETE

Crew/ PAX Brief	COMPLETE

Start Selectors	AIR START/IGN ON

# BEFORE LANDING

Landing Gear/ Anti-skid	DOWN / CHECK

Slats–Flaps	LDG POSITION

Airbrakes	IN/LIGHTS OUT

Yaw Damper	AS REQUIRED

Warning Panel	CHECK

Hydraulics	CHECK

Auto Pilot	OFF

Taxi Light	ON CLEARED TO LAND

# AFTER LANDING

Pitot/ Windshield Heat	ALL OFF

Anti-ice	WING OFF/ ENG AS REQ

Ignition	GRD START

Bus Tie	TIED

APU (as applicable)	MASTER/ GEN/ START

Landing Lights, Strobes	OFF

Air Brakes	0

Slats/ Flaps 	CLEAN

Thrust Reversers	STOWED

Trim	SET FOR TAKEOFF

Radar/ Transponder	OFF/ AS REQ

APU/ Bleed Air	AS REQ

# PARKING

Taxi Light	OFF

Parking Brake	SET

STBY Pump	OFF

Engines	CUT OFF

Fuel Panel	SET

Anti Collision Light	OFF

Seat Belts/ No Smoking Light	AS REQ

Transponder	AS REQ

Flight Time/ Fuel Burn	RECORD

* First Flight of The Day

# SHUTDOWN

APU	SHUTDOWN

Boost Pumps	OFF

NAV Lights	OFF

Seatbelt/ Emer Lights/ cabin	OFF

Cabin Lights	OFF

FMS/ Avionics Masters	OFF

Emerg Battery	OFF

Interior lights	SET

Bus Tie	FLIGHT NORMAL

Pilot window	CLOSED

APU Master	OFF

BAT 1 and BAT 2	OFF

Bus C & D	OFF

Chocks/ Parking Brake	IN / RELEASE

# QUICK TURN

FMS Flight time / Fuel	RECORD / ZERO

Clocks	ZERO

Fuel Quantity / Counters 	CHECKED / ZERO

Cabin Press 	SET

Clearance and Code 	COPY AND SET

FMS	LOAD/SET

TOLD/ Vspeeds	COMPLETE/ SET

Engines	START

Bus Tie	FLIGHT NORM

Generator Amps/Volts	CHECK

Windshield Heats	ON

Anti-ice	AS REQ

Warning Panel	CHECK

Slats–Flaps	SET FOR TAKEOFF

Passenger briefing Smoking Seatbelts.

Emergency exits and entry door

Emergency exits diagram

Survival equipment

Flotation equipment &Ditching

Oxygen equipment

Briefing cards

# FUELING

LO level 13,195 lbs High 15,514 lbs

PROFILE

Climb 260/.72 849

# N980DM Falcon 50 Normal Checklist

Oct 11 2026 Orig