# Pre Flight

POWER OFF CHECKS	Cockpit Flow

DC Power	Normal

Radar	OFF

Batteries	ON/Lights OFF

Volts and amps	CHECK

Emergency Battery VLTS	CHECKED

#2 P BK Light	NOT FLASHING

Pilot Window	AS REQUIRED

Parking Brake	SET/LIGHT STEADY

Speed Bugs	SET

Fuel	CHECKED SET TO REAR

Fuel Counters	ZEROED

Gross weight Count	SET

Radios	OFF

Batteries	OFF

Circuit Breakers	IN

Generators	ON

Inverters	AS REQ

Engine Comptrs	ON

STRT Selectors	GROUND START

XFER Pumps	OFF

L and R Intercom	OFF

Boost Pumps	OFF

L and R Crossfeed	CLOSED

Pitot Heats	OFF

Windshield Heat	OFF

HP Bleed Switches and PRV	AUTO

Isolation Valve	OPEN

Cabin and Crew Bleed Air	AUTO

Bag Air	ON

Ani Ice Switches	OFF

Windshield Wipers	OFF

Aircraft Lighting	OFF

Floor Heat	Both OFF

Static Selector	Normal

Compass Switches	SLAVED

Brake Selector	NO 1 ON

Hydraulic Standby Pump	OFF

Temperature Controllers	AUTO/12 O’CLOCK

T/R Emergency Stow Switch	GAURDED

Emergency Fuel Transfer SwitchOFF GUARDED

Altitude Controller	SET

Pressurization Selector	AUTO/GUARDED

Manual Pressurization Knob	GREEN INDEX

Fire Pull Handles	3 IN

Pitot Static Selector	Normal

Oxygen Mask Interphone	CHECKED

Oxygen Mask/ System	TESTED/RESET

Nose Lever	OPEN

ELT/Flap Override (if equipped)	GUARDED

Emer Slat Switch	GUARDED

Slat/Flap Handle	CLEAN

Airbrake	IN

Mach Trim	OFF

Yaw Damper	OFF

Auto Pilot	OFF

# POWER ON	APU START

Batteries	ON CHECKED

Bus Tie	TIED/light on

Bus C& D	TIED

Nav Lights	ON

EMER Batteries	ON

Master Warning Panel	Tested

Fire Warning	TESTED

Landing Gear Panel	TESTED

No 2 Boost Pump	ON

APU	MASTER, GEN, START

BAT’s 1&2	CHARGING

Radio masters	ON

FMS’s	ON

Fasten belts/No Smoking, EMER Lights	ON

Standby Horizon	UNCAGED

TO Config/ ENG 2	TESTED

APU Bleed  stable 2 mins	AS DESIRED

*Hydraulic SBY Pump 	ON/AUTO CHECKED

*Stall 2	TESTED

*Airbrake	CYCLED IN/LIGHTS OUT

*Hydraulic STBY Pump	OFF

*Trims (5)	CHECK

Comm and Nav Frequencies	8 SET

FMS	FUEL, FLT PLAN….SET

Park Brake	SET/ LIGHT ON

# COCKPIT SET UP

# Clearance	received

Altitude select	set

Transponder	set

- FMS setup

  - Vector/ Direct

  - Departure procedure

  - First turn altitude.

  - Course/ Vector to first fix

  - Deviations for Weather

  - Route

  - Arrival

  - Approach

- FD mode	set

# Brief

- TOLD

  - V speeds

  - Power

  - AC Configuration

  - Runway

  - wind/ weather condition

  - Ice configuration and plan

  - AC weight/ Max wt

  - Departure, Routing, Arrival, Approach

  - Level off altitude Alternate plan

  - Eng Failure Alternate Departure Procedure

Amenities:

  - Lav Service

  - Ice

  - Coffee pots

  - Snacks

Oct 11 2026

DM Air iPad APPS

ForeFlight

  - FF Create Flight

  - APG Weight bal to FF and WB@DMair

    - FF W&B to flight files

  - FF Runway analysis

    - Told to flight files

  - FF share flight with copilot

Jet insight

  - JI pre

    - W&B Data

  - Pax & Crew itineraries to FF flight

  - Fratt

After Landing

FMS pic

Jet insight

- post

  - Times

  - Fuel

- Duty off

- Group and submit flight log

Quality Assurance Checklist

A Aircraft -MELS- Green aircraft

W WEATHER/ Departure Arrival

A Airport -NOTAMS- Depart Route Arrival

R ROUTE FMS both checked

E EVERYTHING else, coffee pots , catering,  maintenance, write ups, manuals