ThompsonTraders

Assembly Instructions

ThompsonTraders

Glass Tower Display Assembly Instructions

Contents

| Qty | Description |
| --- | --- |
| 1 | Wooden base |
| 5 | Shelves 18 X 18 tempered glass |
| 3 | Sides 18 X 48 tempered glass |
| 12 | 2 screw brackets |
| 12 | 3 screw brackets |

Tools required

Flat blade screw driver

Measuring tape

Glass cleaner and towels

Assembly (Numbered steps shown on drawing)

- Place 1 side (18 X 48) on the floor

- Along each long side place a  3 screw bracket at 1’, 2’ and 3’ with the long slot inserted on the glass and the short slot pointing up. This will use a total of 6 brackets.

- Along each long side place a 2 screw bracket at 6” and 42” using a total of 4 brackets.

- Insert vertically into the 3 screw brackets 3 shelves (18 X 18).

- Insert the 2nd and 3rd sides (18 X 48) vertically into the 2 screw brackets.

- Place 6-3 screw brackets with the long slot on the top of the 2nd and 3rd sides and the short slot on shelves on the top side.

- Make sure the sides are all aligned top and bottom and the shelves are square to the sides and properly spaced.

- Tighten all the screws.

- Place remaining 2 end shelves (18 X 18) on the floor and install on opposite sides 2 screw brackets at 4” and 12” using the remaining 8-2 screw brackets.

- Tighten the screws holding the brackets to the shelves

- Slide one of the shelves (18 x18) on each end so the brackets on the shelves are on the top of the 2 vertical sides and tighten the screws.

- To stand the tower up, roll 90 degrees to one side and then lift from one end. This allows the tower to roll upright on 2 of the 2 screw brackets limiting the amount of stress on the structure.

- Place the wood base and lift the tower onto it leaving an equal distance from the glass to the edge of the base on all 4 sides.

- Clean the tower with glass cleaner.

ThompsonTraders

Glass Tower Display Assembly Instructions

Step 2

Step 3 and Step 9

Figure 2

Step 4 and Step 6

Step 5 and Step 11