PRODUCT SPECIFICATION – SECTION 09 54 00 – ACOUSTICAL CEILING AND WALL PANELS
Surfacestm Panels: Fabric Wrapped and Fabric Faced series

2.1 MANUFACTURER

A. Chroma Collections

    - Address: 415 Nelson Pl., La Crosse, WI 54601

    - Phone Number: 608-785-1232

    - Fax: 608-785-1233

    - Website: www.chromacollections.com

    - Email: customerservice@chromacollections.com

  - ACOUSTIC CEILING UNITS

A. Acoustical Ceiling Panel (ACP) - [Type ACP-1]

- Name: Surfacestm Fabric Wrapped and Fabric Faced panels

- Physical Characteristics

      - Pattern: [Rectangle, Square]

      - Size:

1. Rectangle

W/L: [24”x 96”, 36”x 96”, 48’’x96”, 24”x 72”, 36”x 72”, 48”x 72”] [Custom size 48”x96”, 48”x120”]

2. Square

W/L: [24”x24”, 30”x30”, 48”x 48”]

3. Custom

      - Thickness: [1”, 1-1/8’’, 2’’, 2-1/8’’]

      - Edge: [Square, Beveled]

      - Fabric Format: [Fabric wrapped around edges, Fabric face only]

      - Fabric Material: [Guilford of Maine FR-701, Guilford of Maine Anchorage, Other]

      - Core Composition: Fiberglass , 6.0 PCF density

      - Recycled Content: 50% minimum

      - Mounting: [Impaling clips and adhesive (adhesive supplied by others), Z-Clips/Z-Bars]

    - Performance Criteria

- Noise Reduction Coefficient (NRC): [1’’ = 0.90, 2’’ = 1.0]

- Flame Spread Classification per ASTM E84: Class A

Chroma Collections shall be held harmless for any damages resulting from the use of this specification guide