PRODUCT SPECIFICATION – SECTION 09 54 00 – ACOUSTICAL CEILING PANELS
Flatstm Canopies: Scrim Surfaced Series

2.1 MANUFACTURER

A. Chroma Collections

    - Address: 415 Nelson Pl., La Crosse, WI 54601

    - Phone Number: 608-785-1232

    - Fax: 608-785-1233

    - Website: www.chromacollections.com

    - Email: customerservice@chromacollections.com

  - ACOUSTIC CEILING UNITS

A. Acoustical Ceiling Panel (ACP) - [Type ACP-1]

- Name: Flatstm Canopy

- Physical Characteristics

      - Pattern: [Rectangle, Square, Oval, Circle, Triangle, Trapezoid, Octagon, Hexagon, Concave, Convex, Chevron] (See PDF attachment)

      - Size:

1. Rectangle

W/L: [23”x 94”, 36”x 94”, 46’’x94”, 23”x 70”, 36”x 70”, 46”x 70”]

2. Square

W/L: 46”x 46” [w/standard 90-degree corners, w/rounded corners 4’’Radius,

w/square 4’’Radius cut outs]

3. Oval

W/L: [46”x 94”, 46’’x 70’’]

4. Circle

DIA 46” [standard circle-no opening,

w/cut out DIA 12” center]

5. Triangle

W/L: [Equilateral 46”x 39.84”, Right 46”x 46”, Half Right 32.52”x 32.52”]

6. Trapezoid

W/L: 46”x 46”

7. Octagon

W/L: 46”x 46”

8. Hexagon

W/L: 39.84”x 46”

9. Concave

W/L: 46”x 46”

10. Convex

W/L: 46”x 46”

11. Chevron

W/L: 46”x 46”

      - Thickness: [1”, 2’’]

      - Edge: [Square]

      - Finished Surface: Painted Scrim

      - Finished Surface Color: [White, Black, Custom Flat Latex Color #]

      - Core Composition: Fiberglass

      - Recycled Content: 34%

      - Mounting: [Impaling clips and adhesive (adhesive supplied by others), Rotofast RCA2P Cloud Anchor ]

    - Performance Criteria

- Noise Reduction Coefficient (NRC) @ 1’’ thickness: 0.90 (1.20 Sabins per SF)

- Noise Reduction Coefficient (NRC) @ 2’’ thickness: 1.00 (1.38 Sabins per SF)

- Flame Spread Classification per ASTM E84: Class A

Chroma Collections shall be held harmless for any damages resulting from the use of this specification guide