Fixed Film Nitrification

- Nitrification is a process carried out by a series of bacterial populations that sequentially oxidize ammonium to nitrate with intermediate formation of nitrite carried out by nitrosomonas and nitrobacter.

  - True

  - False

- Nitrite is converted to nitrate (NO3-) by nitrosomonas bacteria.

  - True

  - False

- Major factors that influence the kinetics of nitrification include:

  - organic loading and hydraulic loading

  - temperature and pH

  - dissolved oxygen concentration, and filter media

  - All of the above

- Some advantages of Trickling Filters include:

  - Simple, reliable process.

  - Suitable in areas where large tracts of land are not available for a treatment system.

  - Ability to handle and recover from shock loads.

  - All of the above

- Some disadvantages of Trickling Filters include:

  - Reduction of ammonia-nitrogen concentrations in the wastewater.

  - Relatively high incidence of clogging.

  - Potential for vector and odor problems.

  - All of the above

- A complex population of organisms that form a “slime growth” on a Trickling Filter that break down the organic matter and convert ammonia to nitrate in wastewater is called?

  - Nitrosomonas bacteria

  - Zoogleal Mass

  - Algae

  - All of the above

- Ammonia nitrogen can be oxidized to nitrogen gas through the use of chlorine?

  - True

  - False

- Nitrification bacteria tend to slow down and become inactive when wastewater temperatures drop below 10ºC.

  - True

  - False

- Nitrification can be accomplished using   _________________ growth reactors   or __________________ growth reactors.

- The two main nutrients which cause algal growths in receiving waters are ______________ and __________.

- The daily flow to a trickling filter is 5,300,000 gpd.  If the BOD content of the trickling filter influent is 280 mg/L, how many lbs/day BOD enter the trickling filter?

a.   11,670 lbs/day      b. 12,470 lbs/day    c. 179.2 lbs/day     d. 437 lbs/day

- The suspended solids concentration entering a trickling filter is 146 mg/L.  If the suspended solids concentration in the trickling filter effluent is 46 mg/L, what is the suspended solids removal efficiency of the trickling filter?

a.  98%                   b. 56%                  c. 68%                   d. 104%

- A treatment plant receives a flow of 1.5 MGD.  If the trickling filter effluent is recirculated at the rate of 2.12 MGD, what is the recirculation ratio?

a.  0.68           b. 1.4            c. 2.8       d.    5

- The following flows were recorded for the months of June, July and August: June – 105.2 MG, July – 129.6 MG, and August 142.8 MG.  What was the average daily flow for this three month period?

a. 3.2 MGD           b.  6.4 MGD        c.  1.4   MGD        d. 4.1 MGD

- A trickling filter, 90 ft in diameter with a media depth of 5 ft, receives a flow of 1,200,000 gpd.  If the BOD concentration of the primary effluent is 205 mg/L, what is the organic loading on the trickling filter in lbs BOD/day/1,000 cu ft?

a. 64.5                       b.15.5                  c. 129                d.  31

Answers:

- True              6. B					11. B

- False             7. A					12. C

- D                  8. A					13. B

- D                  9. Suspended or attached		14. D

- B & C         10. Nitrogen and phosphorus		15. A

Detailed Math Answers:

- The daily flow to a trickling filter is 5,340,000 gpd.  If the BOD content of the trickling filter influent is 280 mg/L, how many lbs/day BOD enter the trickling filter?

BOD loading, lbs/day = BOD, mg/L x Flow, MGD x 8.34 lbs/gal

280 mg/L x 5.34 MGD x 8.34 lbs/gal = 12,470 lbs/day

- The suspended solids concentration entering a trickling filter is 146 mg/L.  If the suspended solids concentration in the trickling filter effluent is 46 mg/L, what is the suspended solids removal efficiency of the trickling filter?

Removal efficiency, % =  In- Out

In         x 100 %

146-46

146      x 100 % = 68%

- A treatment plant receives a flow of 1.5 MGD.  If the trickling filter effluent is recirculated at the rate of 2.12 MGD, what is the recirculation ratio?

Recirculation ratio = recirculation flow

plant flow

2.12 MGD

1.5 MGD    = 1.4

- The following flows were recorded for the months of June, July and August:

June – 105.2 MG, July – 129.6 MG, and August 142.8 MG.  What was the average daily flow for this three month period?

Average daily flow, MGD =   Total monthly flows, MG

Total number of days

Flow total:  105.2 + 129.6 + 142.8 = 377.6 MG

Total days =   June (30 days) + July (31 days) + August (31 days)  = 92 days

  - MG

92 days        =   4.1 MGD

- A trickling filter, 90 ft in diameter with a media depth of 5 ft, receives a flow of 1,200,000 gpd.  If the BOD concentration of the primary effluent is 205 mg/L, what is the organic loading on the trickling filter in lbs BOD/day/1,000 cu ft?

Organic loading, lbs BOD/day 1000/cu ft =     BOD applied, lbs,day

volume of media, 1,000 cu ft.

Calculate: BOD loading, lbs/day = BOD, mg/L x Flow, MGD x 8.34 lbs/gal

205 mg/L x 1.2 MGD x 8.34 lbs/gal   = 2046.72 lbs/day

Calculate: volume of media

Volume, cu ft = 0.785 x D2 x H

= 0.785 x 902 x 5 ft = 31,792.5 cu-ft

31792.5/ 1000 cu ft   or  31.8 x 1,000 cu ft

Organic loading, lbs BOD/day/cu ft	= 2046.72 lbs/day =   64.5 lbs BOD/day/1000 cu ft

31.8 x 1,000 cu ft