Fixed Film Nitrification

- Nitrification is a process carried out by a series of bacterial populations that sequentially oxidize ammonium to nitrate with intermediate formation of nitrite carried out by nitrosomonas and nitrobacter.

  - True

  - False

- Nitrite is converted to nitrate (NO3-) by nitrosomonas bacteria.

  - True

  - False

- Major factors that influence the kinetics of nitrification include:

  - organic loading and hydraulic loading

  - temperature and pH

  - dissolved oxygen concentration, and filter media

  - All of the above

- Some advantages of Trickling Filters include:

  - Simple, reliable process.

  - Suitable in areas where large tracts of land are not available for a treatment system.

  - Ability to handle and recover from shock loads.

  - All of the above

- Some disadvantages of Trickling Filters include:

  - Reduction of ammonia-nitrogen concentrations in the wastewater.

  - Relatively high incidence of clogging.

  - Potential for vector and odor problems.

  - All of the above

- A complex population of organisms that form a “slime growth” on a Trickling Filter that break down the organic matter and convert ammonia to nitrate in wastewater is called?

  - Nitrosomonas bacteria

  - Zoogleal Mass

  - Algae

  - All of the above

- Ammonia nitrogen can be oxidized to nitrogen gas through the use of chlorine?

  - True

  - False

- Nitrification bacteria tend to slow down and become inactive when wastewater temperatures drop below 10ºC.

  - True

  - False

- Nitrification can be accomplished using   _________________ growth reactors   or __________________ growth reactors.

- The two main nutrients which cause algal growths in receiving waters are ______________ and __________.

Answers:

- True

- False

- D

- D

- B & C

- B

- A

- A

- Suspended or Attached

- Nitrogen and Phosphorus