PHOSPHORUS REMOVAL

1: Why is Phosphorus removed from wastewater?

a. It is a food source for fecal coli-form

b. It is a food source for algae

c. Both A & B

2: The types of Phosphorus removal system include:

- Lime Precipitation

- Aluminum Sulfate Flocculation and Precipitation

- Breakpoint Chlorination

- Luxury Uptake

- All of the above.

3: The biological treatment process whereby the bacteria usually found in the activated sludge treatment portion of the secondary treatment are withdrawn to an environment without oxygen is the _______________ ___________ removal system.

4: Equipment necessary for the Lime Precipitation process includes:

- Lime Feed Equipment

- Mixing equipment and mixing chamber

- Clarifiers

- Pumps and Piping

- All of the above

5: Using the formula below will produce the result in % Phosphorus.

Total Phosphorus – Dissolved Phosphorus = Particulate Phosphorus

Particulate Phosphorus * Total Suspended Solids / 100 = % Phosphorus

- True

- False

6:  Aluminum sulfate in combination with wastewater can flocculate phosphorus in much the same way as lime precipitation?

a.   True

b.   False

7:  Three physical or chemical reactions which take place during lime precipitation include:

- Coagulation

- Flocculation

- Depth Filtration

- Sedimentation

8:  Can Short-Circuiting affect Phosphorus removal efficiency?

- True

- False

9:  Aluminum Sulfate feed rate can be determined by performing a jar test?

- True

- False

10:  What forms of phosphorus are included in the total phosphate measurement?

- Orthophosphate

- Polyphosphate

- Organic phosphorus

- Molyphosphate

- All of the above

11. The average polymer use at the plant is 80 lbs/day. If the chemical inventory in stock

is 1,960 lbs, how many days supply is this?

- 24.5 days           b.    4 days        c. 12 days           d. 40 days

12. A feed pump dropped the polymer level in a 4 ft diameter tank 2 foot and 8 inches

over a 24 hour period.  Calculated the flow delivered by the pump in gallons per day.

a. 564 gal/day       b. 250  gal/day           c. 750 gal/day        d. 375 gal/day

13.  Calculate the actual chemical feed rate, in lbs/day, if a bucket is placed under a chemical feeder and a total of 1.8 lbs is collected during a 30-min. period.

- 76 lbs/day        b. 86.4 lb s/day      c.  89 lbs/day        d. 25 lbs/day

14. The required chemical pumping rate has been calculated to be 25 gpm.  If the

maximum pumping rate is 90 gpm, what should the percent stroke setting be?

- 14 %                  b. 42%                     c. 28%                  d. 64%

15. The desired solution feed rate was calculated to be 40 gpd.  What is the feed rate

expressed as mL/min?

- 53 ml/min         b. 105 ml/min          c. 218 ml/min      d. 15.2 ml/min

Answers:

- B				6.  A 			11. A

- A, B, D			7.  A,B,D		12.

- Luxury Uptake		8. A			13.B

- E				9. A			14. C

- B			          10. A,B,C		15. B

Detailed Math Answers

11. The average polymer use at the plant is 80 lbs/day. If the chemical inventory in stock

is 1,960 lbs, how many days supply is this?

Supply, days =       Inventory, lbs                  1,960 lbs

Usage, lbs/day               80 lbs/day        = 24.5 days

12. A feed pump dropped the polymer level in a 4 ft diameter tank 2 foot and 8 inches

over an 8 hour period.  Calculated the flow delivered by the pump in gallons per day.

Convert the tank drop from 2ft 8 inches to feet = 2 ft +  8 inches

12 inches   = 2.66 ft

Calculate gallons of polymer pumped   = Area, sq ft x drop in tank, ft  x 7.48 gal/cu ft

(0.785 x 4ft2  )  x 2.66 ft x 7. 48 gal/cu ft    = 250 gal

Calculate flow, GPD = volume pumped, gal x 24 hr/day         250 x 24

8 hrs                           =           8          = 750 gal/day

13.  Calculate the actual chemical feed rate, in lbs/day, if a bucket is placed under a chemical feeder and a total of 1.8 lbs is collected during a 30-min. period.

Feed rate = Chemical Feed lbs/day

Calculate lbs/min feed rate     =   1.8 lbs

30 min         = 0.06 lbs/min

Convert lbs/min to lbs/day    0.06 lbs   x 24 hr   x   60 min

min         day             h                =             86.4 lbs/day

14. The required chemical pumping rate has been calculated to be 25 gpm.  If the

maximum pumping rate is 90 gpm, what should the percent stroke setting be?

Setting, % = desired feed pump rate, gpm x 100%

maximum feed pump rate, gpm

=	       25 gpm x 100

90 gpm            = 28%

15. The desired solution feed rate was calculated to be 40 gpd.  What is the feed rate

expressed as mL/min?

Convert GPD to liters per day = flow, gal/day x 3.785 liters/gal = 151.4 liters/day

Convert liters per day to liters per min = 151.4 liters x  1day    x  1 hr  = 0.105 L/min

day         24 hr      60 min

Convert liters/min to ml/min   0.105 liter       1000 ml

Min         x       L             =    105 ml/min