PHOSPHORUS REMOVAL

1: Why is Phosphorus removed from wastewater?

a. It is a food source for fecal coli-form

b. It is a food source for algae

c. Both A & B

2: The types of Phosphorus removal system include:

- Lime Precipitation

- Aluminum Sulfate Flocculation and Precipitation

- Breakpoint Chlorination

- Luxury Uptake

- All of the above.

3: The biological treatment process whereby the bacteria usually found in the activated sludge treatment portion of the secondary treatment are withdrawn to an environment without oxygen is the _______________ ___________ removal system.

4: Equipment necessary for the Lime Precipitation process includes:

- Lime Feed Equipment

- Mixing equipment and mixing chamber

- Clarifiers

- Pumps and Piping

- All of the above

5: Using the formula below will produce the result in % Phosphorus.

Total Phosphorus – Dissolved Phosphorus = Particulate Phosphorus

Particulate Phosphorus * Total Suspended Solids / 100 = % Phosphorus

- True

- False

6:  Aluminum sulfate in combination with wastewater can flocculate phosphorus in much the same way as lime precipitation?

a.   True

b.   False

7:  Three physical or chemical reactions which take place during lime precipitation include:

- Coagulation

- Flocculation

- Depth Filtration

- Sedimentation

8:  Can Short-Circuiting affect Phosphorus removal efficiency?

- True

- False

9:  Aluminum Sulfate feed rate can be determined by performing a jar test?

- True

- False

10:  What forms of phosphorus are included in the total phosphate measurement?

- Orthophosphate

- Polyphosphate

- Organic phosphorus

- Molyphosphate

- All of the above

Answers:

- B

- A, B, D

- Luxury Uptake

- E

- B

- A

- A, B, D

- A

- A

- A, B, C