m-TECH ™		                                                             92-731-S

MONOCLONAL TECHNOLOGIES, INC.

## "Immunochemicals for Research"

Catalog #  92-731-S

Lot # -----Q2309                                                                                                                                                      Store at  –20° Centigrade  - Desiccate if possible.

Prior to staining slide should be gently heat-fixed.

# PRODUCT DATA SHEET

Legionella heat-killed antigens are prepared from pure cultures grown for 24 hours at 37C in 2.5% CO2 on our modified BCYE media. Cell suspensions are adjusted to give 100-200 cells per 1,000X microscope field. They are in a phosphate buffer with a clear mordant to assist in the attachment of the cells to the slide.

Legionella Species other than pneumophila

DFA Control Slide (not for ifa testing)

| WELL # | SEROGROUP | DESCRIPTION | STRAIN | PRODUCT # | STANING INTENSITY |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | L. bozemanii | WIGA | 92-120-H |  |
| 2 | 2 | L. bozemanii | Toronto 3 | 92-121-H |  |
| 3 | 1 | L. dumoffii | NY-23 | 92-122-H |  |
| 4 | 1 | L. gormanii | LS-13 | 92-123-H |  |
| 5 | 1 | L. micdadei | TATLOCK | 92-124-H |  |
| 6 | 1 | L. longbeachae | Long Beach 4 | 92-125-H |  |
| 7 | 2 | L. longbeachae | Tucker 1 | 92-126-H |  |
| 8 | 1 | L. jordanis | Bl-540 | 92-127-H |  |
| 9 | 1 | L. oakridgensis | Oak Ridge 10 | 92-128-H |  |
| 10 | 1 | L. wadsworthii | 81-716 | 92-129-H |  |
| 11 | 1 | L. feeleii | WO-44C-C3 | 92-130-H |  |
| 12 | 2 | L. feeleii | 691-WI-H | 92-131-H |  |
| 13 | 1 | L. sainthelensi | Mt. St. Helens 4 | 92-132-H |  |
| 14 | 1 | L. anisa | WA-316-C3 | 92-133-H |  |
| 15 | 1 | L. santicrucis | SC-63-C7 | 92-134-H |  |
| 16 | 1 | L. hackeliae | Lansing 2 | 92-138-H |  |
| 17 | 2 | L. hackeliae | 798-PA-H | 92-139-H |  |
| 18 | 1 | L. maceachernii | PX-1-G2-E2 | 92-140-H |  |

m-TECH™ is a registered trademark of  Monoclonal Technologies, Inc./ 16335 New Bullpen Rd.. / Milton, GA  30004 / Toll Free (U.S.): 1-888-683-2414 / Phone: 770-521-1960  / Fax: 770-521-1930 / e-mail: m-tech@mindspring .com / http://www.4m-tech.com