SURVEY AND VALUATION REPORT
FOR ROBERTS 38FT 2003 SAILING VESSEL

“ ZESTER ”

CECIL M. LANGE, AMS
MARINE SURVEYOR AND CONSULTANT
MEMBER- SCIETY OF ACCREDITED MARINE SURVEYORS

VHF Channel 22
Tel: 011-52-612-122-7066
Cell: 044-612-131-7459
Morelos #915 esq. Josefa O. de Dominguez
La Paz, B.C.S., México  23000
Email: cecillange1925@yahoo.com

# SUMMARY

In accordance with the request for a marine survey of this vessel for the purpose of evaluating its present condition and estimating its market value and replacement cost, I hereby submit my conclusions based on the following report.  The subject vessel was personally inspected and found to be a well-constructed, appointed and comfortable vessel.

Subject to corrections of the Requirements and Recommendations listed in this report, and predicated on the assumption that engines and machinery are in working order, the vessel is considered to be “fit for its intended use” of recreational coastal and offshore cruising OR sport fishing and coastal cruising.

# SCOPE OF SURVEY

Constraints placed on Surveyor:	Unless otherwise noted, all items inspected by the surveyor (such as, but not limited to, seacocks, thru-hull fittings, and hoses) is limited to that which is readily and visibly accessible.

Ships Papers Onboard:	Yes

Engine(s) Run:	No

Sea Trial:	No

Special Test Meters Used during Survey:  Altra sound

NOTE: Use of asterisks (*) in the body of the report indicate that a finding is listed in the “Recommendations” or “Requirements” Section pertaining to that item.

THE MANDATORY STANDARDS PROJULGATED BY THE UNITED STATES COAST GUARD (USCG), UNDER THE AUTHORITY OF TITLE 46 UNITED STATES CODE (USC) TITLE 33, AND TITLE 46, CODE OF FEDERAL REGULATIONS (CFR), AND THE VOLUNTARY STANDARDS AND RECOMMENDED PRACTICES DEVELOPED BY THE AMERICAN BOAT AND YACHT COUNCIL (ABYC) AND THE NATIONAL FIRE PROTECTION ASSOCATION (NFPA) HAVE BEEN USED AS GUIDELINES IN THE CONDUCT OF THIS SURVEY.

The vessel was inspected “On the Hard” only. Engines were NOT run. The rigging was inspected by a qualified rigger who was contracted by the owner. An Ultra Sounding was done on the bottom plating.

# VESSEL STATISTICS

| NAME OF VESSEL ZESTER | DATE OF SURVEY August 14 2010 |
| --- | --- |
| HOME PORT //////////////// | SURVEY REQUESTED BY Owner |
| MANUFACTURER’S NO.: CFLOOOO1E700 | OWNERS NAME Miller Adams |
| OFFICIAL NUMBER: 1142540 | OWNERS ADDRESS1626 ///////////// |
| TYPE OF VESSEL Roberts Sailing vessel | INTENDED USE Recreational |
| YEAR BUILT: 2003 | INSPECTED AFLOAT: No |
| BUILDER: Lindberg Marine | INSPECTED HAULED: August 14 2010 |
| VESSEL MODEL: Roberts 38 | Accommodations: Five plus |
| DESIGNER: Roberts | DISPLACEMENT 30,000 lbs |
| LOA 38 FT.3” | TONNAGE MEASUREMENTS |
| BEAM: 11 FT 3” | GROSS TONS: 12 |
| DRAFT: 6 FT | NET TONS: 11 |
| DESIGNED WATER LINE: NA | DEPTH : 5 ft 8 “ |

# VALUATION

MARKET VALUE    $  73,000 00               REPLACEMENT COST $ 350,000 00

EQUIVELENT VALUES

Roberts 38 ft   cutter steel 1980 $ 75,000 00  Australia  Yacht World

Roberts 38 ft  cutter steel  1982  $  56,368 00  New Zealand  Yacht World

Roberts 34 ft Spray steel 1995  $  70,048 00 sale pending  Yacht World

SYSTEMS, HULL, DECK, SUPERSTRUCTURE

| HULL CONSTRUCTION AND CONDITION: | Steel 3/16” core 10 Longtitudinals 1 1/4 x 5/16 flat straps Transverse frames 2 1/2 ”x 5/16” steel, good condition , no rust |
| --- | --- |
| KEEL DESIGN: | Long fin keel 6,500 lbs lead ballast |
| KEEL CONSTRUCTION AND CONDITION: | Steel , good condition |
| DECK CONSTRUCTION AND CONDITION: | Steel, 3/16” core 10, no rust |
| SUPERSTRUCTURE CONSTRUCTION AND CONDITION: | Steel 3/16” core10, no rust |

## Electrical – DC 12 volt

| NO. OF BATTERIES: | Five Group 31, AGM 12 VOLT Four house and one starting battery, new |
| --- | --- |
| BATTERY VENTILATION: | Satisfactory |
| TERMINALS CONDITION: | Clean |
| TERMINALS PROTECTED: | Yes |
| BATTERIES SECURED: | Yes |
| BATTERY CHARGING: | Engine alternator,, with Link 1000 battery monitor. 160 Amp Balmar run off a jack shaft, and a Freedom 10 , 1000 watt Heart inverter / charger 50 amp. |
| BATTERY SWITCH: | Two |
| WIRING TYPE/GRADE: | Marine grade wire, tinned. |
| CIRCUIT PROTECTION: | Breakers and fuses, |

## Electrical – AC  110 volt 35 amp

| SHORE POWER CONNECTION: | 110 volt, 30 amp with 50 ft cord |
| --- | --- |
| WIRING TYPE/GRADE: | Marine grade copper |
| AC CIRCUIT PROTECTION: | Breakers |
| AC GALVANIC ISOLATOR: | Heart voltage mind transformer |
| INVERTER: | Freedom 10 -1000 watt inverter / charger. |

## Propulsion and Auxiliary Power

| PROPULSION MANUFACTURER: | Sabb |
| --- | --- |
| MODEL: | Sabb 2GZ |
| RATED HORSEPOWER: | 22 hp |
| YEAR MANUFACTURERED: | Purchased in 1985 |
| YEAR INSTALLED: | 2003 |
| SERIAL NUMBER: | AO233 |
| ENGINE HOURS: | 1,750 hrs |
| FUEL TYPE: | Diesel |
| ALARMS | Oil pres., water temp, bilge high water |
| FRESH water cooled by grid cooling system: | Yes |
| ENGINE MOUNTS CONDITION: | Good |
| DRIP PAN: | Built-in |
| FUEL RETURN: | Yes |
| EXHAUST CLEAR OF WOOD WORK: | Yes |
| EXHAUST MUFFLER MATERIAL: | Steel, exhaust hose, FRP, water-cooled with water lift muffler |
| TRANSMISSION REDUCTION GEAR: | SABB 2-1 reduction |
| PROPELLER SHAFT SIZE: | St Steel 1 1/4” |
| PROPELLER SHAFT LOG: | Bronze |
| PROPELLER SHAFT SEALS: | Sabb sealed with grease lubrication |
| PROPELLER: | Bronze 2 bladed feathering and variable pitch 22”- |
| PROPELLER POSITION: | Protected |
| BEARING CONDITION: | Good, Sabb sealed system |
| GENERATOR: | NO |

## Corrosion Control

| BONDING: | Yes |
| --- | --- |
| ZINC SYSTEM: | Yes |
| ZINC CONDITION: | 75 % good |
| IMPRESSED CURRENT SYSTEM: | No |
| ANODE CONDITION: | N/A |
| GROUND PLATE: | No |

## Thru-hull Fittings, Seacocks and Hoses

| THRU-HULL MATERIAL: | Steel welded to hull, |
| --- | --- |
| THRU-HULL CONDITION | Appear sound |
| SEACOCK MATERIAL: | Bronze ball valves |
| SEACOCK CONDITION: | Good condition, free to turn |
| WOOD EMERGENCY PLUGS: | Yes |
| HOSE TYPE: | Neoprene wire reinforced |
| HOSE CONDITION: | Appear sound |
| HOSES DOUBLE-CLAMPED: | Double clamped |

## Steering

| HELM TYPE: | Wheel |
| --- | --- |
| STEERING METHOD: | Rack and pinion |
| EMERGENCY STEERING: | Tiller, autopilot, and Sail O Mat wind vane self steering |
| NO. OF STATIONS: | One |
| RUDDER: | Welded steel foil rudder skeg mounted |
| RUDDERING BEARINGS: | Good condition |
| RUDDER PACKING: | Sealed type with grease fitting from inside of vessel |

## Tankage

| NUMBER OF FUEL TANKS: | One |
| --- | --- |
| FUEL TANK MATERIAL: | Painted black ion |
| FUEL TANK LOCATION: | Under cockpit sole |
| FUEL TOTAL CAPACITY: | 53 gallons |
| PROPERLY SECURED: | Yes |
| FILL PIPE MATERIAL: | Fuel hose |
| FILL PIPE SIZE: | 1-1/2” |
| FILL PIPE GROUNDED: | Yes |
| FILL PIPE TIGHT AT DECK FITTING: | Yes |
| VENT LINE: | Yes |
| OVERFLOW GOES OVERBOARD: | Yes |
| FUEL LINE SHUT-OFF: | Yes |
| WATER TANK MATERIAL: | St Steel |
| NUMBER OF WATER TANKS: | Four |
| WATER TOTAL CAPACITY: | 80 gallons |
| HOLDING TANK MATERIAL: | St Steel |
| NUMBER OF HOLDING TANKS: | One, 20 gallons |

## Electronics and Navigation

| RADIOS: | VHF: ICOM IC M127 SSB ICOM M700 Automatic tuner |
| --- | --- |
| RADAR: | Furuno 7” LCD 1712 -2.2 KW. |
| GPS CHART PLOTTER: | Garmin 182 C |
| WIND AND SPEED | Ray Marine ST 60 |
| DEPTH SOUNDER: | Ray Marine ST 60 |
| SPEED INDICATOR: | GPS and Ray Marine ST 60 |
| AUTOPILOT: | Simrad TP 30, tiller attached to wind vane |
| COMPASS: | Ritchie 6 “ |
| SHIP’S CLOCK: | Yes |
| BAROMETER: | Yes |
| NAVIGATION LIGHTS: | International |
| HORN: | Yes canister |
| BELL: | Not seen |

## Sanitation

| MARINE SANITATION DEVICE: | One manual Wilcox Crittenden, Skipper 1550 |
| --- | --- |
| MSD SYSTEM VALVING: | Holding tank, deck pump out, overboard |

# SAFETY EQUIPMENT AND MANDATORY NOTICES

| BILGE PUMPS: | Electric automatic Rule 2000. Center, a manual Whale Gulper 220 portable and a Bosworth guzzler, located in bilge center.. |
| --- | --- |
| PERSONAL FLOTATION DEVICES: | 4 adult type II, 2 Type III. |
| THROWABLE DEVICES (TYPE IV): | Life sling and one 24” ring float |
| VISUAL DISTRESS SIGNALS, TYPE: | Out of date |
| EPIRB: | No |
| LIFE RAFT: | Givens 6 man , (needs servicing) |
| FIRE EXTINGUISHERS: | Two B2 SIZE1 POWDER, 1 ADC powder B2 , size ii Located forward, main salon, and galley. |
| AUTOMATIC FIRE SUPRESSION SYSTEM: | Fire boy in engine room |
| LIFE LINES: | St Steel stanchions and hand rails |
| PULPITS: | St Steel |
| OIL DISCHARGE PLAQUE: | Yes |
| WASTE MANAGEMENT PLAQUE: | Yes |
| WASTE MANAGEMENT WRITTEN PLAN: | Not required |

# GROUND TACKLE

| ANCHORS: | CQR 45 LBS WITH 300 FT ht 5/16” chain CQR 35 LBS WITH 25 ft chain and 150 ft 7/8 “ Nylon rode, Danforth 22 lbs, with chain and rode |
| --- | --- |
| ANCHOR WINDLASS: | Muir Hercules manual windlass |

# SPARS AND RIGGING

| MAST AND BOOM: | Aluminum |
| --- | --- |
| BOOM VANG: | Hard, |
| STANDING RIGGING: | 1x19 St Steel |
| RUNNING RIGGING: | Dacron |
| ROLLER FURLING: | Hood Sea Furl 5. |
| WINCHES: ANDERSON | Two # 46 St, Two # 52 St, 6 # 40 St, 1 # 28 St - Rope clutches Harkin traveler and car |
| CHAINPLATES: | Steel welded to hull |
| POLE: | Two whisker poles |
| SAIL INVENTORY AND CONDITION: | Main, Yankee, Jib, storm jib, 130/%genoa. All reported in good condition |

# GALLEY AND INTERIOR

| STOVE TYPE: | Propane Force 10 three burner stove with oven |
| --- | --- |
| STOVE WOODWORK PROTECTED: | Yes |
| STOVE FUEL CONNECTION METHOD: FUEL CONECTION FUEL LOCATION | Propane Propane hose with regulator ,( no gage), with sniffer In designated locker , vented |
| REFRIGERATION: | Built in with Glacia Bay II compressor and cold plate |
| AIR CONDITIONING: | No |
| HOT WATER TANK: | 6 gallon, engine and diesel heater heated |
| OTHER EQUIPMENT: | Wabasco diesel heating system |

# ADDITIONAL EQUIPMENT

| WATER MAKER: | H RO system, 500 GPD |
| --- | --- |
| DINGHY: | Avon Red crest 9 ft |
| OUTBOARD MOTOR: | 8 hp outboard motor |
| ENTERTAINMENT SYSTEM: | Sony AM / FM Tape player with a Sony 10 disk changer TV Philips 10 ‘ AC-DC , plus speakers |
| OTHER: Fenders 5 Dock lines 6 Many spares | Two St Steel swivel boarding ladders Sail O Mat Wind vane Solar Cell 75 watt |

HULL BELOW THE WATERLINE

The hull below the waterline was inspected and sounded with an ultrasound, a minimum of wastage was detected, no rust , all through hull fitting appear sound, the rudder bearings are sealed and lubricated  from above, the shaft bearings are a sealed Sabb unit also lubricated from the inside, all in good condition.

HULL ABOVE THE WATERLINE

The hull above the waterline including cockpit, decks, and superstructure are in good condition including the paint work, no rust, several small arrears of stick on non skid need repairs.

MECHANICAL

The steering is rack and pinion and is in good condition, both the rudder and shaft seals are sealed units in good condition. The motor was not run due to injector pump problems, sea cocks were activated and free to turn, hoses are in good condition and are double clamped, the bilge pump was activated. Both the engine and head anti siphon valves need servicing, no oil ore water was found in the bilge.

FORDECK

The foredeck consists of a double bow anchor roller with 2 anchors mounted ready to deploy, a manual windlass, cleats, St Steel  stanchions and bow pulpit with double St Steel safety lines and an escape hatch 22x22 and two 24x14”.

COCKPIT

The cockpit has a seat aft with binnacle steering, engine controls, compass and instruments, a seat on both sides and a bridge deck, a wind vane mounted on the transom, stern pulpit, a dodger and 2 ladders mounted on each side.

RIGGING AND HARDWARE

All deck hardware is in good working condition, the rigging was inspected by a separate Yacht rigger, see riggers report.

ACCOMMODATIONS

Forward is a V berth with cabinets, draws and a hanging locker. Aft of the V berth on the port side is the bathroom that consists of a vanity with hand basin, cabinet with mirror, toilet and a shower. Aft of the bathroom is the main salon is a U shaped settee with a drop leaf table, opposite is a settee with storage under, behind the settees are cabinets and shelves above.

GALLEY

The galley is aft of the main salon , is U shaped with a stove, refrigeration, sink and cabinets, under the center counter top is the engine, opposite on the starboard side is electronics and a chart table with a quarter berth aft, center is the companion way leading to the cockpit.

# SURVEYOR’S COMMENTS

Zester is a steel built vessel, an Dakota Ultrasonic model MMX-6 ultra sound was used below the water line, almost zero wastage was noted, she is a well built vessel, no rust was observed she was well coated and painted from new with foam above the bilge area, She is well equipped with the best of equipment.

# RECOMMENDATIONS

REQUIREMENTS                                                         RECOMENDATIONS

1 UPDATE FLARES                                              1    SERVICE LIFE RAFT

2 Install gage on propane regulator       2 Service fire extinguishers

3 Service engine and head anti siphon valves

# SURVEYOR’S CERTIFICATION

I certify that I have made a personal inspection of the vessel that is the subject of this report, and to the best of my knowledge and belief:

Specifics were obtained from sources available at the time of inspection and are believed correct, but are not guaranteed to be accurate.

The report analyses, opinions, and conclusions are limited only by the reported assumptions and limiting conditions, and is my personal, unbiased professional analyses, opinions and conclusions.

Observations and findings are limited to that which is visibly accessible.

I have no present or prospective interest in the vessel that is the subject of this report, and I have no personal interest or bias with respect to the parties involved.

My compensation is not contingent of the reporting of a predetermined value or direction in value, or direction in value that favors the cause of the client, the amount of the value estimated, the attachment of a stipulate result, or the occurrence of a subsequent event.

Attending Surveyor                                                                                                                                 ------------------------------------------------------------------------------------------------------------------------------------        Cecil M Lange, AMS # 937, Member—Society of Accredited Marine Surveyors

Official number

RELATED  PHOTOGRAPHS

Hull number

Out of the water

Foredeck

Facing aft

V berth

Galley

Main salon

Nav station

Electric panel

Sabb Diesel